3GPP TSG-WG SA2 Meeting #163	S2-2406514

Jeju, Korea, May 27 – May 31, 2024	(revision of S2-2xxxxxx)

Source:	CATT

Title:	KI#1: Evaluation and conclusion on multiple USS

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_ph3/ Rel-19

Abstract: It is proposed to add the evaluation and conclusion on KI#1 for multiple USS.

1.1 Introduction

There are 2 solutions addressing the KI#1 for multiple USS. This paper is proposed to provide an evaluation and conclusion.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-59.

# * * * * First change (All new text)* * * *

7.X	Evaluation for KI#1: Solutions for Multiple USS

There are 2 solutions (solutions #10 and #11) addressing KI#1 for Multiple USS.

Solution #10 proposes that the USS sends a Pre-mission flight planning assistance request to the UAS NF/NEF with a list of candidate border-crossing point(s), indication that the UAV will cross a USS border, then the NEF may map a list of the candidate border-crossing point(s) in form of geographical coordinates to a list of border cell IDs/TAIs. The NEF collects the information from related 5GC NF (e.g. NWDAF, GMLC, AMF) and responds to serving USS. If the serving USS detects that the UAV has left or is about to leave its service area, the USS/UTM may trigger USS/UTM changeover to the target USS/UTM to serve the UAV. This solution provides a complete procedure, which can address the issue for the scenario of multiple USS serving different geographical areas corresponding to the UAV flight path.

Solution #11 proposes to enhance the UUAA procedure for USS to provide the UAS NF with a list of serving USS info with service area, or the UAS NF and UAV UE store the list of serving USS info internally.

# * * * * Second change (All new text) * * * *

8.X	Conclusion for KI#1: Multiple USS

The following principles are recommended in normative work for KI #1 on Multiple USS:

-	The NEF/UAS NF determines a UAV enters/leaves a border cell/TA (belongs to the service area of serving USS) and sends the notification to the USS.

-	When the USS receives event report about a UAV enters/leaves a border cell/TA (belongs to the service area of serving USS) by invoking the existing Deferred 5GC-MT-LR Procedure in clause 6.3 in TS 23.273[x] (the event type is set to area, the geographical area is set to the border area of USS), it may determine to trigger USS/UTM changeover to a new USS/UTM to serve the UAV.

-	The NEF/UAS NF could map the service area (e.g. geographical areas served by USS) to 3GPP identifiers (e.g. cell IDs, TAI). And the NEF/UAV NF could map a 3GPP identifier (e.g. cell ID or TAI) back to specific border-crossing point information.

# * * * * End of changes * * * *