3GPP TSG-SA WG2 Meeting #163	S2-2406529

Jeju, Korea, May 27 – May 31, 2024

Source: 	Thales(rapporteur)

Title: 	Introduction for conclusions on the architecture principles following the NWM discussions, for KI2

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes Introduction for conclusions on the architecture principles following the NWM discussions, for KI2.

# 1 Discussion

This paper summarizes architecture principles that can be deduced after the 2 rounds of NWM discussion (see https://nwm-trial.etsi.org/#/documents/8860).

Method used is to directly deduce conclusions from observations made, observations being made following choices of the majority (i.e.: the greatest number) of answers.

# 2 Proposal

It is proposed to agree below proposed changes to 23.700-29.

*** First Change (all new text) ***

# 8	Conclusions

## 8.X Introduction for conclusions on the architecture principles

This chapter summarizes architecture principles that can be deduced after the 2rounds of NWM discussion (see https://nwm-trial.etsi.org/#/documents/8860).

Method used is to directly deduce conclusions from observations made, observations being made following choices of the majority (i.e.: the greatest number) of answers.

### 8.X.y Introduction for conclusions on the architecture principles for KI#2

It is proposed to agree on the following principals:

KI2_P1: Multi-satellites scenario needs to be supported, single satellite scenario being a particular case.

KI2_P2: Roaming needs to be supported, where the network on the ground could be a visited network.

KI2_P3: Data bearer to support are MO/MT SMS and CIoT CP. CIoT UP support is nice to have option.

KI2_P4: No new mechanism is required. R17 / R18 mechanisms can be reused for UE location verification.

KI2_P5: SA3 feedback is required od security aspects for the different solutions.

KI2_P6: No specific principles to optimize UE power consumption needs to be considered in this R19 release.

KI2_P7: There is no strong requirement on time to attach and access services.

KI2_P8: Legacy UEs (Rel-17 & Rel-18 UEs) cannot be supported without modification.

KI2_P9: In addition to onboard RAN, objective shall be to minimize the CN functionalities on the satellite payload.

KI2_P10: No new NFs need to be introduced on CN side to support S&F.

KI2_P11: MME split is considered in several answers but does not correspond to new NF on CN side.

KI2_P12: HSS and P-GW need to be on the ground for home routing purpose.

KI2_P13:SA3 feedback is requested to know if, in addition to the ground HSS, HSS feature can be on-boarded.

KI2_P14: Full or part of MME need to be on board.

*** End Of Change ***