3GPP TSG-WG SA2 Meeting #163	S2-2406537

Jeju Island, South of Korean, 27th – 31th, May 2024	(revision of S2-240xxxx)

Source:	CATT

Title:	Initial conclusions for DualSteer

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract: Initial conclusions for DualSteer.

- Introduction

This paper provides an initial conclusion for KI#1.1, KI#1.2, KI#1.3 and KI#1.4 respectively.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54 0.3.0.

# * * * * First change * * * *

8.1	Conclusions for DualSteer

8.1.1	Conclusion for KI#1.1: Subscription aspects to support DualSteer

The following principles are proposed to be considered in normative work:

-	UDM/UDR enables to associate the two SUPIs applying to DualSteer service in the user subscription data for each SUPI.

-	DualSteer Authorization shall be included in the user subscription data for each SUPI to indicate whether the SUPI is authorized to use the DualSteer service.

-	The dedicated Access Type/RAT restriction for the DualSteer service shall be included in the AM subscription data of each SUPI.

NOTE:	The RAT selection may need coordination with SA1/CT1.

-	Which SUPIs can be identified as primary SUPI in the user subscription data is up to the operator’s polices.

-	The same UE PCF selection information shall be included in the user subscription data to enable the two SUPIs applying to DualSteer service to select the same PCF during registration.

8.1.2	Conclusion for KI#1.2: Registration and mobility management for DualSteer

The following principles are proposed to be considered in normative work:

-	The DualSteer Device shall indicate to the AMF its capabilities to support the DualSteer service during registration for each SUPI.

-	AMF needs to indicate to the DualSteer Device whether the network supports the DualSteer service during registration for each SUPI.

-	The DualSteer Device shall perform separate registration procedures for each of the two SUPIs.

-	The PCF in HPLMN will provide policies to instruct the DualSteer Device when and how to trigger the Second SUPI to perform the registration and when and how to establish connectivity to the second 3GPP access network.

NOTE:	The policy related to PLMN selection may depend on SA1/CT1 discussion.

8.1.3	Conclusion for KI#1.3: Session management aspects for DualSteer

The following principles are proposed to be considered in normative work:

-	The DualSteer Device shall establish two separate PDU sessions for each of the SUPIs.

-	Two separate PDU sessions for each of the SUPIs can be correlated in SMF/PGW-C.

-	The DualSteer device includes the DualSteer capabilities in the PDU Session establishment of each SUPI.

-	Two DualSteer PDU sessions shall be always anchored in the same SMF/PGW-C and UPF/PGW-U.

-	The AMF for each SUPI shall enable to select the same SMF for two DualSteer PDU sessions of the two SUPIs.

-	The SMF/PGW-C shall enable to select the same UPF/PGW-U for two DualSteer PDU sessions of the two SUPIs.

-	The SMF (or UPF) shall allocate the same IP address for the correlated two DualSteer PDU Sessions to minimize service interruption during traffic switching.

8.1.4	Conclusion for KI#1.4: Policy enhancements for DualSteer

The following principles are proposed to be considered in normative work:

-	The URSP rule shall be extended to support the DualSteer service, e.g., to indicate the Access Type/RAT/PLMN selection for the second 3GPP access network and control the secondary PDU Session establishment.

NOTE:	The policy related to PLMN selection may depend on SA1/CT1 discussion.

-	The Access Type preference in Route Selection Descriptor shall be extended to include a Dual Steering indication or Multiple Accesses.

-	The PCC rules shall be extended to include the DualSteer policy which will control how to switch the service between two connected 3GPP access networks for DualSteer Device.

-	The DualSteer rules can be based on the principles of ATSSS rules with necessary extensions and steering mode limitations in place.

-	The enhanced N4 rules can be used to enforce the DualSteer policies in the DL direction.

# * * * * End of changes * * * *