3GPP TSG-WG SA2 Meeting #163	S2-2406550

Jeju, KR, May 27th – 31st, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Discussion on Link Layer Proxy as a deployment option with existing procedures

Document for:	Discussion

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_Ph3_ARCH / Rel-19

Abstract: The LLP is perceived as RAN by the CN and perceived as CN by the RAN. It can be deployed based on operator policy without upgrading existing AMF/MME functionality.

# 1. Introduction

This discussion paper analyses how the Link Layer Proxy in solution#9 can be deployed without upgrading existing AMF/MME functionality and the operations an LLP would have to perform when deployed.

# 2. Discussion

## 2.1. Functionality of the LLP

The functionality of Link Layer Proxy in solution#9 can be described as follows:

-	The LLP acts as an GTP-U tunnel endpoint between RAN and UPF/P-GW.

-	NG/S1-AP enhancements for regenerative satellite access are used between the LLP and gNB/eNB only.

-	Non UE-associated messages between on-board RAN and the AMF are handled by the LLP as follows:

-	For non UE-associated Interface Management related NG/S1-AP messages between a gNB/eNB and AMF/MME, they are terminated/initiated by the LLP, including NG/S1AP Setup, NG/S1AP Configuration Update, etc.

-	For non UE-associated Configuration Transfer related NG/S1-AP message (who’s contents is relayed by AMF/MME transparently), they are relayed by LLP without involving the AMF/MME, including Uplink/Downlink RAN Configuration Transfer, etc.

-	For other non UE-associated NG/S1-AP messages between RAN and AMF/MME, they are relayed by the LLP and the LLP replaces RAN node ID, if there is any, with LLP IDs (with same format of RAN node IDs).

-	UE associated NG/S1-AP messages between on-board RAN and the AMF/MME are relayed by the LLP and the LLP replaces RAN node IDs with LLP IDs (with same format of RAN node IDs), RAN UE NGAP IDs with and LLP UE NGAP IDs, user plane tunnel information, if there is any, with LLP user plane tunnel information.

-	Paging messages from an AMF/MME are sent by the LLP to gNB/eNBs on satellites which cover the TAIs in the paging message.

-	Xn and N2 handover procedures are reused and the LLP does not replace Target ID in HANDOVER REQUIRED with the LLP.

## 2.2. Protocol Stack and Architecture with LLP

Using EPS system as the basis, Figure 2.2-1 and Figure 2.2-2 present the control plane stack for eNB-MME and UE-MME with the LLP; Figure 2.2-3 and Figure 2.2-4 present the user plane protocol stack for eNB-SGW and UE-PGW with the LLP. For 5GS, eNodeB, MME and S-GW are replaced with gNB, AMF and UPF, and reference interfaces are replaced with their corresponding interfaces in 5GS.

Figure 2.2-1: Control Plane for S1-MME Interface with LLP

Figure 2.2-2: Control Plane between UE and MME with Link Layer proxy

Figure 2.2-3 User Plane between eNB and S-GW with Link Layer proxy

Figure 2.2-4: UE – P-GW User Plane with Link Layer proxy

Control Plane

eNB

SAT#1

eNB

SAT#2

MME

S/P-GW

S1-MME

S1-U

S1-MME

S1-MME

User plane

S1-U

S1-U

S11

Link Layer Proxy

UE

X2

Figure 2.2-5: EPS architecture for regenerative-based satellite access with Link Layer Proxy (LLP)

## 2.3 Procedures

### 2.3.1 Non UE-associated S1AP messages handling

#### 2.3.1.1 S1AP Setup

This procedure uses S1AP Setup and Disconnect messages to show how non UE-associated Interface Management related S1AP messages are terminated/initiated by the LLP.

Figure 2.3.1.1-1: S1AP Setup and Disconnect with LLP

-	Steps 1-3: The eNB onboard the satellite initiates S1 SETUP REQUEST which terminates in the LLP. The LLP stores TAI list and RAN node ID mapping from the S1 SETUP REQUEST and responds to the eNB on the satellite with an S1 SETUP RESPONSE. Steps 1-3 are performed multiple times by different satellites.

-	Steps 4-5: The LLP initiates/forwards the S1 SETUP REQUEST towards the MME as the first S1AP procedure triggered by LLP, or the LLP may initiate eNB Configuration Update towards the MME if the S1 SETUP REQUEST was not performed. The LLP replaces RAN node ID to the LLP ID.

-	Steps 6-8: Some eNBs on satellites may move out of the serving area and S1 is disconnected. The LLP updates the TAI list and RAN node ID mapping information (i.e., remove the TAI list of the RAN node moving out of LLP serving area).

NOTE: 	How S1 is disconnected depends on RAN3. The overall flow is equally applicable to all ways to disconnect S1.

#### 2.3.1.2 Configuration Transfer Messages handling

As MME does not interpret the transferred RAN configuration information (e.g., SON Configuration Transfer), the node level configuration transfer messages are transferred by LLP transparently without MME involvement.

Figure 2.3.1.2-1: Configuration Transfer with LLP

- Steps 1-2: The procedure is initiated with an ENB CONFIGURATION TRANSFER message sent from the eNB to MME and MME transfers it to target eNB transparently. When an LLP is deployed, the LLP handles the Configuration Transfer Messages without MME involvement and forwards it to the target eNB.

#### 2.3.1.3 Other Non UE-associated Messages handling

For other Non-UE associated Messages between RAN and MME, they are relayed by the LLP and if they contain RAN node ID, the LLP replaces it with LLP ID. This procedure uses PWS RESTART INDICATION messages to show how these messages are relayed by the LLP.

Figure 2.3.1.3-1: PWS RESTART INDICATION relayed by LLP with RAN node ID replacement

- Step 1-3: The eNB sends a PWS RESTRAT INDICATION to the LLP. The LLP replaces the RAN node ID with its LLP ID and sends it onto the MME.

### 2.3.2 UE level S1AP messages relayed by LLP

The procedures in this clause present how the LLP can be involved in EMM/ESM procedures, and shows there is no standardization impacts on the existing procedures.

#### 2.3.2.1. Initial Attach with LLP

Figure 2.3.2.1-1: Initial Attach with LLP

As well as the LLP mapping eNB ID with LLP ID and mapping eNB UE S1AP ID with LLP S1AP ID in UE level S1 messages, the other LLP behaviours for Initial Attach procedure are listed as follows:

- Steps 1-3: The LLP receives the S1AP INITIAL UE MESSAGE carrying the Attach request from the onboard eNB in step 2 and sends it to the MME in step 3, after perform the mapping of eNB ID and eNB UE S1AP ID. The LLP does not need to know the Initial UE message is carrying an ATTACH message.

- Steps 4-6: After receiving the S1 INITIAL CONTEXT SETUP REQUEST with Attach accept from MME in step 4, the LLP allocates the Up Link LLP S1-U tunnel resources and replaces the S1-U tunnel information of S-GW with the Up Link LLP S1-U tunnel information, and sends it to eNB.

- Steps 7-10: After the S1 INITIAL CONTEXT SETUP RESPONSE from the eNB is received in step 7, the LLP allocates Down Link LLP S1-U tunnel resources and replaces the S1-U tunnel information of eNB with the Down Link LLP S1-U tunnel information, and sends it to MME.

The LPP maintains the GPT-U tunnel between eNB and S-GW during the lifetime of the S1-U tunnel for the UE EPS bearer. The Up Link and Down Link user plane data is transferred via the LLP to S-GW/eNB.

#### 2.3.2.2. X2 Handover with LLP

Figure 2.3.2.2-1: X2 Handover with LLP

As well as the LLP mapping the eNB ID with LLP ID and mapping eNB UE S1AP ID with LLP S1AP ID in UE level S1 message, the other LLP behaviours for X2 Handover procedure are listed as follows:

- Steps 1-2c: After the LLP receives the S1 PATH SWITCH REQUEST from the Target eNB in step 2a, the LLP replaces the tunnel information in EPS bearers with DL LLP tunnel information, and sends the message to MME.

- Step 2d: The X2 handover procedure can continue as the LLP allocates new tunnel information.

- Step 3-4: The MME sends the S1 PATH SWITCH REQUEST ACKNOWLEDGE in step 3a to the LLP, based on the S1 PATH SWITCH REQUEST ACKNOWLEDGE message the LLP modifies the DL LLP user plane path to the target eNB in step 3b. The LLP then sends the S1 PATH SWITCH REQUEST ACKNOWLEDGE to the target eNB in step 3d.

#### 2.3.2.3. S1 Handover with LLP

Figure 2.3.2.3-1: S1 Handover with LLP

As well as the LLP mapping the eNB ID with LLP ID and mapping eNB UE S1AP ID with LLP S1AP ID in UE level S1 message, the other LLP behaviours for S1 Handover procedure are listed as follows:

- Steps 1-2: After the LLP receives the S1 HANDOVER REQUIRED message from the Target eNB in step 2a, the LLP sends the message to MME. The S1 handover procedure can continue as the LLP allocates new tunnel information.

- Steps 3a-3b: MME sends S1 HANDOVER REQUEST to the LLP in step 3a, based on received EPS bearers to setup in the S1 HANDOVER REQUEST, the LLP determines the UL LLP tunnel information to replace S-GW tunnel information for the EPS bearers to setup and sends it to target eNB in step 3c. Other IEs such as Source eNB to Target eNB Transparent Container IE are transferred by LLP to the target eNB transparently.

- Steps 4a-4d: The Target eNB sends the Handover Request Ack to the LLP in step 4a, based on received EPS bearers to setup in the S1 HANDOVER REQUEST ACKNOWLEDGE, the LLP determines DL LLP tunnel information to replace RAN tunnel information and sends the message to the MME in step 4c.

- Steps 5a-5c: The MME sends the S1 HANDOVER REQUEST message to the LLP in the step 5a, based on EPS Bearers to release in the message, the LLP releases the corresponding tunnel resources and sends the Handover Command to the Source eNB.

- Steps 6-9: After the LLP receives the S1 HANDOVER NOTIFY message in step 8a, the LPP sends it to MME after performing eNB ID and eNB UE S1AP ID mapping in step 8b.

#### 2.3.2.4. S1 Handover with LLP and MME relocation

This procedure presents how LLP relocation happens when MME relocation is needed, MME relocation may happen with UE handover between satellites using different earth station location.

Figure 2.3.2.4-1: S1 Handover with LLP and MME relocation

As well as LLP mapping the eNB ID with LLP ID and mapping eNB UE S1AP ID with LLP S1AP ID in UE related S1 message, the other LLP behaviours for S1 Handover with MME relocation procedure are listed as follows:

- Steps 1-5: After the LLP receives the S1 HANDOVER REQUIRED message from Target eNB in step 2, the LPP determines that the Target eNB is not under this LLP’s management and does not replace the Target eNB ID with LLP ID. The LLP sends the message to Source MME in step 3. Based on the target TAI and target eNB ID (which is not the LLP ID), the source MME can deduce MME relocation is needed, step 4a and step 5 is same with current MME relocation procedure.

- Steps 6a-6c: Based on the target TAI, the Target MME send the S1 HANDOVER REQUEST to the Target LLP (which the MME believes to be the Target eNB) in step 6a. After the Target LLP receives the S1 HANDOVER REQUEST, based on EPS bearers to setup, the target LLP allocates UL LLP tunnels resources and replaces the S-GW tunnel information with the allocated the LLP tunnel information. The target LLP then sends the S1 HANDOVER REQUEST to the Target eNB.

- Steps 7a-7c: The Target eNB sends the S1 HANDOVER REQUEST ACKNOWLEDGE to the Target LLP in step 7a, based on the EPS Bearers to setup in the message, the LLP allocates DL LLP tunnel resources and replaces RAN tunnel information with the DL LLP tunnel information. The Target LLP then sends the S1 HANDOVER REQUEST ACKNOWLEDGE message to the Target MME.

- Steps 8-14: Same with current S1 handover with MME relocation procedure with the difference that the LLP transfers UE level S1 messages between RAN and MME and performs eNB ID and eNB UE SA1AP ID mapping.

#### 2.3.2.5. Paging with LLP

This procedure presents how LLP helps determining which eNB to send the paging message containing recommended cell.

Figure 2.3.2.5-1: Paging with LLP

As well as the LLP mapping the eNB ID with LLP ID and mapping the eNB UE S1AP ID with LLP S1 AP ID in UE level S1 message, the other LLP behaviours for Paging are listed as follows:

- Steps 1a-1b: The UE may not move after entering into ECM_IDLE. When the MME needs to page the UE, for example both eNB#1 and eNB#2 might be covering the UE located TAIs, but only eNB#2 is covering the recommended cell (i.e., mapped Cell).

- Step 2-4: The MME sends an S1 PAGING message to the LLP (which it believes to be the target eNB) which includes the recommended cell. After receiving the message, based on TAIs, the LLP determines that eNB#1 and eNB#2 are covering the recommended cell associated with the geographic area. The LLP sends the S1 PAGING message to eNB#1 and eNB#2.

- Step 5: As eNB#1 and eNB#2 are covering the recommended cell, the selected eNB#1 and eNB#2 pages UE after receiving the paging message.

#### 2.3.2.6. Service Request procedure with LLP

This procedure presents how the LLP is involved in UE triggered service request procedure.

Figure 2.3.2.6-1: UE triggered Service Request

The LLP maps the eNB ID to LLP ID, the eNB UE S1AP ID to LLP S1 AP ID and replaces the S1-AP tunnel information in UE related S1 message. The other behaviours for UE triggered Service Request procedure are listed as follows:

- Steps 1-2c: The UE sends NAS message Service Request towards the MME. The eNB forwards the message to LLP (which it believes to be the MME) via an S1-AP message, the LLP performs mapping of the eNB ID with LLP ID and mapping eNB UE S1AP ID and forwards the S1-AP message to MME.

- Step 3: Same as step 3 of Figure 5.3.4.1-1 in TS 23.401. The LLP transfer security related NAS messages transparently.

- Step 4-4b: The MME sends S1-AP Initial Context Setup Request to the LLP; the LLP allocates UL S1-U tunnel resources and replace the SGW address and S1-TEID(s)(UL) with LLP address and allocated UL LLP S1-U TEID(s).

- Step 5: Same as step 5 and 6 of Figure 5.3.4.1-1 in TS 23.401.

- Step 6-8: The eNodeB sends an S1-AP message Initial Context Setup Complete to the LLP, the LLP allocates DL S1-U tunnel resources and replace eNB address and S1-TEID(s)(DL) with LLP address and allocated DL LLP S1-U TEID(s).

- Step 9: Same with step 8-12 of Figure 5.3.4.1-1 in TS 23.401.

# 3. Conclusion and proposal(s)

The LLP is perceived as RAN by the CN and perceived as CN by the RAN.

A deployment may use a Link Layer Proxy as an implementation option to minimise the impacts of regenerative satellite access to 5GC/EPC. There would not be normative changes required for the deployment option and the deployment option shall be documented in an informative annex in 5GC/EPC.

Proposal#1: Document the functionality description of the LLP in an informative annex in 5GC/EPC for deployment guidance.

# Annex A.

# A.1 S1-AP procedures clarification per LLP handling behaviours

This annex classifies the S1-AP procedures into the following categories (highlighted with different colours) per LLP handling behaviours. The List of S1AP Elementary procedures of TS 36.413 is also highlighted with corresponding colours.

- Non-UE associated messages terminated/initiated by LLP.

- Non-UE associated messages relayed/transferred by LLP with RAN node ID replacement if there is any.

- UE associated messages relayed by LLP with RAN UE NGAP ID, RAN node ID and tunnel information replacement if there is any.

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| E-RAB Setup | E-RAB SETUP REQUEST | E-RAB SETUP RESPONSE |  |
| E-RAB Modify | E-RAB MODIFY REQUEST | E-RAB MODIFY RESPONSE |  |
| E-RAB Modification Indication | E-RAB MODIFICATION INDICATION | E-RAB MODIFICATION CONFIRM |  |
| E-RAB Release | E-RAB RELEASE COMMAND | E-RAB RELEASE RESPONSE |  |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| S1 Setup | S1 SETUP REQUEST | S1 SETUP RESPONSE | S1 SETUP FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| MME Configuration Update | MME CONFIGURATION UPDATE | MME CONFIGURAION UPDATE ACKNOWLEDGE | MME CONFIGURATION UPDATE FAILURE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| Kill | KILL REQUEST | KILL RESPONSE |  |
| UE Radio Capability Match | UE RADIO CAPABILITY MATCH REQUEST | UE RADIO CAPABILITY MATCH RESPONSE |  |
| UE Context Modification Indication | UE CONTEXT MODIFICATION INDICATION | UE CONTEXT MODIFICATION CONFIRM |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE |  |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Handover Notification | HANDOVER NOTIFY |
| E-RAB Release Indication | E-RAB RELEASE INDICATION |
| Paging | PAGING |
| Initial UE Message | INITIAL UE MESSAGE |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| NAS non delivery indication | NAS NON DELIVERY INDICATION |
| Error Indication | ERROR INDICATION |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| DownlinkS1 CDMA2000 Tunnelling | DOWNLINK S1 CDMA2000 TUNNELLING |
| Uplink S1 CDMA2000 Tunnelling | UPLINK S1 CDMA2000 TUNNELLING |
| UE Capability Info Indication | UE CAPABILITY INFO INDICATION |
| eNB Status Transfer | eNB STATUS TRANSFER |
| MME Status Transfer | MME STATUS TRANSFER |
| Deactivate Trace | DEACTIVATE TRACE |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| eNB Direct Information Transfer | eNB DIRECT INFORMATION TRANSFER |
| MME Direct Information Transfer | MME DIRECT INFORMATION TRANSFER |
| eNB Configuration Transfer | eNB CONFIGURATION TRANSFER |
| MME Configuration Transfer | MME CONFIGURATION TRANSFER |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Downlink UE Associated LPPa Transport | DOWNLINK UE ASSOCIATED LPPA TRANSPORT |
| Uplink UE Associated LPPa Transport | UPLINK UE ASSOCIATED LPPA TRANSPORT |
| Downlink Non UE Associated LPPa Transport | DOWNLINK NON UE ASSOCIATED LPPA TRANSPORT |
| Uplink Non UE Associated LPPa Transport | UPLINK NON UE ASSOCIATED LPPA TRANSPORT |
| PWS Restart Indication | PWS RESTART INDICATION |
| Reroute NAS Request | REROUTE NAS REQUEST |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| NAS Delivery Indication | NAS DELIVERY INDICATION |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| eNB CP Relocation Indication | eNB CP RELOCATION INDICATION |
| MME CP Relocation Indication | MME CP RELOCATION INDICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Handover Success | HANDOVER SUCCESS |
| eNB Early Status Transfer | eNB EARLY STATUS TRANSFER |
| MME Early Status Transfer | MME EARLY STATUS TRANSFER |

NOTE:	Error Indication can be a non-UE associated or UE associated message.