3GPP TSG-SA WG2 Meeting #163	S2-2406576

27 May - 31 May 2024, Jeju, South Korea	(revision of S2-240xxxx)

Source:	CATT

Title:	Conclusion for KI#2.1

Document for:	Discussion/Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes an evaluation and conclusion for KI#2.1.

1.	Introduction

This contribution proposes an evaluation and conclusion for KI#2.1.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54.

# * * * * First Change (all new text)* * * *

8.2	Conclusions for ATSSS_Ph4

8.2.1	Interim Conclusion for Key Issue #2.1

It is concluded that:

1) 	MPQUIC steering functionality to be extended with the following CONNECT protocols:

-	Only CONNECT-IP [7] to tunnel IP packets between a client (UE) and a proxy (UPF) using HTTP/3 over MPQUIC for the IP PDU Session type and Ethernet PDU Session Type;

Note: The CONNECT-TCP and CONNECT-Ethernet are not supported in this release.

2) 	During MA PDU Session Establishment with an IP-based PDU Session type, the UE indicates to SMF what CONNECT methods it supports (e.g. CONNET-IP or CONNECT-UDP). The SMF is configured, as part of the DNN configuration, with what proxy protocols are supported. The SMF determines what CONNECT methods are supported for the MA PDU Session, considering UE capabilities, network capabilities, and network local policies, and indicates that to the UE in PDU Session Establishment Accept. If multiple CONNECT methods are supported for a traffic flow, the UE selects the CONNECT method based on the application type (e.g. UDP or IP) and/or based on UE implementation.

3)	For IP-based PDU Session type, in a single MA PDU Session, different proxy protocols (e.g. CONNECT-IP or CONNECT-UDP) may be used for different service data flows.

4)	For Ethernet PDU Session type, only if the above payload is IP, the CONNECT-IP can be used for the above IP service data flows.

Editor's note:	Whether CONNECT-UDP can be used for the UDP packets over the Ethernet PDU Session is FFS.

# * * * * End of change* * * *