SA WG2 Meeting #163	S2-2406636

Jeju, Korea, May 27 – May 31, 2024

Source:	OPPO

Title:	KI#1, Conclusion Principles for AL-FEC Awareness at RAN

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2/Rel-19

Abstract of the contribution: This contribution proposes conclusion principles for AL-FEC awareness at RAN.

- Discussion

This paper proposes conclusion principles for AL-FEC awareness at RAN of KI#1, and can be updated based on SA4 and RAN2 responses to S2-2405604.

2.	Text proposal

It is proposed to agree the following changes to TR 23.700-70:

* * * 1st  change (all new text)* * *

8.X	Key Issue #4 Conclusion Principles

The following conclusion principles are agreed for AL-FEC awareness at RAN.

- If FEC applied,  the FEC related parameters (e.g. redundancy ratio) can be provided to the NG-RAN via CP or UP.

  - A static redundancy ratio for a given QoS Flow can be provided to the NG-RAN via CP and the same FEC content ratio is expected to apply for all PDU Sets within the QoS Flow.

  - If the AS includes a dynamic redundancy ratio in the PDU Set Information Header in the RTP header, the PSA UPF marks the redundancy ratio for the PDU Set in the GTP-U header based on N4 rules.

- NG-RAN can leverage static or dynamic redundancy ration when making packet discarding decisions during congestion.

Editor's note: the above can be updated based on SA4 and RAN 2 feedback.

.

* * * End of change * * *