3GPP TSG-WG SA2 Meeting #163	S2-2406667

Jeju Island, Korea, May 27th – 31st, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion of key issue 2

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract: this contribution proposes conclusion for key issue#2.

1. Introduction/Discussion

Solution #4, #5, and #6 in TR 23.700-59 address key issue #2 “Network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information”.

Following table summarized the information used for NWDAA proposed in three candidate solutions.

|  | Information obtained/collected/derived by network for DAA | How the information helps for DAA |
| --- | --- | --- |
| Solution #4 | Not mentioned | Unclear |
| Solution #5 | Relative distance between UAVs(real-time), by means of R18 LCS/ranging Relative distance between UAVs (predictions), by means of Relative Proximity Analytic from NWDAF | Relative distance, if smaller than a threshold, indicates potential confliction. |
| Solution #6 | Same as solution#5 | Same as solution #5 |

Way forward proposal:

Solution #4 proposed LDS (an application server) to interact with UAV via application layer. For example, LDS may send deconfliction information or warn the UAV of potential conflict, as shown in step 6 of the information flow.

In addition, it is unclear the information obtained/collected/derived by network for DAA.

It is proposed not to consider solution#4 as key issue#2 conclusion.

Solution #5 and solution #6 propose the same solution principles, and address all the technical issues listed in key issue#2 description. Moreover, solution#5 proposal is more comprehensive i.e. covers the solution#6 solution.

It is proposed to select solution#5 as key issue#2 conclusion, and baseline solution for normative work.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-59.

# * * * * First change (all texts new)* * * *

8.X	Conclusion for key issue#2

Solution#5 proposed to leverage network-based positioning/ranging to determine the distance between UAVs, which is considered as key issue#2 conclusion, and baseline solution for normative work.

# * * * * End of changes * * * *