3GPP SA WG2 Meeting #163	S2-2406720

Jeju, South Korea, 27 – 31 May, 2024

Source:	Nokia

Title:	KI#2 Conclusions update

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto/ Rel-19

Abstract of the contribution: This paper proposes an update interim conclusions of KI#2.

# Discussion

During NWM discussions, two options are mainly considered for the KI#2 conclusions, namely,

- Option A: UDM/UDR for non-roaming, V-UDR for roaming; and

- Option D: (H)-UDM/UDR for both non-roaming and roaming.

These two options rely on the agreed interim conclusions which is that the NEF API, i.e., parameter provisioning is used to support the 5G Femto CAG provisioning. In option D, NEF API is used to provision the information in HPLMN for both roaming and non-roaming scenarios. It is clear that for a non-roaming scenario, the proposal would work as similar to the existing mechanisms. However, for a roaming scenario, Option D relies on a pre-configured information at AF to know which PLMN that the UE’s subscription belongs to. So that, when there is a CAG provisioning update for this UE, AF will know HPLMN of UE and provides updates to NEF of the HPLMN of the UE. This assumption/constraint is not easy to achieve as firstly, the AF should know the HPLMN of the UE(s) as part of the CAG provisioning information that is provided by the CAG owner/authorized administrator; and secondly AF needs to have credentials/pre-configured information for every PLMN that the CAG owner/authorized administrator PLMN has roaming agreements. On the other hand, Option A uses the same API without all these constraints as when the CAG owner/authorized administrator provides CAG provisioning information, the updates are provided to UE(s) as VPLMN provided CAG information based on the existing mechanisms used for PNI-NPN.

Therefore, this paper updates the interim conclusions of KI#2 to remove the EN and clarifies that for a roaming scenario, the CAG owner/authorized administrator provides updates to its own PLMN.

# Proposal

It is proposed to remove Editor’s note and update KI#2 conclusions in the FS_5G_Femto TR 23.700-45.

*** First change ***

## 8.1	Key Issue #2: Enabling provisioning of subscribers allowed to access CAG cell and managing access control by the CAG owner or an authorized administrator

For conclusions, the following aspects are taken as way forward:

-	The NEF API, i.e. Parameter Provisioning specified in the 23.502 [3] clause 4.15.6 is enhanced to support the 5G Femto CAG provisioning for both non-roaming (to H-NEF) and roaming (to V-NEF) scenarios.

-	CAG owners/authorized administrators provide input to the PLMN that their subscription belongs to.

*** END of changes ***