3GPP TSG-SA WG2#163	S2-2406824

Jeju, Korea, May 27 – May 31, 2024             	(was S2-240xxxx)

Source:	Apple

Title:	Interim Conclusion for KI#1.2

Document for:	Discussion/Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes interim conclusion for KI#1.2.

# 1.	Introduction

This contribution proposes interim conclusion for KI#1.2 based on NMW discussion report in S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v0.3.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

## 2.1 	Interim Conclusions for KI#1.2

-	An operator policy should be specified that enables the HPLMN to control when the DualSteer device registers to the second 3GPP access network.

NOTE:	SA1/CT1 should decide how the UE selects the RAT and the PLMN.

-	The DualSteer Device indicates to the AMF its capabilities to support for DualSteer during registration.

-	The UE does not need to indicate to AMF whether it acts (or can act) as the primary UE or secondary UE.

-	The AMF indicates to the DualSteer Device whether the network supports DualSteer feature during registration.

-	The DualSteer Device performs separate registration procedures for each of the subscriptions/SUPIs.

-	The DualSteer Device does not provide any assistance information to the network after registering its first SUPI to help registering the second SUPI.

-	The DualSteer feature has impact on AMF for Registration.

# >>>>END OF CHANGES<<<<