3GPP TSG-SA WG2#163	S2-2406827

Jeju, Korea, May 27 – May 31, 2024             	(was S2-240xxxx)

Source:	Apple

Title:	Interim Conclusion for KI#1.4

Document for:	Discussion/Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes interim conclusion for KI#1.4.

# 1.	Introduction

This contribution proposes interim conclusion for KI#1.4 based on NMW discussion report in S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v0.3.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

## 2.1 	Interim Conclusions for KI#1.4

-	The DualSteer feature requires extension to URSP rules.

-	Each SUPI should have its own URSP rules but the rules controlling the DualSteer feature should be the same for both SUPIs.

-	The Access Type preference in Route Selection Descriptor is extended to include a Dual Steering indication or Multiple Accesses.

-	The DualSteer Device can always attempt the registration of the second SUPI.

-	Policy information when to establish a secondary PDU Session is controlled via URSP rules.

-	DualSteer policy should control how to switch the service between two connected 3GPP access networks.

-	PCC rules need to be extended to include DualSteer policies. If a single SM policy association is used, the PCC rules would apply to both PDU Sessions.

-	The DualSteer rules can be based on the principles of ATSSS rules with necessary extensions and steering mode limitations in place.

-	Enhanced N4 rules can be used enforce DualSteer policies in the DL direction.

# >>>>END OF CHANGES<<<<