SA WG2 Meeting #S2-163	S2-2406864

27 - 31 May, 2024, Jeju, South Korea

3GPP TSG-RAN WG3 Meeting #124	R3-243941

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Title:	Reply LS to SA5 on improved KPIs involving end-to-end data volume transfer time analytics

Response to:	S5-241086 – R3-240058

Release:	Release 18

Work Item:	NR_AIML_NGRAN-Core

Source:	RAN3

To:	SA5

Cc:	SA2, CT3, CT4, RAN2

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA5 for their LS.

RAN3 evaluated carefully the two specifications and made the following observations with respect to TS 28.552 and TS 28.558:

- TS 28.558 does not define M4 measurement.

- There seems to be a misalignment between the two specifications with respect to which entity performs the measurements. In particular, RAN3 identified the following cases:

- The “Average delay DL air-interface” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “Average delay DL in gNB-DU” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “UL PDCP SDU Loss Rate” is measured by the GNBCUUPFunction and NRCellCU in TS 28.552 and by GNBCUUPFunction in TS 28.558.

From RAN3 perspective, the above misalignment should be corrected in TS 28.558 based on TS 28.552.

Also, RAN3 made the following observations concerning the S-TMSI identifier:

- “S-TMSI” should be modified to “5G-S-TMSI”.

- S-TMSI is not available in the gNB-DU and in the gNB-CU-UP.

- S-TMSI may not be always available in the gNB-CU-CP.

2. Actions:

To SA5 : 	RAN3 would like to ask SA5 to add the missing M4 measurement in TS 28.558 and also correct possible misalignments between TS 28.558 and TS 28.552 with respect to the entity that performs the UE measurements. RAN3 would also like to ask SA5 to provide feedback on the observations concerning the S-TMSI as reported above.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting RAN3#125					19 - 23 August 2024							Maastricht, NL

TSG-RAN WG3 Meeting RAN3#125bis				14 - 18 October 2024							China