3GPP TSG-SA WG2 Meeting #163	S2-2406888

Jeju Island, South Korea, 27th – 31st May 2024	(revision of S2-2406061)

Title:	Reply LS on ECS Configuration Information

Response to:	LS S2-2405857 (C1-242674) on ECS Configuration Information from CT1

Release:	Rel-18

Work Item:	EDGE_Ph2/EDGEAPP_Ph2

Source:	TSG SA2

To:	TSG CT1, TSG SA6

Cc:	TSG SA3, TSG CT3, TSG CT4

Contact person:	Hyesung Kim

hs1207.kim@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1	Overall description

SA2 would like to provide response to the questions from CT1 as follows:

SA2 Answer to CT1 on Q1 (Security parameters in the ECS Configuration Information):

From SA2 perspective, no misalignment is identified and no update for TS 23.548 is required.

SA2’s understanding is that the security parameters is already included in the ECS configuration information as described in Table 8.3.2.1-1 of TS 23.558. The ECS Address Configuration Information (EACI) in Table 4.15.6.3d-1 (TS 23.502) contains the ECS configuration information defined in TS 23.558 as described in clause 6.5.2.1 of TS 23.548 as follows: “The ECS Address Configuration Information consists of one or more ECS Configuration Information as defined in clause 8.3.2.1 of TS 23.558”. The EACI (in Table 4.15.6.3d-1, TS 23.502) containing the ECS Configuration Information in TS 23.558 is delivered to the UE as per SA2 procedure. It means that the delivery of the security parameter in the ECS configuration information in TS 23.558 is supported without any update on the related SA2 specification. Accordingly, SA2 confirms that neither technical misalignment nor the need of update on the TS 23.548 is identified

SA2 Answer to CT1 on Q2 (PLMN ID in TS 23.502 and the list of supported PLMN ID(s) in TS 23.558):

From SA2 perspective, no update for SA2 TS is required because no misalignment is identified. SA2 clarifies that the whole information elements in the ECS Configuration Information (including the list of supported PLMNs) are delivered to the UE.

SA2 would like to further clarify that the PLMN ID in Table 4.15.6.3d-1 of TS 23.502 and the list of supported PLMN(s) in Table 8.3.2.1-1 of TS 23.558 are different and used for different purposes.

-	The list of supported PLMN ID(s) in Table 8.3.2.1-1 of TS 23.558 indicates “The identifier of a PLMN for which EDN configuration information can be provided by the ECS.”

-	The PLMN ID in Table 4.15.6.3d-1 of TS 23.502 is used by 5GC to determine whether the ECS configuration information should be provided to the UE when the UE roams to the identified PLMN.

Additionally, SA2 identifies that there can be potentially duplicated information elements in the ECS Configuration Information and would like to share this potential issue with SA6. These potentially duplicated information elements are:

- 	a list of PLMN IDs noted as part of one of the alternatives of Spatial Validity Condition in clause 6.5.2.1 of TS 23.548; and

-	the list of supported PLMN(s) in Table 8.3.2.1-1 of TS 23.558.

SA2 would like to understand whether there can be a case where Table 8.3.2.1-1 of TS 23.558 contains two lists of PLMN ID(s), one is as part of Spatial Validity Condition and the other one is the dedicated entry in the table. SA2 would like to get feedback from SA6 on whether it is duplicated or not. If duplicated, SA2 kindly asks SA6’s feedback on how the potential issue should be resolved from SA6 perspective.

2	Actions

To 3GPP TSG CT1, TSG SA6

ACTION: 	SA2 kindly requests CT1 to take into account the above information that there is no identified technical misalignment related with the questions from CT1 and asks CT1 to drive the stage 3 specifications, accordingly.

SA2 kindly requests SA6 to take into account the above information and provide feedback.

3	Dates of next TSG SA WG 2 meetings

TSG-SA2 Meeting #164		19th – 23rd August 2024	Maastricht, Netherlands

TSG-SA2 Meeting #165		14th – 18th October 2024	Hyderabad, India