3GPP TSG-SA WG2#163	S2-2406941

Jeju, Korea, 27-31 May 2024	(was S2-2406894)

Source:	Qualcomm Incorporated, Samsung, Nokia, Deutsche Telekom, vivo, Ericsson

Title:	Conclusions of KI#5

Document for:	Discussion/Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2/Rel-19

Abstract of the contribution: This paper proposed conclusions on KI#5: Handling of PS data off exemption for services over IMS DC.

# 1.	Discussion

Solution #28 and 29 are documented for handling PS data off exemption for services over IMS DC. Following principles are derived from the above two solutions and proposed as conclusions for KI#5.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-77:

# >>>>BEGINNING OF CHANGES<<<<

## 8.X	Conclusions of KI#5

For KI#5 on " Handling of PS data off exemption for services over IMS DC " the following conclusions are agreed:

1.	"Services over IMS Data Channel" is configured as a whole service in the list of 3GPP PS Data Off Exempted Services.

2.	When the user activates/de-activates PS Data Off in the UE, existing procedures shall be used.

3.	The IMS AS may further notify the DCSF about the PS Data Off status.

4.	If IMS DC is not configured as an exempted service and PS Data Off is activated in the UE, all bootstrap and application data channels for this UE must be terminated.

The UE shall enforce as follows:

- If there is standalone IMS DC session, then UE shall terminate the standalone IMS DC session.

- If there is IMS DC session including audio/video media while audio and/or video media is exempted, UE shall keep only exempted audio and/or video media and release IMS DC (both bootstrap DC and application DC).

The network shall enforce as follows:

- If there is IMS DC session with audio/video media while audio and/or video media is exempted, IMS AS shall ensure that only exempted audio and/or video media is permitted.

# >>>>END OF CHANGES<<<<