SA WG2 Meeting #S2-163	S2-2406949

27 - 31 May, 2024, Jeju, South Korea	(Revision of S2-2406910 was 5944)

Source:	Samsung, Vivo, OPPO, Tencent, Tencent Cloud, InterDigital Inc, NTT DOCOMO, ZTE, ETRI, SK Telecom, Verizon, Nokia, Lenovo, Huawei, HiSilicon, ICS, MediaTek Inc, CMCC, Ericsson

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN /Rel-19

Abstract of the contribution: This contribution proposes conclusions for KI#3.

# Discussion

Based on the discussion on NWM and documented solutions in TR 23.700-84, this contribution proposes the conclusions for KI#3.

# Proposal

It is proposed to adopt the following changes into TR 23.700-84.

*** Start of the change ***

## 8.x	Conclusion for KI#3: NWDAF-assisted policy control and QoS enhancement

Following conclusions are proposed for KI#3:

Based on PCF request, NWDAF may provide assistance information analytics to PCF to assist the QoS determination and/or modification of QoS parameters. The PCF may take the assistance information into account to determine QoS parameters for a QoS flow, based on the operator's policy.

After the PCF determines the QoS parameters, if updated assistance information is received from NWDAF (the updated assistance information may include candidate QoS parameters, the analytics of the duration and usage of established QoS Flows), the PCF may determine to modify the QoS parameters for a QoS flow and provide the modified QoS towards the SMF, based on the operator's policy.

In particular,

- The PCF may send a analytics request or subcription to NWDAF for QoS and policy assistance information analytics. The PCF may include an analytics ID, Filter Information, one or multiple sets of QoS parameters, the candidate value list(s) for each QoS parameter, requesting expected service experience (e.g. QoE), etc.

NOTE 1:	details of the input parameters in the PCF request will be determined in normative stage.

NOTE 2:	whether to introduce a new analytics ID or reuse existing one will be determined in normative stage.

  - Based on the PCF request, the QoS and policy assistance information analytics provided by NWDAF may include: expected QoE can be achieved bythe candidate QoS parameters (the values of the QoS parameters are within the candiate value list provided by the PCF).

NOTE 3: whether the time window(s) associated with QoS parameters can be provided by PCF and can be then used to determine the candidate QoS will be discussed in the normative work.

NOTE 4:	 	whether to introduce new or reuse existing service/ service operation for providing candidate QoS will be determined in normative phase.

- The PCF may request one or more analytics from the NWDAF that are used within the same analytics target period, i.e. Observed service experience, QoS sustainability, Network Performance analytics, analytics of the duration and usage of the established QoS Flows.

NOTE 5:	Possible enhancements to the existing analytics and/or introducing new analytics will be determined in the normative phase.

NOTE 6	details of the input data and corresponding data sources for analytics will be determined in normative stage.

NOTE 7:	To provide the assistance for policy control and QoS, NWDAF shall not be aware of PCF internal logic.

NOTE 8: NWDAF can monitor the accuracy performance of the assistance information analytics based on a request from PCF. Whether to reuse the existing procedures for Analytics Accuracy Monitoring (defined in clause 6.2D of TS 23.288 [5]) or not can be decided in the normative phase.

*** End of the change ***