3GPP TSG-WG SA2 Meeting #163	S2-2406955

Jeju, Korea, May 27 - 31, 2024	(revision of S2-2405909, S2-2406884)

Source:	China Mobile, China Unicom, Huawei, HiSilicon, vivo

Title:	TR 23.700-70 KI#2: Evaluation & Conclusion for Supporting PDU Set Info. Identification for E2E Encrypted XRM Traffic

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract: This pCR proposes to conclude the Ki#2.

1	Discussion

This paper provides evaluation and conclusion of KI#2: Support PDU Set information identification for end-to-end encrypted XRM traffic.

2	Proposal

It is proposed to include the below changes into TR 23.700-70 .

First Changes

- 8. Conclusions

8.X	Conclusions for Key Issue #2

The following principles are recommended to be included in the conclusions:

Media over QUIC (MoQ) and Proxy-UDP-in-HTTP/3+QUIC-Aware Proxying should be supported in normative work.

-	For MoQ, the PDU set information is accommodated via Metadata in MoQ. UPF supports the MoQ relay functionality and identifies the PDU Set information from the MoQ metadata.

-	For Proxy-UDP-in-HTTP/3 + QUIC-Aware proxying, PDU set information is included in HTTP datagram via a new datagram mode or via packet transforms in QUIC-Aware proxying. UPF supports the HTTP/3 Client functionality.

NOTE: It is assumed neither of the solution categories has 3GPP standardization impact on UE.

Editor’s Note: The details of the above i.e., MoQ and Proxying-UDP-in-HTTP/3+QUIC-Aware Proxying, will be updated in the following meeting.

End of Changes