3GPP TSG-WG SA2 Meeting #163	S2-2406962

Jeju, Korea, May 27 – 31, 2024	(revision of S2-2406933)

Source:	Huawei, HiSilicon, CATT, Tencent, Meta USA, Lenovo, China Mobile

Title:	KI#9: Initial conclusion of KI#9 on enhancement for XR related network information exposure

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: conclusion of KI#9 on enhancement for XR related network information exposure is proposed in this paper.

1. Introduction

In draft TR 23.700-70, there are 3 candidate solutions agreed on enhancement for XR related network information exposure as following:

-	In Sol#19, the Alternative QoS Profiles are extended to include the PDU Set QoS parameters. In case of QNC enabled, the QNC together with the reference to the matching Alternative QoS Profile can be exposed by the AF via the control plane or the user plane.

-	In Sol#33, the available data rate for (non-)GBR QoS Flow can be exposed by the NG-RAN to the AF via the control plane or the user plane.

-	In Sol#34, the PDU Set delay and PDU Set Loss Rate are proposed to be measured and reported to the AF.

Based on the summary above, this paper intends to propose the conclusion for KI#9 in this paper.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change * * * *

8.x	Conclusions for Key Issue #9

The following aspects are concluded for the normative work:

-	For GBR QoS Flow, exposure of available data rate via user plane:

-	Based on the AF requests, the PCF instructs SMF to enable the available data rate report and exposure for a GBR QoS Flow.

-	Following the instruction of SMF,

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes, based on mechanisms defined in clause 5.8.2.17 of TS 23.501, the available data rate to the AF directly or via the local NEF.

NOTE: It is recommended that the available data rate exposure is not reported very frequently.

NOTE: This may be refined based on the potential feedback from RAN3.

# * * * * End of changes * * * *