3GPP TSG-WG SA2 Meeting #163	S2-2406968

Jeju, Korea, May 27th – 31th, 2024	(revision of S2-2406937)

Source:	Huawei, HiSilicon, Samsung, Nokia, Qualcomm

Title:	KI#2, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_ph2 / Rel-19

Abstract: This paper proposes the conclusion for KI#2.

# Introduction

This paper proposes the conclusion for solutions of KI #2.

# Discussion

This paper proposes the conclusion of solutions for KI#2.

# 3. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-77.

# * * * * First change * * * *

## 8.X Conclusions of Key Issue #2

The following conclusions are agreed for KI#2:

-	IMS network exposes its capabilities through the NEF, which can be provided by a specific NEF instance serving the IMS network exposure only or by the NEF instance serving the 5GC network exposure simultaneously.

-	The following capabilities need to be exposed by the IMS network:

-	establishing, updating, terminating of bootstrap data channels;

-	establishing, updating, terminating of (P2A, P2A2P, P2P) application data channels.

-	The Nnef and Nimsas services and/or service operations need to be enhanced to support the capabilities mentioned above.

NOTE: The detailed impacts to the services and/or service operations are defined in normative phase.

-	In Rel-19, the only service provider is IMS AS. The service consumer needs to discover the IMS AS for a given user via HSS before sending a request to IMS AS. If the user is not registered yet, the request is rejected.

# * * * * End of changes * * * *