3GPP TSG-WG SA2 Meeting #163	S2-2407080

Jeju, Korea, May 26th – 31th, 2024	(revision of S2-2406725)

Source:	ZTE, Nokia, Samsung, ETRI

Title:	KI#2 conclusion proposal

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: This paper proposes conclusion on KI#1in TR 23.700-63.

1. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-63.

# * * * * First change * * * *

8.2	Conclusions for Key Issue #2: Enhancements on UPF information exposure

The following aspects are concluded as principles for normative work:

-	UPF event exposure service is enhanced to allow NF consumer to obtain one NATed UE public IP address and Port for a particular PDU Session from the SMF and UPF, based on the private UE IP address allocated by 5GC.

-	The remote end IP address is mandatory input for the above to avoid exposing the full NAT mapping for a UE;

NOTE 1:	Whether it is needed to define a new SMF service will be determined in normative phase.

-	NEF/NWDAF can use UE IP address to discover the PSA UPF and subscribe directly to PSA UPF for data collection.

NOTE 2:	Restriction of subscribing particular Event ID(s) (like QoS Monitoring Event) indirectly via SMF remains as defined in Rel-18.

NOTE 3: 	NEF/NWDAF cannot use UE IP address to discover the local PSA UPFs.

-	For periodic reporting type of the UPF event exposure service with direct notifications when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change.

-	When the PDU session is released, if the UPF event subscription is via the SMF, the SMF may send a notification of subscription termination.

NOTE 4:	Whether to send the subscription termination indication to the consumer or the SMF implicitly subscribes to PDU session release event on behalf of the consumer will be determined in normative phase.

# * * * * End of changes * * * *