SA WG2 Meeting #S2-163	S2-2407093

27 - 31 May, 2024, Jeju, South Korea	(revision of S2-2406221)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0138 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the GPSI and Application Layer ID Mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 sends the LS (S2-2405863/C3-242616) to ask SA2, which NEF API is used to implement the GPSI and Application Layer ID mapping. This CR proposes that Nnef_ueid service defined in the 23.502 is used to for the GPSI and Application Layer ID mapping. In the 23.273, the GPSI and Application Layer ID mapping description (in clause 6.20.1 and 6.20.3) refer to 23.586 clause 4.3.9. In the 23.586 clause 4.3.9 and 4.3.7, it does not describe how This CR proposes to add related description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying the service operation used for the Application Layer ID to GPSI Mapping. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the 23.273 and 23.586, there is no description on which service operation is used for |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.7, 4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.3.7	NEF

In addition to the functions defined in TS 23.287 [6] and TS 23.304 [7], the NEF supports the following:

-	To enable AFs to provide service specific information to the 3GPP network, the NEF supports additional service parameters for Ranging/SL Positioning policy which can use the description of the clause 6.2.5 in TS 23.304 [7].

-	Interacting with GMLC for AFs and UEs in Ranging/SL Positioning procedures to perform 5GC-MT-LR (e.g. Forward location request from the AF including GPSI or Application Layer ID).

-	Update the UE subscription data stored in the UDR with Onboarding Enrolment Information.

-	Provide the GMLC with the GPSI of the UE based on Application Layer ID and ID mapping information stored in UDR application data.

-	Provide the GMLC with the Application Layer ID based on GPSI of the UE and ID mapping information stored in UDR application data.

-	Request UDR to store as application data the GPSI and Application Layer ID mapping information provisioned by AF.

Next CHANGE

### 4.3.9	GMLC

In addition to the functions defined in TS 23.273 [8], the GMLC supports the following:

-	enable trusted AFs and NFs to perform MT-LR by accessing GMLC directly with additional service parameters for Ranging/SL Positioning.

-	enable AFs and NFs to perform MT-LR by accessing GMLC through NEF with additional service parameters for Ranging/SL Positioning.

-	determine the serving AMF instances for the UEs involved in the Ranging/SL Positioning request and forward the request to respective serving AMF.

-	receive response from AMF and event reports from and return Ranging/SL Positioning result to NFs and AFs.

-	perform Application Layer ID to GPSI or GPSI to Application Layer ID resolution by querying NEF.

-	request Home GMLC of the UE(s) that belong to different PLMN(s) to retrieve the mapping information between GPSI and Application Layer ID (if needed) and to check those UE's Ranging/SL Positioning privacy profile.

NOTE:	When receiving location request by using SUPI of a UE from LCS client after translating SUPI to GPSI by using NUDM_sdm get service, GMLC performs Application Layer ID to GPSI or GPSI to Application Layer ID resolution by querying NEF with Nnef_UEid service operation defined in the 23.502 [3] clause 5.2.6.27.

END OF CHANGES