SA WG2 Meeting #163	S2-24071113

Jeju, Korea, May 27-31 2024

# Title:	KI#1: conclusions

# Source:	Nokia

# Document for:	Approval

Agenda Item:	FS_VMR_Ph2/19.6

Abstract:	this paper provides conclusions on KI#1 on MWAB architecture

1. Introduction

Based on the outcomes of the moderated email discussion we propose to provide an update to the conclusions for KI#1

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-06.

# Proposed text

## 8.1	KI#1 Conclusion

It is proposed to proceed normatively based on these principles.

1) 	The MWAB connection to OAM can use the default (S-NSSAI, DNN) for the MWAB-UE. It is established by the MWAB-UE upon request from the MWAB-gNB when the MWAB-gNB intends to provide service. The OAM Server Address is configured in the MWAB-gNB.

2) 	The OAM configures the MWAB-gNB with the S-NSSAI and DNN for the N2, N3 connections (they maybe be the same or different PDU sessions). The IP connectivity the MWAB-UE obtains via this(these) PDU session(s) is used by the MWAB-gNB to establish the N2, N3 connections. The MWAB-gNB OAM PDU session may be the same or different from the one used for N2 and N3 connections.

3)	The OAM configures the addressing information of the AMFs that the MWAB-gNB needs to connect to.

8) The MWAB-gNB indicates to the AMFs it is a MWAB when it establishes the N2 connection to the AMFs.

NOTE 1: This requires RAN3 to define this indication.

4)  If QoS optimization of the N3 connection is required, then the MWAB-gNB is configured (e.g. by OAM) with the mapping of the 5QIs to the DSCP field for TNL used by the UPFs/gNBs in the PLMN/SNPN serving the UE via the MWAB-gNB. This is the used to establish SDFs at the BH-PDU session used for N3 connection(s) based on existing procedures.

NOTE:	Interaction between MWAB-gNB and MWAB-UE are not further defined by SA2.

5)  MWAB-UE can establish more than 1 BH-PDU sessions using different S-NSSAIs in the BH-PLMN for N3, each supporting a set of network slices used in the PLMN/SNPN serving the UEs. OAM of the gnb provides the mapping of the S-NSSAIs used in the PLMN/SNPN served by the gnb to the HPLMN S-NSSAI(s) TO BE used by the MWAB UE

13) MWAB multi-hop prevention can be enforced at the AMF, which can prevent serving a MWAB-UE based on awareness that the the gNB that would be serving it is a MWAB-gNB. This can apply at registration time, or also during N2 HO preparation, or (if Xn will be supported) at path switch.

# End of proposed text