SA WG2 Meeting #163	S2-2407214

Jeju, Korea, May 27 – 31, 2024	(revision of S2-2407134 of S2-2405973)

Source:	LG Electronics, InterDigital, Qualcomm Incorporated, Ericsson, Huawei

Title:	KI#1: Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This paper proposes Evaluation and Conclusion for KI#1.

Discussion

For KI#1 on pre-mission flight planning and in-mission flight monitoring for UAVs, conclusion is needed.

The following solutions for KI#1 on pre-mission flight planning and in-mission flight monitoring for UAVs are described in TR 23.700-59v0.3.0:

-	Solution #1: Support Pre-Mission Planning and In-Mission Monitoring Flight

-	Solution #2: UAV flight path deviation exposure

-	Solution #3: UAV flight planning and monitoring

This pCR is used as baseline for conclusion on KI#1 on pre-mission flight planning and in-mission flight monitoring for UAVs (aka KI#1.1).

Proposal

It is proposed to agree the following changes into TR 23.700-59.

* * * * Start of 1st Change * * * *

All New Texts

8.1	Conclusion for Key Issue #1: Enhancement of NEF services to support service exposure and interactions between MNOs and UTM functions

8.1.1	Conclusion for Key Issue #1 - Pre-mission flight planning and in-mission flight monitoring for UAVs

It is agreed to take the following principles and way forwards for normative work:

-	NEF Assist Pre-mission Flight Planning and NEF Assist In-mission Flight Monitoring should focus on operations by and with UAV while excluding operations by and with UAV-C.

-	Solution#1 with necessary update is considered for developing a framework for NEF-assisted pre-mission flight planning and in-mission flight monitoring by a 3GPP network. However, Solution#1 can be complemented with the following aspects from other solution: the use of QoS Sustainability analytics from Solution#3. In detail, the following aspects can be considered for the update of Solution#1 and complemented with Solution#3.

-	For NEF Assist Pre-mission Flight Planning:

-	Enhance the Movement Behavior Analytics by taking into consideration of the height information of UAV for the analytics generation.

-	Update the description of the NEF service to make the assistance information for pre-mission flight planning clearer, e.g. the assistance information may be the best matched route among the ones provided from the USS/UTM, or potential flight route(s) if candidate flight route(s) is not provided from the USS/UTM.

-	For NEF Assist In-mission Flight Monitoring:

-	Enhance the procedure for USS/UTM to subscribe to the network for UAV location for periodical notification, and based on the collected information to determine e.g. whether UAV is flight in right route/any deviation happened.

-	USS/UTM requests QoS Sustainability Analytics containing the UAV flight path information defined as 3D location waypoints as proposed by Solution#3.

* * * * End of Changes * * * *