3GPP TSG-WG SA2 Meeting #163	S2-2407226

Jeju, Korea, May 27 – May 31, 2024	was S2-2407222

Source:	Huawei, HiSilicon, InterDigital Inc.

Title:	Conclusion on KI#2 of UIA

Document for:	Approval

Agenda Item:	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract: This paper proposes the evaluation and conclusion for KI#2 of FS_UIA_ARC.

# 1. Discussion

This paper proposes the conclusion for KI#2.

# 2. Text Proposal

The evaluation is proposed for KI#2 for incorporation in the UIA_ARC TR23.700-32.

# * * * * First change * * * *  all is new text

# 8	Conclusions

## 8.X	Conclusion for Key Issue #2: Authentication and Authorization of Users and Restrictions on Users

The following bullets are the interim conclusion principles for KI#2:

-	These KI conclusions are based on conclusion of KI#1

-	The authentication of a User Identifier is performed over NAS-MM as defined in clause 8.1.2

-	If the Authentication of a User identifier is carried out by AAA Server, the auth credentials shall be stored and managed by AAA Server

-	If the Authentication of a User identifier is carried via out by AUSF/UDM, the auth credential shall be determined by SA3.

- AAA Server is selected based on the home network of the UE and User Identifier, which is in NAI format (see clause 8.1.1).

- The authentication credentials are not 3GPP credentials.

NOTE 1: The detailed authentication mechanisms are in remit of SA WG3

NOTE 2:	Where user credentials are stored can be decided by SA WG3.

-  AMF or SMF shall be able to use information in UDM to enforce the restriction that only one user shall be active with a UE’s subscription at a given time based on the active user identifier in the UE subscription.

# * * * * End of changes * * * *