3GPP TSG-WG SA2 Meeting #164	S2-2407685

# Maastricht, Netherlands, August 19 - 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5439 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#5, Support of burst size |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the conclusion of KI#5 in TR 23.700-70, Data Burst Size based handling is supported as described in clause 8.5. The related enhancements in clauses 5.37.8.1 and 5.37.8.3 need to be added to support the Data Burst Size related handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Data Burst Size based handling is introduced in clause 5.37.8.1 and 5.37.8.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data Burst Size based handling for XR service is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.8.1, 5.37.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.503 CR 1324 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.37.8	UE power saving management

5.37.8.1	General

The following traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX or to assist efficient radio resource scheduling:

-	UL and/or DL Periodicity;

-	N6 Jitter Information associated with the DL Periodicity;

-	Indication of End of Data Burst;

-    Data Burst Size.

The UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity are provided by the CN to NG RAN via TSCAI.

# * * * * Second change * * * *

5.37.8.3	End of Data Burst Indication and Data Burst Size

An indication of End of Data Burst and/or Data Burst Size may be provided to the NG-RAN by the UPF, e.g. to configure UE power management schemes like connected mode DRX, or to assist efficient radio resource scheduling.

Based on the End of Data Burst Marking Indication in a PCC rule and/or on local operator policies, SMF should request the UPF to detect the last PDU of the data burst and mark the End of Data burst in the GTP-U header of the last PDU in downlink. Based on the Data Burst Size Marking Indication in a PCC rule and/or on local operator policies, SMF should request the UPF to identify the data burst size and mark the Data Burst Size in the GTP-U header of the downlink PDU in the data burst. The SMF may provide the PSA UPF the End of Data Burst Marking Indication, and/or Data Burst Size Marking Indication, and Protocol Description used by the service data flow. The Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1.

According to the End of Data Burst Marking Indication from the SMF, the UPF identifies the last PDU of a Data burst in the DL traffic based on the End indication according to the Protocol Description or UPF implementation and provides an End of Data Burst indication to the NG-RAN over GTP-U of the last PDU of a Data burst.

NOTE:	There can be some packets from the Data Burst received by NG-RAN after the PDU with End of Data Burst Indication if packets are received out of sequence.

According to the Data Burst Size Marking Indication from the SMF, the UPF identifies the data burst size in the DL traffic based on the N6 DL packet protocal, i.e., RTP header extension defined in TS 26.522 [179], and provides Data Burst Size to the NG-RAN over GTP-U of the downlink PDU of a data burst.

# * * *  End of changes * * * *