3GPP TSG-WG SA2 Meeting #164	S2-2407798

# Maastricht, NL, 19-23 August 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4880 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of on-demand broadcast of GNSS assistance data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_OBGAD |  |  |  |  |  | Date: |  |  | 2024-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | AMF event exposure service is enhanced to support LMF’s subscription on the number of UEs, which subscribes given GNSS assistance data type, presence in a geographical area. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new event filter enabling filting UEs which subsciribes given LCS broadcasting assistance data type is added for AMF exposure event ‘Number of UEs presence in a geographical area’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The on-demand broadcast of GNSS assistance data cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2.2.3	Namf_EventExposure service

5.2.2.3.1	General

Service description: This service enables an NF to subscribe and get notified about an Event ID.

Following UE access and mobility information event are considered (Event ID is defined in clause 4.15.1 and Table 4.15.3.1-1):

-	Location Report (TAI, Cell ID, N3IWF/TNGF node, UE local IP address and optionally UDP source port number);

-	UE moving in or out of a subscribed "Area Of Interest" as described in clauses 5.3.4.4 and 5.6.11 of TS 23.501 [2];

-	Number of UEs served by the AMF and located in "Area Of Interest";

-	Time zone changes (UE Time zone);

-	Access Type changes (3GPP access or non-3GPP access);

-	Registration state changes (Registered or Deregistered);

-	Connectivity state changes (IDLE or CONNECTED);

-	UE loss of communication;

-	UE reachability status;

-	UE indication of switching off SMS over NAS service;

-	Subscription Correlation ID change (implicit subscription);

-	UE Type Allocation code (TAC);

-	Frequent mobility re-registration;

-	Subscription Correlation ID addition (implicit subscription);

-	User State Information in 5GS, as described in clause 5.4.4 of TS 23.632 [68];

-	UE access behaviour trends (see clause 4.15.4.2);

-	UE location trends (see clause 4.15.4.2); and

-	Total number of Mobility Management transactions:

-	The Total number of Mobility Management transactions is used to collect the number of MM transactions of a SUPI or Internal Group ID, for example Dispersion Analytics as specified in TS 23.288 [50]. The Total number of transactions is incremented when the NAS signalling transactions from Authentication, Registration, De-Registration, Service Request and UE Configuration Update procedures is completed. Only the periodic reporting mode applies.

In addition to UE access and mobility information event, AMF exposes the "S-NSSAIs per TAI mapping" event providing, per TAI, the related access type and list of supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF). The Event Consumer may use as target of event reporting a list of TAIs, or "any TAI" and may use event filter information including a list of "S-NSSAIs". Whenever there is a change in supported S-NSSAIs (including indication of S-NSSAIs restricted by AMF) for a TAI, the event notification is generated with the updated information.

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for AMF exposure.

NOTE:	The conditions to match can be set based on AMF-associated expected UE Behaviour parameter(s) to only notify the event when the UE's behaviour deviates from its expected UE behaviour as described in TS 23.288 [50].

Table 5.2.2.3.1-1: Example of Event Filters for AMF exposure events

| Event ID | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Location Report | <Parameter Type = LocationFilter, Value = TA1> |
| UE moving in or out of Area of Interest | <Parameter Type = TAI, Value = TA1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = NSI ID, Value = NSI ID1> <Parameter Type = PRA ID, Value = PRA ID value> <Parameter Type = RAN Node ID, Value = RAN Node ID value> <Parameter Type = Cell ID, Value = Cell ID value> <Parameter Type = RAN timing synchronization status change event > (NOTE 1) (NOTE 2) <Parameter Type = Adjust AoI based on RA, Value = Yes or No> to indicate that AMF may adjust the received AoI depending on UE's current Registration Area. Absence of this parameter in the request is interpreted as "AoI remains unchanged". <Parameter Type = Notify the consumer considering UE identity, Value = List of SUPIs or Internal Group ID > The parameter may be included when the request is targeted to Any UE. Absence of this parameter in the request is interpreted as "AMF reports an event regardless to UE's identity triggering the event". (NOTE 2) <Parameter Type = Notify the consumer considering DNN/S-NSSAI, Value = DNN and/or S-NSSAI> Absence of this parameter in the request is interpreted as "AMF reports the event without checking to which DNN/S-NSSAI the UE has PDU sessions established". (NOTE 2) |
| Access Type | <Parameter Type=AN Type, Value=3GPP Access"> |
| Location | <Parameter Type=TAI, Value=wildcard> (to report any TAI change) |
| Location | <Parameter Type=TAI Value=abnormal> (to report only when the TAI deviates from expected values based on Expected UE Moving Trajectory). |
| Reachability Filter | Applicable to the event UE reachability. Value = UE reachability status change or UE reachable for DL traffic. Absence of this parameter in UE reachability event request is interpreted as "UE reachability status change". |
| Total number of Transactions | <Parameter Type = TAI, Value = TA1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| Number of UEs present in a geographical area | <Parameter Type = UE Type, Value = Aerial UE> <Parameter Type = PDU session status, Value = PDU session established for DNN subject to aerial services> <Parameter Type = LCS broadcasting assistance data type, Value = A bitmap indicates broadcast location assistance data types which to be broadcasted using ciphering via corresponding positioning SIB type (see Table 7.1-3 in clause 7.1 of TS 23.273 and cluase 6.1.6.2.72 of TS 29.503)> (NOTE 3) |
| NOTE 1: The Parameter Type = RAN timing synchronization status change event can be present with Parameter Type defining a RAN Node ID. NOTE 2: The use of event filter parameters is described in clause 5.3.4.4 of TS 23.501 [2]. NOTE 3: The mapping relationship between assistance data element and corresponding positioning SIB type is described in clause 7.2 of 3GPP TS 37.355 [56]. |  |

The following service operations are defined for the Namf_EventExposure service:

-	Namf_EventExposure_Subscribe.

-	Namf_EventExposure_UnSubscribe.

-	Namf_EventExposure_Notify.

# * * * * End of changes * * * *