3GPP TSG-WG SA2 Meeting #164	S2-2407918

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1144 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Vertical Federated Learning General Description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The new functionalinity of Vertical Federated Learning has been concluded in TR 23.700-84. The general principles of the VFL support have been agreed in Clause 8.2 TR 23.700-84. The agreed principles of overall VFL support need to be reflected in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Definitions agreed in the R19 study item phase are included. 2) The description of NWDAF capabilities are extended to include analtyics generation and model training based on VFL support. 3) General principles of VFL support agreed in R19 study item are reflected in a new Clause describing the Vertical Federated Learning Capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Concepts agreed in the Study Item phase for R19 FS_AIML_CN are not reflected at the NWDAF architecture and description in the TS 23.288. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.1, 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change  * * * *

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4]. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Analytics Accuracy Information: Represent a performance measure of an analytics ID provided by an NWDAF containing AnLF, which is composed of the number of correct predictions of the analytics ID out of all predictions and the corresponding number of samples.

ML Model Accuracy Information: Represent a performance measure of a ML Model provided by an NWDAF containing MTLF, which is composed of the number of correct predictions by the ML Model out of all predictions and the corresponding number of samples.

Analytics Feedback Information: Indicates that the consumer NF has taken action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data.

Horizontal Federated Learning (HFL): A federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs).

Vertical Federated Learning (VFL): A federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs).

Label: A label is the training objective in supervised machine learning.

VFL Server: An NWDAF or AF that integrates local training results, computes gradient information or loss information and send them to VFL client(s) for the local ML model update in VFL training process. It also coordinates the VFL training process by discovering and selecting VFL clients. In VFL inference process, the VFL server aggregates local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer. Only one VFL server exists for each VFL process.

VFL Client: An NWDAF or AF that holds the local data set and local ML Model and performs local training and inference as asked by VFL Server. There may exist multiple VFL Clients in VFL training and inference.

# * * * * Second change * * * *

4.1	General

The NWDAF (Network Data Analytics Function) is part of the architecture specified in TS 23.501 [2] and uses the mechanisms and interfaces specified for 5GC in TS 23.501 [2] and OAM services (see clause 6.2.3.1).

The NWDAF may support the following functionalities:

-	Data collection based on subscription to events provided by AMF, SMF, UPF, PCF, UDM, NSACF, AF (directly or via NEF) and OAM;

-	Analytics and Data collection using the DCCF (Data Collection Coordination Function);

-	Retrieval of information from data repositories (e.g. from UDR via UDM for subscriber-related information or optinally via NEF(PFDF) for PFD information);

-	Data collection of location information using LCS (finer granularity location information determined by LMF);

-	Storage and retrieval of information from ADRF (Analytics Data Repository Function);

-	Analytics and Data collection from MFAF (Messaging Framework Adaptor Function);

-	Retrieval of information about NFs (e.g. from NRF for NF-related information);

-	On demand provision of analytics to consumers, as specified in clause 6.

-	Provision of bulked data related to Analytics ID(s).

-	Provision of Accuracy information about Analytics ID(s).

-	Provision or retrieval of ML Model Accuracy Information or ML Model accuracy degradation about a ML Model.

-	Joint model training via Vertical Federated Learning among NWDAFs or between NWDAF(s) and AF(s).

-	Joint analytics generation via Vertical Federated Learning among NWDAFs or between NWDAF(s) and AF(s).

A single instance or multiple instances of NWDAF may be deployed in a PLMN. If multiple NWDAF instances are deployed, the architecture supports deploying the NWDAF as a central NF, as a collection of distributed NFs, or as a combination of both. If multiple NWDAF instances are deployed, an NWDAF can act as an aggregate point (i.e. Aggregator NWDAF) and collect analytics information from other NWDAFs, which may have different Serving Areas, to produce the aggregated analytics (per Analytics ID), possibly with Analytics generated by itself.

NOTE 1:	When multiple NWDAFs exist, not all of them need to be able to provide the same type of analytics results, i.e. some of them can be specialized in providing certain types of analytics. An Analytics ID information element is used to identify the type of supported analytics that NWDAF can generate.

NOTE 2:	NWDAF instance(s) can be collocated with a 5GS NF.

# * * * * Third change (ALL NEW TEXT ) * * * *

5.X	Vertical Federated Learning (VFL) Support

Vertical Federated Learning (VFL) is a machine learning technique in the core network used for:

-	joint ML Model training of multiple local ML Models across multiple decentralized entities, referred as VFL Training, and

-	analytics inference performed across multiple decentralized entites using previously jointly trained ML models, referred as VFL Inference.

Vertical Federated Learning is supported among multiple NWDAFs in a single PLMN or between AF(s) and NWDAF(s) in a single PLMN.

An NWDAF or AF supporting VFL process, e.g., training, inference, holds a local ML Model different from the other ML models involved in the same VFL process, and a local dataset, without exchanging (or sharing) local data set or ML models.

NOTE:	The local data set in Vertical Federated Learning process means that the NWDAF or AF has different feature space (e.g., uses different input information for the local ML model) for the same samples (e.g., UE IDs).

In a VFL process, NWDAF or AF have the VFL Server or VFL Client capability and there is only one NWDAF or AF acting as VFL Server. When an NWDAF is acting as VFL Server in a VFL Process there could be one or multiple NWDAF(s) and/or AF(s) acting as VFL Clients for the same VFL process. When an AF is acting as VFL Server there could be one or multiple NWDAF(s) acting as VFL Client for the same VFL process. Whenever an untrusted AF is involved in the VFL process, the interactions with the AF are via NEF.

NWDAF(s) and AF(s) with VFL capabilities register to NRF with NF Profile including the VFL capability information. Details in Clause 5.2.

Only the NWDAF or AF acting as VFL Server holds the labels used for determining the completion of the VFL Training and/or the accuracy of the VFL Inference.

VFL Server functionalities include:

-	During VFL Training process:

-	discovers VFL Clients candidates to participate in the VFL training.

-	triggers sample alignment and generates the intersection of samples for the VFL training process.

-	determines the final list of VFL Clients supporting the samples for VFL training.

Editor's note:	When untrusted AF is acting as VFL Server, it is FFS to determine whether and how NEF is involved in generating the intersection of candidate samples and determining the final list of participants.

-	provides the initial ML Model(s) to the VFL client(s) based on request from the VFL client(s).

-	allocates the VFL Correlation identifier (VFL Correlation ID), associated with the distributed ML Models (i.e., each local ML Model of the VFL Participants) in the VFL joint model training process and used to correlate the VFL participants during the VFL training and subsequent VFL inference processes.

-	aggregates intermediate results from VFL client(s), trains a local model, computes intermediate results based on its local ML model, and sends the intermediate results towards VFL Clients involved in the same VFL training process

-	holds the labels for a VFL training process and uses the labels to determine the VFL training process is concluded.

-	During VFL inference process:

-	determines corresponding VFL client(s) for VFL Inference process based on the same VFL Correlation ID used in the VFL Training process.

-	aggregates local inference results from VFL clients.

-	generates the final VFL inference result.

-	provides the analytics output based on the final VFL inference result to the Analytics Consumer.

VFL Client functionalities include:

-	VFL Training process:

-	performs sample alignment with the VFL Server.

-	performs local training as tasked by the VFL Server with the available local dataset and local ML model.

-	reports the intermediary results of the local VFL training process to the VFL Server.

-	receives updated VFL training information from the VFL Server, that is used to update its local ML model information and training process.

-	VFL Inference process:

-	generates VFL inference results based on a ML model associated with a VFL Correlation ID.

-	provide VFL inference results to VFL Server.

# * * * * End of changes * * * *