SA WG2 Meeting #164	S2-2407960

August 19 – 23, 2024, Maastricht, The Netherlands

Source:	Ericsson

Title:	Handling of MPQUIC-E over 3GPP access

Document for:	Discussion / Approval

Agenda Item:	19.13.2

Work Item / Release:	MASSS / Rel-19

Abstract of the contribution: This contribution discusses how to handle Ethernet header compression when MPQUIC-E is used.

# Discussion

In the conclusions for KI#2.1, the following NOTE is captured:

NOTE 1:	How to address the issue with Ethernet header compression in RAN, without any RAN impact, when MPQUIC-E is used is to be discussed during normative phase.

The background to the NOTE is that when PDU Session Type Ethernet is used, the PDCP layer in NG-RAN and UE may perform Ethernet header compressions (as described e.g. in TS 38.300 and TS 38.323). However, when MPQUIC is used to proxy Ethernet frames, the lowest protocol layer in the PDU will be IP instead of Ethernet, which may cause issues in a gNB that tries to apply Ethernet Header Compression.

Below we describe three solution alternatives, with pros and cons, and propose a way forward.

# Discussion

Alternative 1: Indicate "Unstructured PDU Session type" to RAN

In this alternative, if SMF is configured to apply MPQUIC-E for that DNN/slice, the SMF indicates to RAN that the PDU Session Type is Unstructured. This ensures that RAN will not perform Ethernet HC. RAN will also not indicate to the UE to use Ethernet HC, which ensures that the UE will not perform EHC in uplink.

Pros:

- 	Simple solution and small impacts on SMF. No impacts to the UE or UPF.

- 	This option avoids new functionality to process Ethernet headers (compared to alt 2 and 3) and can more easily be extended in the future if desired.

Cons:

- 	Not possible to perform HC on Ethernet traffic flows that are using ATSSS-LL

- 	Not possible to perform HC on Ethernet traffic using MPQUIC, unless IETF enhances the connect-ethernet solution

Alternative 2: Insert "dummy" Ethernet header below the QUIC/UDP/IP packet

In this alternative, the UE and UPF insert a "dummy" Ethernet header as the lowest header in the PDU. RAN (and UE) may perform Ethernet header compression on this header.  Static values of MAC addresses etc may be used, that allows the UE/UPF to distinguish it from a real Ethernet header. This Ethernet header is discarded by the receiver (UE, UPF). This alternative is illustrated in Fig 1.

Fig 1. Alternative 2

Pros

- 	Possible to perform HC on Ethernet traffic flows that are using ATSSS-LL

Cons

- 	Extra overhead added to MPQUIC traffic (two bytes over the air, if EHC is used).

Alternative 3: Put the real Ethernet header as the lowest header in the PDU.

In this alternative, the sender (UE in uplink, UPF in downlink) takes the real Ethernet header and puts it as the outer header of the QUIC/UDP/IP packet before sending it to UPF. No Ethernet header is included in the proxied payload. The receiver (UPF in uplink, UE in downlink) recovers the PDU by combining the Ethernet header with the payload. This alternative is illustrated in Fig 2.

Fig 2. Alternative 3

Pros

- 	Possible to perform HC on Ethernet traffic flows that are using ATSSS-LL

- 	No extra overhead added to MPQUIC traffic. RAN and UE can do HC on the Ethernet header also in this case.

Cons

- 	Requires Ethernet header manipulation in UE and UPF.

# Proposal

It is proposed to discuss the options and decide a way forward. To the authors, alternative 1 seems preferable as it is the simplest option with least impact on UE and NFs. A CR implementing alternative 1 is available in S2-2407961.