3GPP TSG-WG SA2 Meeting #164	S2-2408662

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1187 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling VFL training in 5GC with AF as Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introducing VFL training capability in 5GC, when AF is acting as the VFL server |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new clause is added to describe VFL training with AF as the VFL Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training with AF as VFL Server will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.2X	VFL training with AF as VFL Server

Figure 6.2X-1: VFL Training Procedure with AF as VFL server

- AF as a VFL server may initiate VFL training with VFL clients after sample and feature alignment by sending Nnef_VFLTRaining Subscribe to the NEF. Nnef_VFLTraining Subscribe may include analytic ID, VFL model correlation ID, ML model ID, the requested performance metric (e.g., required accuracy of trained ML model), and additional information for ML Model. Additional information for ML Model may include information for initial ML model. Additional information for initial ML Model may include different information per VFL clients according to VFL client’s registered capability (e.g., supported ML model and additional information for ML model including indication of preconfigured ML Model or configurable ML model, input feature, and output features). When VFL clients support configuration of the requested ML model for analytic ID, additional information for ML Model may include initial ML model parameters to configure the VFL local model to VFL client, e.g., ground truth data. VFL model correlation ID is used to exchange VFL server and VFL clients to exchange the interim result and the final result for VFL training and will be used to correlate the trained ML model for an Analytic ID for VFL inference. AF may also include the requested performance metric (e.g. required accuracy of trained ML model) in the request.

Editor’s Note: Whether and how VFL server aware Analytics ID is FFS.

Editor’s Note: When multiple NWDAFs as VFL clients are involved, how AF may communicates with multiple NWADF is FFS.

2.	NEF may map the external UE IDs to the internal UE IDs based on the information exchanged with the UDM.

3.	NEF sends the Nnwdaf_MLModelTraining_Subscribe to each VFL client including the internal UE IDs, VFL model correlation ID, ML model ID, and additional information for ML Model.

NOTE : When AF is a trusted AF, AF may send the Nnwdaf_MLModelProvision_Subscribe to the NWDAF.

4.	VFL client may collect data from NF data provider for the requested list of UEs and features list. VFL client may perform local model training based on the initial local model with the collected data.

VFL client may acquire Ground truth data for local ML model training from NF data provider based on ML model configuration included in information for initial ML model from VFL server.

5.	VFL client may send the Nnwdaf_MLModelTraining_Notify to the NEF including analytic ID, VFL model correlation ID, ML model ID, and additiona information for ML model. The additional information for ML model may include intermediate results identification information, which identifies the intermediate results in the VFL training service, e.g. time stamp.

6.	NEF may send Nnef_VFLTraining Notify to the AF which include analytic ID, VFL model correlation ID, ML model ID, and additiona information for ML model.

7.	AF, based on the VFL model correlation ID, may use the intermediate result to generate the training result for global ML model for analytic ID and verify the training result satisfy the performance metric (e.g., accuracy requirements, etc). When needed, AF may determine to update local ML model of each VFL client for further training (e.g. the updated ML model for local training, updated features, updated UE’s list, updated service area and/or time period). VFL server may send Nnef_VFLTraining Subscribe which include analytic ID, VFL model correlation ID, ML model ID, and additional information for ML model. Additional information for ML model may include information for updated local ML model, other parameters (e.g., loss function information, etc) to be applied at the updated local ML model training, and performance metric (e.g. accuracy level for local ML model training) to be applied at the updated local ML model training.

8.	NEF may send the Nnwdaf_MLModelTraining_Subscribe to the VFL client which include analytic ID, VFL model correlation ID, ML model ID, and additional information for ML model.

9.	VFL client may update local ML model and perform local training (as shown in step 4). The step 4 to step 9 may repeate until VFL server determine the termination condition is reached (e.g., performance requirement for global ML model training is satisfied using the trained local ML model.)

10.	VFL server may send Nnef_VFLTraining Unsubscribe to finish VFL training with indication VFL training completed. After successful VFL training completion, VFL server store the trained ML model with trained local ML model information from each VFL client and associated it to VFL model correlation ID.

11.	NEF sends the Nnwdaf_MLModelTraining_Unsubscribe to the NWDAF to terminate the VFL training progress. When receiving Nnwdaf_MLModelProvision_Unsubscribe with indication VFL training completed, each VFL client store the trained local ML model information for the analytic ID and associate it to VFL model correlation ID.

# * * * * End of changes * * * *