3GPP TSG-SA2 Meeting #164	S2-2409549r01

# Maastricht, Netherlands, 19 - 23 August 2024	       	 (revision of S2-2409423)

Title:	[draft] LS on XRM metadata exchange between 3GPP Core and an Application server

Response to:

Release:	Rel-19

Work Item:	FS_XRM_Ph2

Source:	Nokia, Will be SA2

To:

IETF Multiplexed Application Substrate over QUIC Encryption (MASQUE) WG <statements@ietf.org>

Cc:	liaison-coordination@iab.org, 3gpp-liaison@ietf.org.

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

3GPP SA2 is considering solutions where an Application Server (AS) collaborates with 5G network by providing metadata to help improving the QoS handling in the Radio Access Network (RAN).

In one of those solutions the 3GPP User Plane Function (UPF) detects an e2e QUIC connection and establishes a UDP tunnel to a proxy that may be co-located with the target AS. The tunnel is used to send metadata related to sets of downlink packets. Note that in this solution, the connect-UDP client is different from the client of the e2e connection.

In the case of media packets being transported over QUIC, the forwarded mode of the “QUIC-Aware Proxying Using HTTP” is proposed, to optimize the use of node and network resources. In this case, metadata is encoded into forwarded packets using a new packet transform.

Since the connect-UDP client is separate from the end-to-end QUIC client it can not coordinate as tightly with the client QUIC stack as is assumed by the “QUIC-Aware Proxying Using HTTP” draft. The solution proposes that the connect-UDP client in the UPF extracts Source CIDs from e2e QUIC Initial packets and uses these values in REGISTER_CLIENT_CID and REGISTER_TARGET_CID capsules. Prior to forwarding short header packets, the connect-UDP client and the proxy validate that destination CIDs are identical to already registered CIDs, if not, the packets are sent in tunnelled-mode. The connect-UDP client can attempt to register new CIDs upon detection of changes. This is assumed to work well if the new CID is of the same length, or longer than the CID observed in an Initial packet. If CID field (with an assumed length) is not stable, the traffic will be tunneled instead of forwarded.

It should be noted that out-of-band coordination between the application and the network is an assumed pre-requisite of this solution. Specifically, an Application Function (AF) provides the network with assistance information for traffic detection.

The 3GPP SA2 working group would like the IETF to provide feedback on the aspects of the solution relating to CID detection and registration, given the somewhat different deployment model than what is assumed in “QUIC-Aware Proxying Using HTTP”. 3GPP SA2 working group would especially like IETF to confirm or further clarify the following points:

- Whether a UDP tunnel client at the UPF will be able to initiate forward mode based on the e2e QUIC connection IDs from the long header packets.

- Whether a UDP tunnel client at the UPF can observe changes in the e2e QUIC connection IDs in subsequent short header packets and register the new e2e QUIC connection IDs to be able to resume using the forwarded mode, provided that the length of the QUIC connection IDs has not changed.

In case that the length of the e2e QUIC connection IDs change, whether the UDP tunnel client at the UPF and the proxy at least will be able to communicate in tunneled mode.

UE

UPF

- Actions:

To IETF Multiplexed Application Substrate over QUIC Encryption WG and IETF Media Over QUIC WG::

ACTION: 	SA2 kindly asks IETF Multiplexed Application Substrate over QUIC Encryption WG and IETF Media Over QUIC WG to answer to the questions above.

- Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#165 | October 14th – 18th, 2024 | Hyderabad | India |
| SA2#166 | November 18th –22nd, 2024 | Orlando | USA |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/