3GPP TSG-SA2 Meeting #164	S2-2409549r05

# Maastricht, Netherlands, 19 - 23 August 2024	       	 (revision of S2-2409423)

Title:	[draft] LS on XRM metadata exchange between 3GPP Core and an Application server

Response to:

Release:	Rel-19

Work Item:	FS_XRM_Ph2

Source:	Nokia, Will be SA2

To:

IETF Multiplexed Application Substrate over QUIC Encryption (MASQUE) WG <statements@ietf.org>

Cc:	liaison-coordination@iab.org, 3gpp-liaison@ietf.org., MASQUE WG chairs: Dennis Jackson, Eric Kinnear.

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

3GPP SA2 is working to provide dedicated Quality of Service (QoS) for XRM (eXtended Reality) traffic flows over 3GPP networks (5G); for this purpose, a 3GPP network needs in band (along with the packets) certain metadata about the structure and characteristics of the application media content in the traffic flows even though the traffic flows are subject to end-to-end ciphering (e.g. provided by QUIC).

3GPP SA2 is considering solutions where an Application Server (AS) collaborates with 5G network by providing XRM metadata to help improving the QoS handling in the Radio Access Network (RAN).

In one of those solutions the 3GPP User Plane Function (UPF) detects an e2e QUIC connection and establishes a UDP tunnel (per RFC 9298 ) to a proxy that may be co-located with the target AS. The tunnel is used to send XRM related packets as well as related XRM metadata. Note that in this solution, the connect-UDP client (UPF) is different from the client of the e2e (e.g. QUIC) connection (the UE).

In the case of media packets being transported over QUIC, the forwarded mode of the “QUIC-Aware Proxying Using HTTP” is proposed, to optimize the use of node and network resources. In this case, XRM metadata are encoded into forwarded packets using a new packet transform to be defined by 3GPP.

Since the connect-UDP client (in the UPF) is separate from the end-to-end QUIC client (in the UE) it cannot coordinate as tightly with the client QUIC stack as is assumed by the “QUIC-Aware Proxying Using HTTP” draft. The solution proposes that the connect-UDP client in the UPF extracts Source CIDs from e2e QUIC Initial packets and uses these values in REGISTER_CLIENT_CID and REGISTER_TARGET_CID capsules. Prior to forwarding short header packets, the connect-UDP client and the proxy validate that destination CIDs are identical to already registered CIDs, if not, the packets are sent in tunneled-mode.

The connect-UDP client can attempt to register new CIDs upon detection of changes. This is assumed to work well if the new CID is of the same length, or longer than the CID observed in an Initial packet. If CID field (with an assumed length) is not stable, the traffic will be tunneled instead of forwarded.

It should be noted that out-of-band coordination between the application and the network is an assumed pre-requisite of this solution. Specifically, an Application Function (AF) provides the network with assistance information for traffic detection.

The 3GPP SA2 working group would like the IETF to provide feedback on the aspects of the solution relating to CID detection and registration, given the somewhat different deployment model than what is assumed in “QUIC-Aware Proxying Using HTTP”. 3GPP SA2 working group would especially like IETF to confirm or further clarify the following points:

- Whether the connect-UDP client (in the UPF) will be able to initiate forward mode based on the e2e QUIC connection IDs from the long header packets.

- Whether the connect-UDP client (in the UPF), upon observing changes in the e2e QUIC (Destination ) Connection IDs in subsequent short header packets, can register the new e2e QUIC connection IDs to be able to resume using the forwarded mode,

QUIC allows new end-to-end CID values to be negotiated between the UE (QUIC client) and the Target (QUIC Server) anytime during the lifetime of the end-to-end QUIC connection. When this happens, the connect-UDP client (in the UPF) would not be able to learn the actual new end-to-end CID values from the NEW_CONNECTION_ID frames that are end to end encrypted. But the UPF can detect new end-to-end CID values are used in QUIC packets it receives with short headers, even though UPF can only do this detection based on the length of the initial end-to-end CID having been used (as QUIC packets with short headers do not contain CID length).

Once it has detected a new end-to-end CID is being used, the UPF, acting as a QUIC-Aware proxying client, can negotiate the usage of a new virtual CID to be associated with the (possibly subset of the) new end-to-end CID it has detected. If CID length) is reduced, the traffic will likely end up be tunneled instead of forwarded as the UPF is likely to detect too frequent CID changes , since the original length is assumed by the connect-UDP client at the UPF.

IETF MASQUE WG are asked to provide any feedback on this foreseen mechanism

- In case that the length of the e2e QUIC connection IDs change, whether the connect-UDP client (in the UPF) and the proxy at least will be able to communicate in tunneled mode.

- Actions:

To IETF Multiplexed Application Substrate over QUIC Encryption WG.

ACTION: 	SA2 kindly asks IETF Multiplexed Application Substrate over QUIC Encryption WG to answer to the questions above.

- Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#165 | October 14th – 18th, 2024 | Hyderabad | India |
| SA2#166 | November 18th –22nd, 2024 | Orlando | USA |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/