3GPP TSG WG2 Meeting #165	S2-2411013

Hyderabad, India,  2024-10-14 -- 2024-10-18	(revision of S2-2410734)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5454 | rev | 8 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Handling of Headers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Vodafone, ETRI, ZTE, Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specification of KI#3 outcome Enhancement of UPF handling of headers according to approved WID: SP-240995 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to incorporate the agreed enhancements for KI#3 as in WID, as follows: - Feature description - Description of how Traffic Influence on routing API is used - Adding text to UPF descriptions. FAR is used to send instructions to UPF. Summary of change was not correct in last revision (tdoc S2-2409555) discussed in SA2 #164 and because of this, it was not approved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of Headers Enhancements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.16, 5.6.x (new), 5.6.x.1 (new), 5.6.x.2 (new), 5.8.2.x (new), 5.8.5.6, 5.8.2.17, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 4877, TS 23.503 CR 1329 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR refers from new clause 5.6.x to a new clause 6.1.3.x in TS 23.503 (CR 1329) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * first change  * * * *

### 4.2.16	Architecture to support User Plane Information Exposure via a service-based interface

As depicted in Figure 4.2.16-1, the 5G System architecture allows user plane information exposure to some NFs via service-based interface in UPF.

Figure 4.2.16-1: Architecture to support User Plane Information Exposure via a service-based interface

NOTE 1:	In this Release of the specification, only NWDAF/DCCF/MFAF, NEF/AF, SMF and TSNAF/TSCTSF are considered as the receiver of the UPF event notifications.

NOTE 2:	UPF information exposure is not restricted to SBI interface, i.e. reporting via PFCP over N4 to SMF is still applicable.

Not all events can be subscribed to UPF directly. The details and constraints for the subscription to UPF event exposure service (i.e. direct vs. indirect) and the information exposed to certain NFs by UPF, as well as the information contained in the event notifications, are defined in clause 5.2.26.2 of TS 23.502 [3] and clause 5.8.2.17.

# * * * * next change (all new) * * * *

### 5.6.x	Handling of Payload Headers

#### 5.6.x.1	General

Handling of Payload Headers is an optional feature that allows exchange of information in-band in the user plane packets between the application (in the UE or in the application server) and UPF in the 5GS by supporting insertion, replacement, or removal of payload headers of PDUs. Handling of Payload Headers can be requested by the AF.

NOTE 1: 	The Handling of Payload Headers is not limited to the protocol header but can be any field or tag supported by a protocol.

NOTE 2:	Handling of Payload Headers is expected to be used instead of Header Enrichment (see clause 5.8.5.6) as it allows more actions than just insertion, supports different protocols and allows the usage of actions in both directions.

The content of this clause applies to non-roaming and to Home Routed roaming scenario, i.e. to cases where the involved entities (AF, PCF, SMF, UPF) belong to the Home PLMN and the AF has an agreement with the Home PLMN.

The PCF controls Handling of Payload Headers in UPF by provisioning Header Handling Control information in the PCC rule. The PCF derives the Header Handling Control information as described in 6.1.3.x of TS 23.503 [45] based on the AF request and provides it to the SMF in the related PCC rule as described in clause 6.3.1 of TS 23.503 [45]. SMF instructs UPF via N4 interactions by means of FAR (see clause 5.8.5.6).

The UPF performs Handling of Payload Headers and may notify events related to the Handling of Payload Headers to SMF, to NEF/AF or to both as described in clause 5.6.x.2.

#### 5.6.x.2	Application Function influence on Handling of Payload Headers

An AF may request the Handling of Payload Headers in order to insert, replace, or remove payload headers of PDUs as well as to subscribe to notifications on events related to detection of payload headers or related to the actions performed on payload headers.

For the Handling of Payload Headers, it is assumed that a service level agreement (SLA) exists between the operator and the third party AF. The mechanism(s) required for detection and actions performed on payload headers (e.g. protocol layer, type of encryption, etc.) is agreed as part of the SLA. The reference(s) to the UPF configuration(s) corresponding to the header detection as well as reference(s) to SMF configuration corresponding to SMF context information are also part of the SLA. The AF is required to provide such reference(s) in the AF requests for Handling of Payload Headers.

In the non-roaming scenario, the features Handling of Payload Headers, Application Function influence on Service Function Chaining (as defined in clause 5.6.16) and Application Function influence on traffic routing (as defined in clause 5.6.7) can be applied simultaneously.

In the Home Routed roaming scenario, the features Handling of Payload Headers and Application Function influence on Service Function Chaining (as defined in clause 5.6.16) can be applied simultaneously.

The AF requests for Handling of Payload Headers are sent to the PCF via N5 (in the case of requests targeting specific on-going PDU Sessions of individual UE(s), for an AF allowed to interact directly with the 5GC NFs) or via the NEF. The AF requests that target existing or future PDU Sessions of multiple UE(s) or of any UE are sent via the NEF and may target multiple PCF(s), as described in clause 6.3.7.2. The PCF(s) transform(s) the Header Handling Control information provided in the AF requests into Header Handling Control information of the PCC rules that apply to the respective PDU Sessions. When the AF has subscribed to events related to Handling of Payload Headers, such notifications are sent by the UPF(s) either directly to the AF or via an NEF.

When the request is sent via NEF, Nnef_TrafficInfluence service is used. The procedures for AF to request Handling of Payload Headers are described in clause 4.3.6 in TS 23.502 [3].

The AF requests may contain the information as described in the Table 5.6.x.2-1:

Table 5.6.x.2-1: Information elements contained in AF request

| Information Name | Applicable for PCF or NEF (NOTE 1) | Applicable for NEF only | Category |
| --- | --- | --- | --- |
| Traffic Description | Defines the target traffic on which to apply handling of headers, represented by the combination of DNN and optionally S-NSSAI and application identifier or traffic filtering information. | The target traffic can be represented by AF-Service-Identifier, | Mandatory |
| Target UE Identifier(s) | Indicates the UE(s) that the request is targeting, i.e. an individual UE, a group of UE represented by Internal Group Identifier(s) (NOTE 2), or any UE accessing the combination of DNN and S-NSSAI | GPSI can be applied to identify the individual UE, or External Group Identifier can be applied to identify a group of UEs. | Mandatory |
| Spatial Validity Condition | Indicates that the request applies only to the traffic of UE(s) located in the specified location, represented by areas of validity. | The specified location can be represented by geographical area. | Optional |
| Temporal Validity Condition | Time interval(s) when the request applies or duration(s). | N/A | Optional |
| AF transaction identifier | The AF transaction identifier refers to the AF request. | N/A | Mandatory |
| Header Handling Control information |  |  |  |
| Header Detection Reference | A reference to a UPF configuration which defines how to detect the protocol or the message in a protocol for which to perform the header handling actions. | N/A | Mandatory |
| Header Detection Support Information (NOTE 3) | Any dynamic information provided by the AF which is required for the detection of the headers and cannot be preconfigured for the Header Detection Reference. | N/A | Optional |
| Header Handling Reporting Endpoints | List of notifications endpoints, i.e. per notification endpoint a Notification Target Address and a Notification Correlation ID. | N/A | Optional |
| Header Handling Control Reference (NOTE 5) | A reference to a Header Handling Action related information pre-configured in the UPF. | N/A | Optional |
| Header Handling Direction (NOTE 4) | Indicates if the header handling applies to UL or DL direction. | N/A | Optional |
| Header Handling Action (NOTE 4) | Indicates the action to be performed on a specific header field. | N/A | Optional |
| Header Information (NOTE 4) | A reference to a UPF configuration which defines how to identify or build a specific header field for which to perform the header handling action. | N/A | Optional |
| Header Value (NOTE 4) | A string providing the value (or a reference to information to be provided by SMF) of the specific header field. | N/A | Optional |
| Header Handling Condition (NOTE 4) | Indicates the condition for performing the header handling action. | N/A | Optional |
| Header Handling Reporting (NOTE 4) | Indicates whether reporting is requested for the performed Header Handling Action and the notification endpoint(s). | N/A | Optional |
| NOTE 1: When the AF request targets existing or future PDU Sessions of multiple UE(s) or of any UE and is sent via the NEF, as described in clause 6.3.7.2, the information is stored in the UDR by the NEF and notified to the PCF by the UDR. NOTE 2: Internal Group ID can only be used by an AF controlled by the operator and only towards PCF. NOTE 3: The NEF, PCF and SMF do not need to understand the Header Detection Support Information. NOTE 4: These parameters are provided together to request a Header Handling Action. Multiple sets of these parameters (and thus multiple header handling actions) can be provided. NOTE 5: If a Header Handling Control Reference is provided, one or more of these parameters (marked by NOTE 4) can be provided to overwrite Header Handling Control information that is pre-configured in the UPF as per the Header Handling Control Reference. Multiple Header Handling Control References can be provided. |  |  |  |

For each information element mentioned above as part of Header Handling Control Information in the AF request, a detailed description follows:

1)	Header Detection Reference:

A reference to a UPF configuration which defines how to detect the protocol or the message in a protocol for which to perform the header handling actions (e.g. information about protocol layer, type of encryption, message type etc).

2)	Header Detection Support Information:

Header Detection Support Information is not standardised, but it can be interpreted by UPF based on SLA. It is sent transparently by NEF, PCF and SMF to UPF. It includes dynamic information provided by the AF which is required for the detection of the headers and cannot be preconfigured for the Header Detection Reference in UPF.

3) 	Header Handling Reporting Endpoints:

A list of notifications endpoints for the notifications related to the Handling of Payload Headers. Per notification endpoint in the list, a Notification Target Address and a Notification Correlation ID has to be provided.

4) 	Header Handling Direction:

Header Handling Direction indicates to which direction the Header Handling Action applies to. It can have the values uplink (UL) and/or downlink (DL).

5)	Header Handling Action:

The action to be performed on a specific header field. One of the following can be requested:

- 	Detect. It is used to request the detection of a header field that is identified based on the UPF configuration referenced by the Header Information parameter and the information contained in the Header Value parameter, if it is provided.

-	Remove. It is used to request the removal of a header field that is identified based on the UPF configuration referenced by the Header Information parameter and the information contained in the Header Value parameter, if it is provided.

-	Replace. It is used to request the replacement of information in a header field that is identified based on the UPF configuration referenced by the Header Information parameter. The information to be replaced in the header field is also defined by that UPF configuration. The Header Value contains information that is used for the replacement.

-	Insert. It is used to add a header field according to the UPF configuration referenced by the Header Information parameter. The Header Value contains information that is used for the insertion, if it is provided.

When multiple actions are requested on the target traffic, the enforcement of the Header Handling Action is performed in the following order:  Detect, Remove, Replace, and Insert.

NOTE 1:	The execution order is relevant to build a request that has the intended impact on the traffic. For example, a header that is removed cannot be replaced.

6)	Header Information:

A reference to a UPF configuration which defines how to identify or build a specific header field for which to perform the header handling action.

7)	Header Value:

A string providing the value (or a reference to information to be provided by SMF) of the specific header field relevant for the action (i.e. to detect in case of actions Detect or Remove, and for updating in case of actions Insert or Replace). Header Value may optionally be provided apart from the Replace action for which it is mandatory.

8)	Header Handling Condition:

This defines how to apply the Header Handling Action. It may have one of the following values: every match or only for the first match.

NOTE 2:	In addition, Spatial Validity Condition and Temporary Validity Condition parameters in the request can be used to indicate that the request applies only in certain UE location(s) or during certain time interval(s), respectively.

9)	Header Handling Reporting:

Indicates whether reporting is requested for the performed Header Handling Action or not. If reporting is requested, the notification endpoint(s) for this reporting have to be indicated.

10)	Header Handling Control Reference:

A reference to a Header Handling Action related information that is pre-configured in the UPF. This pre-configuration in UPF comprise parameters 4-9. The Header Handling Control Reference can be provided in the AF request instead of providing the parameters of the Header Handling Control Information individually.

NOTE 3: 	The reference to a Header Handling Control information that is pre-configured in the UPF and the corresponding header handling behaviour has to be agreed as part of the SLA. In a deployment, for simplicity, an operator can choose to set this reference to the same value as the Header Handling Detection Reference.

# * * * * next change (all new) * * * *

#### 5.8.2.x 	Handling of Payload Headers

Support of Handling of Payload Headers is an optional UPF feature.

The SMF receives Header Handling Control information in a PCC rule (as described in clause 6.3.1 of TS 23.503 [45]), it forwards the Header Handling Control information to the UPF via N4 interactions. The SMF adds the Header Handling Control information relevant for the UL direction into the FAR that corresponds to the UL PDR of that PCC rule and the Header Handling Control information relevant for the DL direction into the FAR that corresponds to the DL PDR of that PCC rule.

If the Header Value parameter contains a reference to information to be added by SMF, the SMF shall overwrite the Header Value parameter with the indicated information of the corresponding PDU Session.

If SMF determines based on local configuration that it shall receive notifications related to the Handling of Payload Headers, it can modify the Header Handling Reporting of the header handling actions of interest by setting it to “yes” and by adding itself as an endpoint. In addition, the SMF shall add its Notification Target Address and a Notification Correlation ID to the Header Handling Reporting Endpoints.

When UPF receives a Header Handling Control Rule from SMF over N4 (see 5.8.5.6), it contains a Header Detection Reference (see clause 5.6.x.2) which points to a predefined configuration in UPF which includes the details of the protocol and possibly the message for which a header handling action shall be performed (e.g. HTTP, HTTP message). The interpretation of the Header Detection Reference is part of the SLA and thus, implementation specific.

The Header Handling Control Rule contains other explicit Header Handling Control information to be used for handling the traffic that matches the Header Detection Reference, see clause 5.6.x.2.

When the Header Handling Control Rule includes Header Handling Reporting information, UPF may send notifications related to the Handling of Payload Headers to SMF and/or NEF/AF.

# * * * * next change * * * *

#### 5.8.5.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.5.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. | NOTE 9. |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included and the action can also indicate that a notification of the first buffered and/or a notification of first discarded packet is requested (see clause 5.8.3.2). For drop action, a notification of the discarded packet may be requested (see clause 5.8.3.2). |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). | NOTE 8. |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN), or to 5G VN internal (i.e. local switch). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, local access to a DN represented by a DNAI) (NOTE 8). Any extension header stored for this packet shall be added. The time stamps should be added in the GTP-U header if QoS Monitoring for packet delay is enabled for the traffic corresponding to the PDR(s). |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. | NOTE 8. |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | The Forwarding policy refers to a preconfigured forwarding behaviour in UPF, which may be related to: - N6-LAN steering to steer the subscriber's traffic to the appropriate N6 Service Functions deployed by the operator; - local N6 steering to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7; - a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Metadata (NOTE 10) | Metadata the UPF needs to add to traffic sent over a SFC. | The metadata information is associated with a TSP ID related to N6-LAN steering. |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| Header Handling Control Rule (see clause 5.6.x) (NOTE Z) |  |  |
| Header Detection Reference | A reference to a UPF configuration which defines how to detect the protocol or the message in a protocol for which to perform the header handling actions. |  |
| Header Detection Support Information | Any dynamic information provided by the AF which is required for the detection of the headers and cannot be preconfigured for the Header handling Detection Reference. |  |
| Header Handling Reporting Endpoints | List of notifications endpoints, i.e. per notification endpoint a Notification Target Address and a Notification Correlation ID. |  |
| Header Handling Control Reference | A reference to a Header Handling Action related information pre-configured in the UPF. |  |
| Header Handling Action (NOTE X) | Indicates the header handling action to be performed on a specific header field | Possible values: Detect, Remove, Replace, Insert |
| Header Information (NOTE X) | A reference to a UPF configuration which defines how to identify a specific header field for which to perform the header handling action. |  |
| Header Value (NOTE X) | A string describing the value of the specific header field. | Mandatory for Replace action, optional for Remove, Insert and Detect action |
| Header Handling Condition (NOTE X) | Indicates the condition for performing the header handling action. | Possible values: first match, every match |
| Header Handling Reporting (NOTE X) | Indicates whether reporting is requested for the performed Header Handling Action and the notification endpoint(s). | Possible values: yes/no; if yes, one or more of the notification endpoints |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF "local switch" and N19 forwarding is used for different 5G LAN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF and used to determine the Forwarding Policy included in the FAR according to the description in clause 5.6.7 and clause 6.1.3.14 of TS 23.503 [45] for local N6 steering and in clause 5.6.16 and clause 6.1.3.14 of TS 23.503 [45] for N6-LAN steering. The Forwarding Policy action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for UL/DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 of TS 29.244 [65]. NOTE 7: The use of "5G VN internal" instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session of the same 5G VN group. NOTE 8: When in architectures defined in clause 5.34, a FAR is sent over N16a from SMF to I-SMF, the FAR sent by the SMF may indicate that the I-SMF is to locally determine the value of this attribute in order to build the N4 FAR rule sent to the actual UPF controlled by the I-SMF. This is further defined in clause 5.34.6. NOTE 9: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 10: The use of Metadata is described in clause 5.6.16. How the UPF transforms the Metadata into actual information sent with the traffic (e.g. in the encapsulation header) is based on local policies related with the Forwarding Policy and not specified. NOTE Z: More detailed description of each of the Header Handling Control Rule parameters can be found in 5.6.x.2 which describes the corresponding AF provided parameters in the Header Handling Control information. NOTE X: These parameters are provided together to request a Header Handling Action. Multiple sets of these parameters (and thus multiple header handling actions) can be provided. One or more of these parameters can be provided to overwrite Header Handling Action related information pre-configured in the UPF as per the Header Handling Control Reference. |  |  |

# * * * * next change * * * *

#### 5.8.2.17	Data exposure via Service Based interface

The UPF may expose information by means of UPF Event Exposure service as described in TS 23.502 [3] clause 5.2.26.2, via a service-based interface directly. The NF consumers, which may receive UPF event notifications, are AF/NEF, TSNAF/TSCTSF, SMF and NWDAF/DCCF/MFAF.

When the UPF supports the data exposure via the service based interface, it may register its NF profile to the NRF including the UPF Event Exposure services and the related Event ID(s).

For data collection from UPF (see clause 4.15.4.5 of TS 23.502 [3]), NF consumers can do the subscription to the UPF either directly or indirectly via SMF. An NF consumer may subscribe to the UPF Event Exposure service directly if target of data collection is "any UE", e.g. to collect user data usage information for NWDAF NF Load analytics (see clause 6.5 of TS 23.288 [86] or if target of data collection is a UE IP address,) and if the subscription is not including any of the following parameters: AoI, BSSID/SSID and DNAI. Otherwise the NF consumer shall subscribe indirectly via SMF.

NOTE:	UE IP address is used to discover the PSA UPF serving the particular UE IP address. NF consumer cannot use UE IP address to discover the local PSA UPF(s) for the case when ULCL/BP are used for the PDU Session.

When the UPF changes for the scenario of UPF relocation, if the UPF event subscription for a UE is directly sent to the UPF, the UPF may send a notification with a subscription termination indication and an optional cause code to the UPF event consumer. In the case of subscribing to UPF event via SMF or directly to UPF, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet when the UPF changes (see clause 4.15.4.5 of TS 23.502 [3]).

To alleviate the load of UPF due to frequent event notification for data collection related events, the event subscription may include Reporting suggestion information. The Reporting suggestion information includes Report urgency and Reporting window information. Reporting urgency information represents whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting window is also provided, which defines the last valid reporting time, and UPF shall report the event before that time. If the Reporting suggestion information allows this, the UPF can concatenate several event reports (of the same event) to the same notification endpoint into one notification message.

The UPF may also expose UE information by means of the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in clause 5.2.26.3 of TS 23.502 [3]. An UPF which is deployed with NAPT (Network Address Port Translation) functionality may support to provide the 5GC UE IP address to NEF based on NEF request containing public IP address and port number using the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in clause 4.15.10 of TS 23.502 [3] for AF specific UE ID retrieval.

The UPF event exposure service may allow NF consumer to obtain one NATed UE public IP address and Port number for a particular PDU Session, based on the UE private IP address allocated by 5GC. An UPF which is deployed with NAT functionality may support to provide the UE public IP address and port number to consumer based on Nupf_EventExposure_Subscribe request as defined in clause 5.2.26.2.3 of TS 23.502 [3].For information flow for getting UE public IP address and port number, see clause 6.2.8.2.4 of TS 23.288 [5].

# * * * * next  change * * * *

### 6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition).

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	NW-TT functionality.

-	High latency communication, see clause 5.31.8.

-	ATSSS Steering functionality to steer the MA PDU Session traffic, refer to clause 5.32.6.

NOTE: Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

-	Inter PLMN UP Security (IPUPS) functionality, specified in clause 5.8.2.14.

-	Event exposure, including exposure of network information, i.e. the QoS monitoring information, as specified in clause 5.8.2.18, events as specified in clause 5.2.26.2 of TS 23.502 [3], exposure of data collected for analytics, as specified in clause 5.2.26.2 of TS 23.502 [3] and exposure of the TSC management information as specified in clause 5.8.5.14

-	Exposure of the UE information, e.g. UE IP address translation information as specified in clause 5.2.26.3 of TS 23.502 [3] and clause 4.15.10 of TS 23.502 [3] if Network address translation (i.e. NAT) functionality of the UE IP address is deployed within UPF.

-	Support PDU Set Handling as defined in clause 5.37.5.

-	Handling of Payload Headers as defined in clause 5.8.2.x

# * * * * End of changes * * * *