3GPP TSG-SA2 Meeting #165	S2-2411187

# Hyderabad, India, 14 – 18th  October 2024	  		(revision of S2-24010256, 2410651, 2410960)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5746 | rev | 3 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Usage of UDP-Option carrying XRM metadata over UDP tunnel for end-to-end encrypted XRM flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google, Futurewei, InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA plenary approved conclusions in 23.700-70 clause 8.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new feature that supports UDP-Option carrying media related inforamtion over in-band connection over N6 which is established using connect-UDP between the AS and the PSA UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | conclusions in 23.700-70 clause 8.2 for KI#2 are not supported by normative work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.X.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR should be implemented after CR 5745 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (ALL New Text)

##### 5.37.X.4 Usage of UDP Option transferring Media Related Information over in-band connection

The mechanism described in this clause uses in-band connection established using UDP-Connect between the UPF as HTTP/3 client and the AS as HTTP/3 proxy according to clause 5.37.X.2 to securely transfer Media Related Information.

The Media Related Information is encoded into UDP-Option as defined in [xx3] and transferred through in-band connection, in which the inner UDP datagram contains end-to-end encrypted XRM media between AS and UE, and the outer UDP datagram between AS and UPF contains UDP-Option carrying encrypted Media Related Information.

Editor’s Note:  The format of the Media Related Information carried by the specific UDP-Option and exchanged between the UPF and the AS will be further defined by 3GPP CT working groups.

Editor’s Note: The details for documenting and assigning a UDP option kind value for carrying Media Related Information in IETF are pending collaboration with IETF.

The usage of UDP-Option transferring Media Related Information over in-band connection is supported with the following features:

-	The AF provides the Protocol Description including an indication of UDP-Option for transfering Media Related Information, an indication of support of UDP-Connect, and the corresponding AS address, together with corresponding QoS requirements. This is done using the AF session with QoS procedure (as defined in clause 4.15.6.6 of TS 23.502 [3]) to support the establishment of an in-band connection between the UPF and the AS, and to instruct the PSA UPF that the Media Related Information is carried in UDP-Option in UDP datagram over in-band connection.

-	The PCF generates a PCC rule including corresponding information as defined in clause 6.1.3.27.4 of TS 23.503 [45].

-	The SMF provides N4 rules according to PCC rules to the UPF including the corresponding information.

-	In the uplink direction, PDR (see clause 5.8.5.3) and FAR (see clause 5.8.5.6) rules are used to detect XRM media flow subject to being associated with an in-band connection for UDP-connect based proxying.

-	In the downlink direction, PDR (see clause 5.8.5.3) is used to detect XRM media flow using UDP-Option for transferring Media Related Information of encrypted traffic based on Protocol Description.

-	The UPF supports the HTTP/3 Client functionality only for negotiation of security keys used for encrypting Media Related Informationcarried in the UDP-Option. After PDU Set Identification and marking for PDU Set based QoS handling, the UPF shall remove the UDP-Option carrying Media Related Information before forwarding the identified PDU Sets of the XRM media packets over right PDU Session towards the NG-RAN and the UE.

Editor’s Note: Security keys for UDP-Option can be negotiated using Connect UDP upgrade token in tunnelled mode between the UPF and AS. The details are up to SA3.

END OF CHANGES