3GPP TSG SA WG2 #165	            S2-2410696

14 - 18 October, 2024, Hyderabad, India 	(revision of S2-2410216+S2-2409984)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1479 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3: DC interworking with MTSI UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on KI#3 conclusion in TR 23.700-77, DC interworking with MTSI UE needs to be specified in Rel-19. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Descriptions on different types of DC interworking with MTSI UE are introduced. It includes the transcoding between the data channel media and the MMTeL media, the DCAS triggering the corresponding actions for terminating MTSI UE and the originating MTSI UE. In addition, - Add EN to cover the missing enhancements to services to support the feature. - Correctly describe the role to be performed by the MF. - Complete some of the steps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DC interworking with MTSI UE cannot be supported completely. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.2.2.3, AC.7.9, AC.7.9.0 (new), AC.7.9.1, AC.7.9.2, AC.7.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AC.2.2.3	Data Channel Media Function (MF)

The MF provides the media resource management and forwarding of data channel media traffic. The MF provides the following functionalities:

-	The MF manages the data channel media resources (bootstrap and application data channel resources, if applicable) under the control of the IMS AS.

-	The MF terminates the bootstrap data channel from the UE and forward HTTP traffic between the UE and DCSF via MDC1.

-	The MF may anchor the application data channel in P2P scenarios, if required, and forward application data traffic from/to the UEs.

-	The MF relay traffic on the A2P/P2A application data channels between the UE and the DC Application Server via MDC2.

-	The MF handles transcodingg of data channel media following the media instructions from the DCSF.

# * * * * Second change * * * *

AC.7.9	IMS data channel setup Signalling Procedure for interworking with MTSI UE

Editor’s Note: The impact on services in clause AA.2.4.1 is FFS.

# * * * * Third change (all text is new) * * * *

AC.7.9.0	General

This clause describes the IMS data channel setup signalling procedures to support interworking with MTSI UE.

When the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, a bootstrap data channel is set up between the originating DCMTSI UE and the originating network (i.e. DCSF and MF), and an MMTEL session for the other media (e.g. audio, video, the MMTEL media other than data channel) may be set up with the terminating side based on the SDP response from the terminating MTSI UE. It is described in clause AC.7.9.1.

The originating DCMTSI UE may initiate an application data channel setup which is associated with the MMTEL session between the DCMTSI UE and the MTSI UE (e.g. for real-time screen sharing). The data channel media sent from the originating DCMTSI UE is transcoded to MMTEL media and transcoded to the terminating MTSI UE via the associated RTP stream, and the MMTEL media sent from the terminating MTSI UE is transcoded to data channel media and transferred to the originating DCMTSI UE via the associated application data channel stream. It is described in clause AC.7.9.2.

The DCMTSI UE may initiate application data channel setup with the DCAS for the data channel applications which is configured to support interworking with MTSI UE. The DCAS subscribes the event to the IMS AS to be notified about the application data channel setup for the data channel applications that require interworking with MTSI UE. It is described in clause AC.7.9.3.

The DCAS may initiate data channel between the serving network of the DCMTSI UE and the DCMTSI UE when it requires interworking with MTSI UE.

Editor's note:	The procedure of the last paragraph will be described based on the network initiated data channel session setup procedure.

# * * * * Forth change * * * *

AC.7.9.1	Bootstrap Data Channel Setup Signalling Procedure

Figure AC.7.9.1-1 depicts a signalling flow diagram for establishing a bootstrap data channel when the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE. In Figure AC.7.9.1-1, the procedure starts with the INVITE including SDP offers. The SDP offers for audio and data channel media are the minimum condition to start this procedure, but it is not limited only for that media (e.g. the video offer may be included).

Figure AC.7.9.1-1: Bootstrap Data Channel set up Signalling Procedure for IMS data channel interworking with MTSI UE

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The initial SDP contains SDP offers for the bootstrap data channel with bootstrap DC stream ID.

2.	Data channel management procedure in originating network is executed as per step2~10 in Figure AC.7.1-1.

3-4.	IMS AS sends the INVITE request to remote network with SDP containing data channel media for terminating network and UE #2.

5-7.	UE#2 and terminating network returns bootstrap DC negotiation result to originating network. The port number of bootstrap data channel in the SDP answer is set to zero implying that the terminating UE #2 does not support IMD DC or is not acceping the establishment of bootstrap data channel.

8.	IMS AS requests MF to update the local DC media resources accordingly.

9-11.	When IMS AS sends the bootstrap DC negotiation result to DCSF, since the port of bootstrap data channel for UE #2 is set to zero, the DCSF knows that the terminating network or UE #2 does not support IMS DC or does not want to use IMS data channel in this session.

12-18.	These procedures are same as step14~20 in Figure AC.7.1-1.

19. The bootstrap data channels have been established only between originating MF and UE#1.

20.	The subsequent procedure applies.

# * * * * Fifth change * * * *

AC.7.9.2	Signalling Procedure of Application Data Channel Interworking via MF

Figure AC.7.9.2-1 depicts a signalling flow diagram for establishing the application data channel when the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, providing interworking of application data channel to MTSI UE via MF. This signalling flow takes the data channel application which enables the transcode between the data channel media and the MMTEL media such as audio and video e.g. Real-time Screen Sharing.

Figure AC.7.9.2-1: Application Data Channel set up signalling procedure for IMS data channel interworking with MTSI UE via MF

1-3.	UE#1 initiates a re-INVITE request to establish an application data channel in which the application data channel and data channel application information (for instance, application ID) is included. The IMS AS validates user subscription data to determine whether this request should be notified to the DCSF as stated in Clause AC.7.2.

4.	After receiving the notification, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information as application ID) in the notification and/or DCSF service specific policy.

5-7.	Since the DCSF has known the terminating network or UE #2 does not support or does not want to use IMS data channel in this session, the DCSF determines that interworking of the application data channel media to MTSI UE is needed and possible, based on the application information, i.e. appliation id. The DCSF invokes Nimsas_MediaControl_MediaInstruction to the IMS AS and instructs the IMS AS to create resources and association for the application data channels on originating side and video media on terminating side. The DCSF also includes the instruction of how to transcode data channel media to video media. The IMS AS invokdes Nmf_MediaResourceManagement to the MF to allocate resource and transcode data channel media to video media according to DCSF’s instruction.

NOTE:	The media instruction enables the MF to perfrorm  ttransport protocol translation from DC protocol (UDP/DTLS/SCTP) on one side and video media (RTP/AVPF) on the other.

8-9.	IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success result of operation and also includes negotiated data channel media resource information for MDC1. The DCSF stores the media resource information and responds to the request received in step 3.

10-11.	After the media resource reservation, the IMS AS sends a re-INVITE request with the SDP offer for anchoring the video media of UE#2 to the MF.

12-14.	After terminating network side negotiation, UE#2 and terminating network returns 200 OK for the re-INVITE.

15-16.	DCSF is notified about the terminating network media negotiation result.

17-21.	The IMS AS sends 200 OK for re-INVITE initiated by UE#1 and UE#1 returns ACK.

22.	UE#1 initiates SCTP association and DTLS connection establishment procedures and establishes the application data channels with the MF.

23-24.	The IMS AS reports media negotiation result to the DCSF, and reports that anchoring video stream is successful. The DCSF sends a Call Control Request to the MMTel AS indicating it to continue the call process.

25.	The IMS AS sends a Media Resource Management Request to the MF to associate application data channel stream between UE#1 and the MF with the allocated video streams between UE#2 and the MF.

The application data channels between the MF and UE#1 are associated with the RTP video streams between the MF and UE#2. The RTP stream of UE#1's (e.g. screenshare sequence) is received by UE#2 from video streams.

# * * * * Sixth change * * * *

AC.7.9.3	Signalling Procedure of Application Data Channel Interworking via DC AS

Figure AC.7.9.3-1 depicts a signalling flow diagram for establishing an application data channel from DCMTSI UE to DC application server, providing interworking of application data channel to MTSI UE via DC application server.

The originating DCMTSI UE sends the data channel application media to the DCAS, which performs the necessary interworking actions on the application media for the terminating MTSI UE.

Figure AC.7.9.3-1: Application Data Channel set up signalling procedure for IMS data channel interworking with MTSI UE via DC application server

The steps in the call flow are as follows:

1. DC AS subscribes via the NEF to be notified when any application DC for an DC application that requires interworking is established.

Editor’s Note: The Figure needs to be updated to properly depict the subscription by the DC AS.

Editor’s Note: The definition of the event is FFS.

2. IMS session and bootstrap data channel have been established as per clause AC.7.1. Selected DC application(s) have been downloaded to UE#1.

The DCSF knows the UE#2 or terminating network does not support IMS DC, or is not willing to use IMS DC  based on procedure in clause AC.7.9.1.

3-5.	The UE#1 initiates a P2P application DC. Steps 1-2 of clause AC.7.2.1 applies.

6. After receiving the session event notification from the IMS AS, the DCSF determines that IMS DC interworking for the requested DC application is to be performed based on the application information, i.e. appliation id. Since the remote network or UE#2 does not support IMS DC or is not willing to use IMS DC, the DCSF changes P2P application DC to P2A application DC.

7-16.	The DCSF instructs the MF via the IMS AS to establish the media resource of the P2A application DC and the DC connection is established between UE and the DC AS. Steps 5-14 of clause AC.7.2.2 applies.

17. IMS AS sends the event notification to the DC AS via the NEF to indicate the DC AS that the interworking should be performed for UE#2. The DC AS receives this event notification before step 11, so it can take the appropriate actions in step 11.

18-19.	IMS AS sends the re-INVITE to remote network side and UE#2, which does not include bootstrap data channel information and application data channel media components in the SDP.

20-29.	Steps 17-26 of clause AC.7.2.2 applies.

30.	UE#1 uses the established application data channel to provide DC application specific information to the DC AS, e.g. sends the file (e.g. a photo) which will be sent to UE#2 to DC AS.

31.	DC AS performs interworking actions based on the notification received in step 17 and the data received in step 30.

NOTE:	How the DC AS performs interworking actions to provide DC application specific information to the MTSI UE is out of scope of this specification. The DC AS can for example generate a hyperlink associated with the file received in step 30 and sends the hyperlink to UE#2 through a SMS, where the user of UE#2 can download the file using the hyperlink if he/she wants.

# * * * * End of changes * * * *