3GPP TSG-SA WG2 Meeting #166	S2-2411550

# Orlando, USA, 18 – 22 November, 2024 		(revision of S2-2409684)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1201 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF collects data from LCS to train ML model for AI positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R19 FS_AIML_CN has concluded KI#1 for LCS supporting Direct AI/ML based positioning in TR 23.700-84. As for data collection, when the NWDAF receives the AIML positioning model training request from the consumer (e.g., LMF), the NDWAF can trigger the LMF to collect data in order to train an ML model for AI positioning. The interaction between the LMF and NWDAF needs to be specified. During SA#165, S2-2410978 capturing the procedure of NWDAF collecting AI positioning training data from LMF has been approved. Thus, the corresponding interaction between NWDAF and LMF is proposed in TS 23.288 clause 6.2.12. Besides, to support the storage/revomal of large quantities of location data, the LMF can storage the location data in the ADRF. Thus, the corresponding service consumer is also added in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NWDAF collects positioning data directly from LMF in order to train ML model for AI positioning. - LMF can be the consumer of ADRF Data Management service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF triggers the data collection for training AIML positioning model is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.1, 6.2.12.1, 6.2.12.2, 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: change into NWDAF collecting input data from LMF directly. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2B.1	General

Collected data and analytics may be stored in ADRF, using procedure as specified in clause 6.2B.2 and clause 6.2B.3. Collected data and analytics may be deleted from ADRF, using procedure as specified in clause 6.2B.4.

Collected data used for ML model training for LMF-based AI/ML positioning may be stored by LMF in ADRF and may be deleted by LMF from ADRF, as specified in TS 23.273 [39].

ML Model may be stored in ADRF, using procedure as specified in clause 6.2B.5. ML Model may be deleted from ADRF, using procedure as specified in clause 6.2B.6. ML Model(s) may be retrieved from ADRF, using procedure as specified in clause 6.2B.7.

# * * * * Next change * * * *

6.2.12	Data Collection using LCS

6.2.12.1	General

The NWDAF may collect location information for a target UE or a group of target UEs using LCS. The collected location related information can include:

‐	Location estimate of the UE in geographical coordinates and/or local coordinates expressed as a shape as defined in TS 23.032 [34] or local coordinate reference system;

-	Time stamp of location estimate;

-	Velocity of the UE as defined in TS 23.032 [34];

-	Information about the positioning method used to obtain the location estimate of the UE;

-	Indication of area event, when UE enters, is within or leaves the Geographical area;

-	Indication of motion event when UE moves by more than some predefined straight line distance from a previous location.

NOTE:	The location information that can be retrieved is defined within the location service response in clause 5.5 of TS 23.273 [39].

NWDAF shall use Ngmlc service as defined in TS 23.273 [39] to collect the location information using LCS. Only Mobile Terminated Location Request (MT-LR) is supported, including both Immediate Location Request and Deferred Location Request.

NWDAF may determine to query LCS system instead of AMF to obtain UE's location information based on the following attributes as received from NWDAF consumer:

-	Analytics ID (e.g. UE Mobility, QoS Sustainability, Relative Proximity, Movement Behaviour);

-	Preferred granularity of location information.

The NWDAF may collect input data from LMF to train an ML model for LMF-based AI/ML positioning as defined in TS 23.273 [39].

# * * * * Next change * * * *

6.2.12.2	Procedure for data collection using LCS

The interactions between NWDAF and LCS for data collection are illustrated in Figure 6.2.12.2-1. The data collected depends on the use cases. This figure is an abstraction of how NWDAF collects location information or input data for training LMF-based AI/ML positioning model using LCS. The actual procedures that NWDAF may use are as follows:

-	For a target UE, both 5GC-MT-LR procedure for the commercial location service as specified in clause 6.1.2 and deferred 5GC-MT-LR procedure as specified in clause 6.3 of TS 23.273 [39] can be utilized;

-	For a group of target UEs, bulk operation of LCS Service Request Targeting to Multiple UEs as specified in clause 6.8 of TS 23.273 [39] can be utilized;

-	For the area of interest, procedure of data collection for training LMF-based AI/ML positioning model as specified in clause 6.x of TS 23.273 [39] can be utilized.

Figure 6.2.12.2-1: Data collection using LCS

1.	NWDAF requests the location information from GMLC about a target UE (that may be identified by a SUPI) or a group of target UEs (identified by a group ID) using Ngmlc_Location_ProvideLocation service.

The NWDAF requests the input data for training LMF-based AI/ML positioning model from LMF about any UEs within an area of interest using Nlmf_DataExposure_Subscribe service.

2a.	GMLC interacts within LCS, i.e. with AMF/LMF as described in TS 23.273 [39], to obtain the UE's location information. If privacy verification is required, GLMC will interact with UE via AMF before sending the location information to NWDAF.

2b.	LMF obtains input data for training LMF-based AI/ML positioning model as described in clause 6.x of TS 23.273 [39].

3a.1	If it is Immediate Location Request, GMLC sends the location service response including the location information for the target UE (or a group of target UEs) within a short time period as specified in clause 4.1a.4 of TS 23.273 [39] to the NWDAF.

3a.2	If it is Deferred Location Request, GMLC sends the location service response including the indication of event occurrence and location information if requested for the target UE (or group of target UEs) at some future time (or times) as specified in clause 4.1a.5 of TS 23.273 [39] to the NWDAF.

3b	LMF invokes the Nlmf_DataExposure_Notify service operation including the requested input data to the NWDAF.

# * * * * Next change * * * *

10	ADRF Services

10.1	General

Table 10.1-1 shows the ADRF services and ADRF service operations.

ADRF service operations may be used to store data or analytics in the ADRF, retrieve data or analytics from an ADRF, or delete data or analytics from an ADRF.

ADRF service operations may also be used to store ML Model(s) or ML Model address(es) in the ADRF, retrieve ML Model(s) from an ADRF or delete ML Model(s) from an ADRF.

Table 10.1-1: NF services provided by ADRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadrf_DataManagement | StorageRequest | Request / Response | DCCF, NWDAF, MFAF, LMF |
|  | StorageSubscriptionRequest | Request / Response | DCCF, NWDAF, LMF |
|  | StorageSubscriptionRemoval | Request / Response | DCCF, NWDAF, LMF |
|  | RetrievalRequest | Request / Response | DCCF, NWDAF, LMF |
|  | RetrievalSubscribe | Subscribe / Notify | DCCF, NWDAF, LMF |
|  | RetrievalUnsubscribe |  | DCCF, NWDAF, LMF |
|  | RetrievalNotify |  | DCCF, NWDAF, LMF |
|  | Delete | Request / Response | DCCF, NWDAF, LMF |
| Nadrf_MLModelManagement | StorageRequest | Request / Response | NWDAF |
|  | RetrievalRequest | Request / Response | NWDAF |
|  | Delete | Request / Response | NWDAF |

# * * * * End of changes * * * *