3GPP TSG-WG SA2 Meeting #166	S2-2411877

Orlando, US, November 18 – 22, 2024	(revision of)

Source:	Samsung

Title:	Conclusions on AIoT KI#3 Service Exposure

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusions for KI#3 on Exposure are proposed.

1. Introduction/Discussion

This contribution proposes to update the conclusions for KI#3.

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-13.

FIRST CHANGE

8.3	Conclusion on Key Issue #3

8.3.1	General

The clause concludes the following aspects:

-	Study how to support information transfer for Ambient IoT services and related system functionality, including the information transfer for an Ambient IoT device and for a group of Ambient IoT Devices.

NOTE:	The above aspect includes studying whether there is a need to support session based transfer between Ambient IoT Device and the network considering the device types and capabilities.

-	Study which of the enabled Ambient IoT services are exposed to AF and how, e.g. for the case AF requests Ambient IoT service for an Ambient IoT Device and for a group of Ambient IoT Devices.

Editor's note:	Additional conclusions are FFS.

8.3.2	AIoT services supported by the 5GC

The following Ambient IoT services are agreed to be supported by the 5GC, which apply to both topology 1 and topology 2:

-	Inventory: Request to perform an inventory operation.

-	Read: Request to read information from an AIoT Device.

-	Write: Request to write information to an AIoT Device.

-	Disable: Request that an AIoT Device has its capability to transmit RF permanently or temporarily disabled.

-	Enable: request to enable a temporarily disabled AIoT Device.

Editor's note:	It is FFS whether and how to support enabling temporarily disabled AIoT devices.

8.3.3	NEF exposure

NEF supports to expose Ambient IoT service towards the AF.

-	Monitoring Coverage of Interesting Area API is supported as an Ambient IoT Service.

END OF CHANGES