3GPP TSG-SA WG2 Meeting #166	S2-2412345

18 - 22 November 2024, Orlando, USA	(revision of S2-2407470)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1407 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Local BDC setup on terminating network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current spec, any kind of DC session is able to set up when originating side and terminating side are DC capable and have subscribed to DC service. According to LS from CT1 (S2-2407421), there is some needs to enable the originating side to initiate a bootstrap DC session between the terminating network and the terminating UE, just with the purpose of to download a DC application to the terminating UE. In this case, the originating UE is not necessary to subscribed to DC service since only the terminating side will use the DC service. Therefore, the IMS DC service subscription and capability negotiation need to be further clarified for local bootstrap DC setup in terminating network, and the procedure for local bootstrap DC setup in terminating network need to be further described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - It is clarified that the IMS AS and DCSF (not S-CSCF) use the DC subscription data in HSS to authorize subscribers to use the DC service. This aligns with IMS DC capability negotiation (clause AC.7.0) which says S-CSCF indicates the IMS network’s support of data channel capability not about the user subscription and step 2 of Bootstrap DC setup procedure (clause AC.7.1) which says IMS AS validates user subscription. - It is further described that the UE and the network may be configured to allow to initiate an IMS DC only in terminating network regardless originating UE being subscribed IMS DC service or originating network supporting data channel capability. - It is introduced how to initiate the local bootstrap DC setup for terminating network based on the existing procedure with some additional differences and clarifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The local bootstrap DC session only between the terminating network and the terminating UE is not able to initiate by the originating side, when the originating UE or the originating network is not DC capable or has no DC service subscription. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.3, AC.4, AC.7.0, AC.7.1, AC.7.1a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

AC.3	IMS Data Channel Service Subscription

Service subscription for IMS Data Channel shall be an extension to the MMTEL related service profile in HSS. The Data Channel subscription data shall be used by the IMS AS and DCSF during IMS registration, session initiation or update to authorize subscribers to use the DC service.

IMS AS Service specific data is enhanced with DC specific service data, optionally stored in HSS (e.g. as repository data) and retrieved by IMS AS using N71/Sh interface.

DCSF service specific data used by DCSF for data channel management, may be stored (e.g. as repository data) and retrieved from HSS using the N72/Sc interface. DCSF service specific data are out of scope of 3GPP.

# * * * * Second change * * * *

AC.4	IMS DC Channel Setup

The following principles apply when an IMS Data Channel is established: -

-	UE can establish an IMS Data Channel simultaneously while establishing an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to an IMS Data Channel session.

NOTE:	An IMS Data Channel established simultaneously with an IMS audio/video session can have impact to the IMS session setup time.

-	The UE may be configured by the HPLMN either via Device Management or in the UICC whether and when to initiate a DC establishment request. If the UE is not configured by the HPLMN, it is left for UE implementation whether and when to initiate the DC establishment request. The UE shall not initiate a DC establishment request if the network has not indicated support for IMS DC service during IMS registration or if the UE is configured to not establish an IMS Data Channel.

-	The IMS AS determines whether the UE has subscribed to IMS Data Channel service by retrieving the subscription data from HSS as described in clause AC.3.

-	If a UE that has not subscribed to IMS Data Channel establishes an IMS Data Channel simultaneously with an IMS audio/video IMS session and if the UE or the network is not allowed to initiate a local DC for terminating network, the IMS AS shall discard the Data Channel request and proceed with the audio/video IMS session establishment.

-	The UE and the network may be configured to allow to initiate an IMS DC only in terminating network (e.g. the originating UE initiates an IMS DC between the terminating network and the terminating UE) regardless originating UE being subscribed to IMS DC service or originating network supporting data channel capability. In this case, the IMS AS determines whether the UE and the network are allowed to initiate a local DC for terminating network based on the subscription data of the UE and the local policy in the IMS AS as described in clause AC.7.1a.

-	The DCSF provides stream ID and a corresponding URL to the MF or MRF via the IMS AS during data channel media resource reservation for the bootstrap data channel that enables downloading a subscriber specific graphical user interface to the UE via the bootstrap data channel. The stream ID is optional. Once the MF or MRF, acting as HTTP proxy, receive a HTTP GET Request containing the root ("/") URL through the bootstrap data channel, the MF or MRF replaces the root ("/") URL in the HTTP GET Request with the subscriber specific URL corresponding to the stream ID (if available) of the bootstrap data channel and forward the request to the DCSF for downloading the subscriber specific graphical user interface to the UE. The MF retrieves the stream ID of the bootstrap data channel from the transport layer of the data channel connection. The UE shall maintain the HTTP session state when interacting with the MF.

# * * * * Third change * * * *

AC.7.0	IMS DC capability negotiation

The IMS network and the UE need to mutually negotiate the capability of supporting IMS data channel when a UE supporting IMS data channel registers to IMS network.

IMS data channel capability negotiation includes two aspects:

-	The network discovers the data channel capability of the UE.

-	The UE discovers the data channel capability of the network.

When the UE supporting IMS data channel registers on the IMS network and the UE is allowed to use IMS data channel as specified in clause AC.4, it includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial REGISTER request and any subsequent REGISTER request to allow the home IMS network to discover its IMS data channel capability.

If the IMS network supports IMS data channel, the S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and any subsequent REGISTER request as specified in clause 9.2 of TS 24.186 [105], which is used by the UE to discover the IMS data channel capability of its home IMS network.

The UE and the network may be configured to allow to initiate an IMS DC only in terminating network (e.g. the originating UE initiates an IMS DC between the terminating network and the terminating UE) regardless originating UE being subscribed to IMS DC service or originating network supporting data channel capability. In this case, the S-CSCF determines whether the UE and the network are allowed to initiate a local DC for terminating network based on the subscription data of the UE and the local policy in the S-CSCF as described in clause AC.7.1a.

The UE may receive a Feature-Caps header field indicating network’s data channel capability in the 200 OK response to a subsequent REGISTER request when the network starts supporting IMS data channel after successful initial registration of the UE in case that the UE has indicated its data channel capability in previous REGISTER request(s).

After receiving the Feature-Caps header field indicating network’s data channel capability in the 200 OK response, when the UE supporting IMS data channel initiates an IMS session, and if the UE is allowed to use IMS data channel as specified in clause AC.4, the UE includes the media feature tag as specified in TS 26.114 [76] in the Contact header field of the initial INVITE or any re-INVITE request message, regardless of data channel media being part of the SDP or not.

The UE shall not include the media feature tag as specified in TS 26.114 [76] in the Contact header field and data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE request message, if the S-CSCF has not included the data channel capability indication in the Feature-Caps header field and has not indicated that it is allowed to initiate an local DC for terminating network without the network’s data channel capability or IMS DC service subscription in the 200 OK response either to the initial REGISTER or a subsequent REGISTER request message.

If the UE is not configured whether to use IMS data channel, it is up to UE implementation whether or not to include the media feature tag in the Contact header field.

# * * * * Fourth change * * * *

AC.7.1	Bootstrap Data Channel Setup Signalling procedure

Figure AC.7.1-1depicts a signalling flow diagram for establishing a bootstrap data channel in a person-to-person use case. The MF anchors the bootstrap data channel, and the originating network is offering a bootstrap data channel to the remote peer as well for application download.

In the call flow the two UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS data channel session.

NOTE 1:	Some SIP signalling not relevant for the procedure, is not shown in the call flow below.

Figure AC.7.1-1: Bootstrap Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The initial SDP contains offers for the bootstrap data channel establishment request with bootstrap DC stream ID. In this example procedure, the SDP contains both bootstrap data channel offers for originating side and terminating side.

NOTE 2:	This SIP INVITE can also be a SIP re-INVITE performed after the initial IMS audio session is setup. The SIP re-INVITE can be initiated either from UE#1 or UE#2.

2.	IMS AS validates user subscription data to determine whether the data channel call request should be notified to DCSF.

If the IMS AS determined, based on the user profile, that data channel call request needs to be notified to DCSF, the IMS AS selects a DCSF for this user based on local configuration or discovery and selection of a DCSF instance via NRF.

If the IMS AS determined, based on the user profile, that the data channel call request need not to be notified to DCSF, or DCSF decides that DC request is not allowed, the IMS AS proceeds with normal IMS procedures to setup the MMTel session without performing Data Channel bootstrapping, by deleting DC related media information and sending the updated SIP INVITE to the originating S-CSCF. If the reason for not to notify the data channel call request is because the user has not subscribed to DC service, then the IMS AS informs the reason to the originating UE.

3.	IMS AS notifies the DCSF of the DC call event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, Session Case, Event initiator, Media InfoList, DC Stream ID) request to the DCSF.

4.	After receiving the DC control request, the DCSF determines the policy about how to process the bootstrap data channel establishment request based on the related parameters in the Data Channel control request (e.g. CallingID, CalledID, DC Stream ID) and/or DCSF service specific policy.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered local bootstrap media, DCSF, based on its policies reserves originating side MDC1 media information, as well as the terminating side MDC1 remote bootstrap media (targeting remote UE), which are used to receive UE request for application downloading from MF or MRF.

6.	DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) operation based on its policies instructing the IMS AS how to set up bootstrap data channel with MF both for originating and terminating side. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

In this scenario, the DCSF instructs the IMS AS to terminate bootstrap data channel establishment request on originating MF, and initiate remote bootstrap data channel establishment request(targeting remote UE) as well as forwarding remote bootstrap data channel establishment request of served user (targeting remote DCSF) towards terminating network.

7.	The IMS AS selects a MF based on local configuration or discovers and selects a MF instance or MRF supporting DC media function via NRF.

8.	IMS AS invokes Nmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct MF to allocate required data channel media resources. IMS AS request creation of two different Media Terminations, one representing the local bootstrap media to be terminated and the other representing the remote bootstrap media to be offered to remote UE. Each Media Termination includes information required to allocate resources in both Mb and the MDC1 interfaces. The MF responds with the negotiated data channel media resource information to IMS AS. If MRF is used, IMS AS uses Mr'/Cr to the MRF to reserve data channel media resources.

NOTE 3:	The MF media resource allocation in step 8 could be done with one or multiple service invocations.

9.	IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success result of operation and also includes negotiated data channel media resource information for MDC1.

10.	The DCSF stores the media resource information and responds to the Notify Request received in step 3.

11-13.	IMS AS sends the INVITE which includes the updated SDP offer adding media information of MF or MRF via the originating S-CSCF to remote network side and UE#2. In this scenario, the SDP offer for bootstrap data channel to UE#2 is included.

14.	UE#2 and terminating network returns an 18X response with the SDP answer to bootstrap data channel to originating network. According to the received SDP answer, IMS AS may instruct MF or MRF to update data channel media resource information for UE#2.

15-16:	UE#2 and terminating network returns a 200 OK response.

17.	The IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, Session ID, Media Info List) to DCSF.

18.	The DCSF responds to the Nimsas notification request.

19.	200 OK forwarded to UE#1 which indicates the bootstrap data channel has been established.

20.	The originating network P-CSCF executes QoS procedure for bootstrap data channel as specified in TS 23.203 [54] and TS 23.503 [95].

21-24:	The bootstrap data channels have been established between originating MF or MRF and UE#1/UE#2. The UEs send application request messages to MF or MRF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF or MRF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 and UE#2 based on their data channel capabilities and their choices through MF or MRF.

The bootstrap data channels have also been established between terminating MF or MRF and UE#1/UE#2. The data channel application is requested and downloaded to UE#1 and UE#2 from terminating DCSF

Steps 20-24 may be executed after step 14, if the SDP answer in 200 OK to the PRACK and UPDATE messages contain the information required to establish bootstrap data channels.

NOTE 4:	IMS-AGW needs to allow the establishment of bootstrap data channels based on the information in the 200 OK to PRACK and UPDATE messages.

25.	Subsequent procedures continue.

NOTE 5:	In the DC application list, the DCSF may provide DC applications supported by both UEs, or only supported by the UE who sends the application request message. The details of how to provide the application list to the UE and how to use it by the UE are not defined in this specification.

# * * * * Fifth change * * * *

AC.7.1a	Local Bootstrap DC Setup for terminating network

The UE and the network may be configured to allow to initiate an IMS DC only in terminating network (e.g. the originating UE initiates an IMS DC between the terminating network and the terminating UE) regardless originating UE being subscribed to IMS DC service or originating network supporting data channel capability.

The local Bootstrap DC for terminating network is initiated after IMS DC capability negotiation based on clause AC.7.0 with the following differences and clarifications:

-	The S-CSCF checks the subscription data of the UE and the operator policy, and determines whether the UE is allowed to initiate local DC for terminating network with or without the network’s data channel capability and IMS DC service subscription. When it is allowed, then the S-CSCF informs to the UE in the 200 OK response to the initial and any subsequent REGISTER request.

-	The UE may include the media feature tag as specified in TS 26.114 [76] in the Contact header field and data channel media description in the SDP offer of the initial INVITE request or any subsequent re-INVITE request message in following cases:

-	If the S-CSCF has not included the data channel capability indication in the Feature-Caps header field in the 200 OK response, but indicated that the UE is allowed to initiate local DC for terminating network without the network’s data channel capability or IMS DC service subscription; or

-	If the S-CSCF has included the data channel capability indication in the Feature-Caps header field in the 200 OK response, and indicated that the UE is allowed to initiate local DC for terminating network IMS DC service subscription.

-	In above cases, The UE shall include the data channel media description only for the local DC in the terminating network (i.e. DC stream ID set to 100 or 110).

The local Bootstrap DC for terminating network is initiated based on Figure AC.7.1-1 with the following differences and clarifications:

-	In step 2: If the IMS AS determined, based on the user profile and the local poliy in IMS AS, that the data channel call request for the local DC for terminating network is allowed for this UE, then the IMS AS proceeds step 3 only when DC Stream ID is set to 100 or 110 in DC media description sent by the originating UE, and the IMS AS informs whether the user has subscribed to DC service or not to the originating UE in step 14 or step 19.

-	In step 14, and step 19: When the UE receives SDP answers only for the media other than data channel or the UE receives SDP answers for data channel only for the DC stream ID with 100 or 110, while it is indicated that the user has not subscribed to DC service, then the UE notices that the SDP offer for data channel or the SDP offer for data channel with DC stream ID other than 100 and 110 was not rejected by the terminating network or the terminating UE.

-	In step 21-24: In case of establishment only for local bootstrap data channel for terminating network, then only step 24 is executed and step 21, 22, and 23 are not executed.

# * * * * End of changes * * * *