3GPP TSG-SA2 Meeting #166	S2-2412914

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024				   (revision of S2-2412692)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1483 | rev | 8 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Address ENs in IMS AS Session Management Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Qualcomm, Huawei, NTT DOCOMO, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The paper is proposed to address the following ENs for Nimsas_ImsSessionManagement Service in clause AA.2.4.4 of TS 23.228: Editor's note: How to support MDC1 and MDC2 media information is FFS. Editor's note: The parameter design is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Address the ENs for Nimsas_ImsSessionManagement Service in clause AA.2.4.4 of TS 23.228. Rev-3: Add Nimsas_ImsSessionManagement_Notify service operation. Correct the conditions to include MDC2 media endpoint address into Media information set. Rv-4: remove subscribed event IDs from the Create and Update requests. List the supported Event IDs for the notify request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs for Nimsas_ImsSessionManagement Service in clause AA.2.4.4 exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.1, AA.2.4.4.1, AA.2.4.4.2, AA.2.4.4.3, AA.2.4.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |
| Nimsas_ImsSessionManagement | Create | Request/Response | NEF, Trusted AF |
|  | Update | Request/Response | NEF, Trusted AF |
|  | Delete | Request/Response | NEF, Trusted AF |
|  | Notify | Subscribe/Notify | NEF, Trusted AF |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

NEXT CHANGE

### AA.2.4.4	Nimsas_ImsSessionManagement Service

#### AA.2.4.4.1	General

Service description: This service enables the consumer to request IMS AS to create and release an IMS session and modify the media in a specific IMS session.

NOTE:	In this release only session management of IMS data channel is specified.

Editor’s Note: Alignment of services against completed procedures for IMS AS session update and deletion is FFS.

#### AA.2.4.4.2	Nimsas_ImsSessionManagement_Create service operation

Service operation name: Nimsas_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with standalone data channel media.

Inputs, Required: Calling ID, Called ID, Media Information set.

Calling ID: It is the public identity of calling IMS Subscriber.

Called ID: It is the public identity of called IMS Subscriber.

Media Information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Media Type: It indicates the media type used in the created session, e.g., Data Channel.

-	Media Parameter: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it indicates the Data Channel Type (ADC or BDC):

-	When ADC is selected, it indicates the ADC type (P2A, P2P, P2A2P), Application Binding Information, and if P2A ADC is selected, the party which this request is targeted

-	If P2A or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When BDC is selected, it indicates the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: . Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.x-1.

Outputs, Required: Result indication .

Outputs, Optional: None.

#### AA.2.4.4.3	Nimsas_ImsSessionManagement_Update service operation

Service operation name: Nimsas_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: , Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the operated media is Data Channel.

-	Media Parameter: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC):

-	When ADC is selected, it indicates the ADC type (P2A, P2P, P2A2P), Application Binding Information, and if P2A ADC is selected, the party which this request is targeted

-	If P2A or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When BDC is selected, it indicates the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: Calling ID, Called ID, Notification Target Address and Notification Correlation ID.

Calling ID: It is the public identity of calling IMS Subscriber.

Called ID: It is the public identity of called IMS Subscriber.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.x-1.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### AA.2.4.4.4	Nimsas_ImsSessionManagement_Delete service operation

Service operation name: Nimsas_ImsSessionManagement_Delete

Description: This service operation enables the consumer NF to release the specific IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### AA.2.4.4.x	Nimsas_ImsSessionManagement_Notify service operation

Service operation name: Nimsas_ImsSessionManagement_Notify

Description: Provides the subscribed event information to the NF Consumer.

Inputs, Required: Session ID, Event ID, Notification Correlation ID.

Inputs, Optional: Media resource information

Media resource information set includes entries corresponding to each media. Each entry in the set includes the Media correlation ID and DC Media Specification. The DC Media Specification shall exist only with media type "DC" and contains additional media resource information, e.g. remote media address for MDC2 interface.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported Event IDs and related parameters.

Table AA.2.4.4.x-1: List of events and related optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentSuccessEvent | Calling or Called party ID |
| SessionEstablishmentFailureEvent | Calling or Called party ID, reason of the failure |
| MediaChangeSuccessEvent | Calling or Called party ID |
| MediaChangeFailureEvent | Calling or Called party ID, reason of the failure |
| SessionTerminationEvent | Calling or Called party ID |

END OF CHANGES