3GPP TSG-SA2 Meeting #166	S2-2411370

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

Source:	Nokia

Title:	KI#2: Proposals to address open issues for NWDAF discovery and selection for VFL

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This contribution discusses open issues for NWDAF discovery and selection for VFL as identified by ENs. Related Changes are in S2-2411383.

# Discussion

The following ENs need to be resolved:

Editor's note:	Whether additional VFL capability information needs to be registered in NRF is required or new IEs are needed in the VFL capability information, is FFS.

Editor's note:	Details of ML Model handling, supported features, Feature alignment and Sample alignment are FFS.

Issue 1: Security Measures for VFL Intermediate results against Label Inference and client Local Data Attacks

In Vertical Federated Learning (VFL), the risk of label inference attacks and local data exposure presents significant privacy and security challenges. Since VFL involves sharing gradients or intermediate results between NWDAFs and AFs, there is a potential for adversaries to infer sensitive label information or extract insights from local datasets, compromising user privacy.

Observation: To mitigate these risks, it may be necessary to consider standard security mechanisms as part of the VFL framework in 3GPP, specifically addressing attacks that target data and label inference

Proposal 01:

- SA3’s Role in Security Standardization: We propose that SA3 explore security guidelines for VFL that address label inference and data leakage risks.

- Vendor Flexibility for Model Definitions with Standardized Security Constraints: While model architectures, information exchange structure and features can remain vendor-specific to allow flexibility, SA3 could specify minimum security standards for protecting local data and labels.

Issue 2: Possible standardisation of VFL features

The support of a feature requires the ability to collect related input data, to handle related models, to receive intermediate results from the server, and to compute intermediate results/gradients for the feature and provide them to the server. The intermediate results/gradients exchanged between the server and the clients are strongly related to the models used. SA2 so far assumes that the content of models is not standardized but rather vendor specific.

Observation 3: A full standardisation of features would likely require a standardisation of related models.

For each analytics ID, several features may be used if VFL is applied and the effort to define such features would also become prohibitive, considering the number of Application IDs that have been standardized.

Proposal 2: VFL features are not standardized but assumed to be defined by vendors.

Issue 3: Discovery and selection of NF clients supporting different features

The following options have been discussed at the last SA2 meeting:

Issue 3, Option 1: Express feature information in NF profile

Issue 3, Option 2: Use a dynamic feature negotiation

The support of features requires ability to collect related input data, to handle related models, to receive intermediate results from the server, and to compute intermediate results/gradients for the feature and provide them to the server. It can therefore be assumed that in many implementations specific code would be required to support a feature and the capability of an VFL client to support a feature would be relatively static.

Observation 4: The capability of an VFL client to support a feature is in many cases relatively static.

For such features, an omission of feature information in the NRF and a subsequent dynamic feature negotiation (option 2) could lead to the selection of an excessive number of candidate NFs (with the need to transfer a high number of extensive NF profiles to the VFL server)

Observation 5: Issue 3. Option 2, may lead to significantly higher signalling load than Issue 3, Option 1.

On the other hand, feature information may be deemed sensitive and protection may be deemed desirable. Both Option 1 and Option 2 involve an exchange of feature information between NFs. In both options feature information would be vendor specific and could be encrypted. However, Option 2 offers in addition the possibility to restrict the feature negotiation to NFs of the same vendor.

Observation 6: Issue 3. Option 2, may offer better privacy for feature information than Issue 3, Option 1.

Proposal 3: Standardize support both for Issue 3, Option 1 (Express feature information in NF profile) and Issue 3, Option 2 (Use a dynamic feature negotiation).

Summary of Proposals

Proposal 2: VFL features are not standardized but assumed to be defined by vendors.

Proposal 3: Standardize support both for Issue 3, Option 1 (Express feature information in NF profile) and Issue 3, Option 2 (Use a dynamic feature negotiation).