3GPP TSG-WG SA2 Meeting #166	S2-2412654

# Nov. 18 – 22, 2024, Orlando, USA								(Revision of S2-2412378, s2-2411412)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5632 | rev | 5 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support PDU Set information identification based on MoQ for encrypted XRM traffic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, Hisilicon, Interdigital, Sony, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PDU Set information can be identified via Metadata based on Media over QUIC (MoQ) for end-to-end encrypted XRM traffic, which should be supported in normative work according to the conclusions for KI#2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PDU Set information identification based on MoQ for encrypted XRM traffic is enhanced for KI#2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mappings of the PDU Set information to the MoQ metadata is not clarified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.1, 5.37.x.2(new), 6.2.3, 6.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The reference [XX2] is documented in S2-2411182 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Clarifications about the agreed CR on MoQ. R5: (1) Modify the UPF selection description to make them more generic: from PDU-session establishment to N4-session procedure; (2) Restructure the paragraph on how ‘The application client may obtain the MoQ relay address via application layer-based schemes’; (3) Add ‘Note X3’ to take care of a possible scenario with one or more MoQ reply’s in a PSA-UPF (4) Regarding defining assumed App-client behhaviors: add NOTES Y1 & X0, along with ‘may’ -> ‘can’. |  |  |  |  |  |  |  |  |

# * * * * Start of changes* * * *

### 5.37.5	PDU Set based Handling

#### 5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as a video frame or video slice. The PDU Set based QoS handling by the NG-RAN is determined by PDU Set QoS Parameters in the QoS profile of the QoS Flow (specified in clause 5.7.7) and PDU Set Information provided by the PSA UPF via N3/N9 interface as described in clause 5.37.5.2. The PDU Set based Handling can be applied for GBR and non-GBR QoS Flows. The AF should provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS Parameters as described in clause 5.7.7

-	Protocol Description: Indicates the transport protocol used by the service data flow (e.g. RTP, SRTP) and information, e.g. the following:

-	RTP [185] or SRTP [186];

-	RTP or SRTP with RTP Header Extensions, including:

-	RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179];

-	Other RTP Header Extensions as defined RFC 8285 [189];

-	RTP or SRTP without RTP Header Extensions, but together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179], and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with other RTP Header Extensions following RFC 8285 [189], and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]).

NOTE 1:	With the Protocol Description options combining SRTP together with RTP Payload Format the UPF can still obtain some of the PDU Set information from the RTP Header (refer to Annex A of TS 26.522 [179]).

- Media over QUIC (MoQ) Transport [XX2] (as further described in clause 5.37.X.2) .

When RTP Header Extensions for PDU Set Marking (as defined in TS 26.522 [179] or other RTP header extensions as defined in RFC 8285 [189] is included, the differentiation between different RTP Header Extension Types should be supported.

When RTP Payload Format is included, the differentiation between different RTP Payload Formats should be supported.

NOTE 3:	Multiplexing of different transport protocols and different media traffic for differentiated PDU Set based handling is not supported in the current Release.

The Protocol Description can be UL only, DL only or UL and DL. The Protocol Description for UL and DL traffic may be different.

AF provided PDU Set QoS Parameters and UL and/or DL Protocol Description may be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45] and the DL Protocol Description may be used for identifying the PDU Set Information and PDU Set Information marking by the PSA UPF.

When the SMF receives the PCC rule, the SMF performs binding of the PCC rule to one QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45]. At least one of the following shall be included in the PCC rule to enable PDU Set based handling: 1) a PSIHI and/or 2) both PSDB and PSER. Based on the PCC rule, the SMF adds the PDU Set QoS Parameters to the QoS Profile of the QoS Flow as described in clause 6.2.2.4 of TS 23.503 [45]. Alternatively, the SMF may be configured to support PDU Set based Handling without receiving PCC rules from a PCF.

For the downlink direction, the PSA UPF identifies PDUs that belong to PDU Sets and marks them accordingly as described in clause 5.37.5.2. If the PSA UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the PSA UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2.

NOTE 3:	If the PSA UPF receives a PDU that does not belong to a PDU Set, then it is assumed that the UPF determines the PDU Set Importance value based on pre-configuration.

For the uplink direction, the UE may identify PDU Sets, and how this is done is left up to UE implementation. The SMF may send the UL Protocol Description associated with the QoS rule to UE.

NOTE 4:	Using the Protocol Description or not is left to UE implementation. The use of Protocol Description does not impact QoS Flow Mapping in the UE.

In this Release, the PDU Set based handling is supported in 5GS for UE registered in 3GPP access for single access PDU Session with IP PDU Session Type.

# * * * * Next change (All new text)* * * *

### 5.37.X.2	Usage of Media Over QUIC in order to Handle end-to-end ciphered XR flows

When a PSA UPF that supports MoQ relay functionality has been selected, encrypted Metadata associated with XR traffic can be transported between the UPF and the AS via Media over QUIC (MoQ) [XX2].

A PSA UPF supporting the MoQ relay functionality can be selected by SMF with the consideration of the DNN and S-NSSAI provided by UE.

NOTE X: The application client uses MoQ Transport protocol and establishes a MoQ connection between the application client and MoQ relay in UPF as defined in MoQ [XX2].

The SMF may get the address of MoQ Relay during N4 session management procedure as defined in the clause 4.4.1 of TS 23.502 [3] or the SMF may get MoQ relay address related to the PSA UPF from the NRF as described in clause 5.2.7.3.2 TS23.502[3].  In the former case, the SMF indicates the PSA UPF to provide MoQ relay address in N4 Session Establishment/Modification Request and the PSA UPF returns MoQ relay address in N4 Session Estabishement/Modification Response,.

NOTE Y1: The application client can obtain the MoQ relay address via application layer-based schemes.

In the case the EASDF based DNS procedure as described in clause 6.2.3.2.2 of TS 23.548 [48] is used, after SMF has received FQDN in DNS message report from EASDF, SMF determines the FQDN is for MoQ and gets the address of MoQ relay and then SMF provides DNS message handling rule with the address of MoQ relay to instruct EASDF to return the address of MoQ Relay in PSA UPF by setting the Forwarding Action as Respond directly to the DNS request according to TS23.548[130].

NOTE X0: it is assumed an application client can send a DNS query to resolve FQDN.

NOTE X1: The FQDN can be the FQDN for MoQ traffic and preconfigured in SMF or it can be the FQDN of the MoQ Relay and preconfigured in SMF and application client. It is assumed there is SLA between operator and application service provider for using MoQ relay functionality provided by operator.

NOTE X2: In some other case, the application client can also obtain the MoQ relay address from the MoQ application server via the query during the MoQ session establishment between the application client and the MoQ application server as in MoQ [XX2], which is out of the scope of 3GPP. The MoQ application server can get the MoQ relay address via local configuration.

NOTE X3: Different MoQ application servers can correspond to different MoQ relays that are deployed on the same PSA UPF

The AF provided Protocol Description indicating Media over QUIC Transport [xx2] can be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45]. Based on the PCC rule, the SMF instructs the PSA UPF to perform PDU Set Information identification. Upon receiving the MoQ subscription message, the MoQ relay can establish a connection to the MoQ application server if there does not exist an established connection yet.

For the downlink service data flow using MoQ Transport, the PSA UPF can identify the PDU Set Information from the MoQ Metadata as defined in clause 7 of MoQ[xx2] that is extracted by the MoQ relay and provide the available PDU Set Information to the RAN in the GTP-U header.

# NOTE X3:	The mappings between the MoQ Metadata and the PDU Set Information can be based on local configuration, or operator-determined implementations.

# * * * * Next change* * * *

### 6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition, IP or MAC filter-based packet detection functionality).

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	NW-TT functionality.

-	High latency communication, see clause 5.31.8.

-	ATSSS Steering functionality to steer the MA PDU Session traffic, refer to clause 5.32.6.

NOTE:	Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

-	Inter PLMN UP Security (IPUPS) functionality, specified in clause 5.8.2.14.

-	Event exposure, including exposure of network information, i.e. the QoS monitoring information, as specified in clause 5.8.2.18, events as specified in clause 5.2.26.2 of TS 23.502 [3], exposure of data collected for analytics, as specified in clause 5.2.26.2 of TS 23.502 [3] and exposure of the TSC management information as specified in clause 5.8.5.14.

-	Exposure of the UE information, e.g. UE IP address translation information as specified in clause 5.2.26.3 of TS 23.502 [3] and clause 4.15.10 of TS 23.502 [3] if Network address translation (i.e. NAT) functionality of the UE IP address is deployed within UPF.

-	Support PDU Set Handling as defined in clause 5.37.5.

-	Support MoQ relay functionality and PDU Set Information identification from the MoQ metadata as defined in clause 5.37.X.2.

-	NAT information exposure functionality (if NAT is deployed within UPF).

-	Handling of operator configurable UPF capability as described in clause 5.8.2.21.

# * * * * Next change* * * *

#### 6.3.3.3	Selection of an UPF for a particular PDU Session

The following parameter(s) and information may be considered by the SMF for UPF selection and re-selection:

-	UPF's dynamic load.

-	Analytics (i.e. statistics or predictions) for UPF load, Service Experience analytics and/or DN Performance analytics per UP path (including UPF and/or DNAI and/or AS instance) and UE related analytics (UE mobility, UE communication, and expected UE behavioural parameters) as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

-	UPF's relative static capacity among UPFs supporting the same DNN.

-	UPF location available at the SMF.

-	UE location information.

-	Capability of the UPF and the functionality required for the particular UE session: An appropriate UPF can be selected by matching the functionality and features required for an UE.

-	Data Network Name (DNN).

-	PDU Session Type (i.e. IPv4, IPv6, IPv4v6, Ethernet Type or Unstructured Type) and if applicable, the static IP address/prefix.

-	SSC mode selected for the PDU Session.

-	UE subscription profile in UDM.

-	DNAI as included in the PCC Rules and described in clause 5.6.7.

-	Local operator policies.

-	S-NSSAI.

-	Access technology being used by the UE.

-	Information related to user plane topology and user plane terminations, that may be deduced from:

-	5G-AN-provided identities (e.g. CellID, TAI), available UPF(s) and DNAI(s);

-	Identifiers (i.e. a FQDN and/or IP address(es)) of N3 terminations provided by a W-AGF or a TNGF or a TWIF;

NOTE 1:	A W-AGF or a TNGF may provide Identifiers of its N3 terminations when forwarding over N2 uplink NAS signalling to the 5GC. The AMF may relay this information to the SMF, as part of session management signalling for a new PDU Session.

-	Information regarding the user plane interfaces of UPF(s). This information may be acquired by the SMF using N4;

-	Information regarding the N3 User Plane termination(s) of the AN serving the UE. This may be deduced from 5G-AN-provided identities (e.g. CellID, TAI);

-	Information regarding the N9 User Plane termination(s) of UPF(s) if needed;

-	Information regarding the User plane termination(s) corresponding to DNAI(s).

-	RSN, support for redundant GTP-U path or support for redundant transport path in the transport layer (as in clause 5.33.2) when redundant UP handling is applicable.

-	Information regarding the ATSSS Steering Capability of the UE session (e.g. any combination of ATSSS-LL capability, MPTCP capability, MPQUIC capability) and information on the UPF support of RTT measurements without PMF.

-	Support for UPF allocation of IP address/prefix.

-	Support of the IPUPS functionality, specified in clause 5.8.2.14.

-	Support for High latency communication (see clause 5.31.8).

-	Support for functionality associated with high data rate low latency services, eXtended Reality (XR) and interactive media services, specified in clause 5.37 (for example, ECN marking for L4S, specified in clause 5.37.3, PDU Set Marking, specified in clause 5.37.5, UE power saving management, specified in clause 5.37.8).

-	Support for the On-path N6 signalling methods,e.g.MoQ relay functionality and/or CONNECT-UDP HTTP client functionality and/or UDP option functionality (as described in clause 5.37.x)

-	User Plane Latency Requirements within AF request (see clause 5.6.7.1 and clause 6.3.6 of TS 23.548 [130]).

-	List of supported Event ID(s) for exposure of UPF-related information via service based interface (see clause 7.2.29 and clause 5.2.26.2 of TS 23.502 [3]).

-	Information regarding required and/or preferred UPF functionalities. If received from UDM, the SMF selects a PSA UPF supporting the required UPF functionalities and the best set of preferred functionalities based on their priorities.

-	Support for operator configurable UPF capability as described in clause 5.8.2.21.

NOTE 2:	How the SMF determines information about the user plane network topology from information listed above, and what information is considered by the SMF, is based on operator configuration.

NOTE 3:	In this release the SMF uses no additional parameters for UPF selection for a PDU Session serving TSC or Deterministic Networking. If a PDU Session needs to connect to a specific UPF hosting a specific TSN 5GS bridge or 5GS router, this can be achieved e.g. by using a dedicated DNN/S-NSSAI combination.

If there is an existing PDU Session, and the SMF receives another PDU Session request to the same DNN and S-NSSAI, and if the SMF determines that interworking with EPC is supported for this PDU Session (as specified in clause 4.11.5 of TS 23.502 [3]), the SMF should select the same UPF if it supports all capabilities required for the new PDU Session. Otherwise, if the SMF determines that interworking with EPC is not supported for the new PDU Session or the UPF of the existing PDU Session does not support all capabilities required for the new PDU Session, a different UPF may be selected according to operator policy.

For the same DNN and S-NSSAI if different UPFs are selected at 5GC, when the UE is moved to EPC network, there is no requirement to enforce APN-AMBR. Whether and how to apply APN-AMBR for the PDN Connection associated with this DNN/APN is implementation dependent, e.g. possibly only AMBR enforcement per PDU Session applies.

# * * * * End of changes * * * *