3GPP TSG SA WG2 (#18)		   Revised 31.5.01 (S2-011538)

(Revised TSGS#8(00)0292)

Source:	WI raporteur

Title:	Revised (Draft 2) WID on Support of Location Services in Release 5, System and Core Network aspects.

# Proposal:

It is proposed to agree this Work Item description regarding location services in  Release 5 as shown below. This Work Item description is based on the Release 4  LCS Work Item description and is intended to cover all system and core network aspects of LCS. In addition it should be evaluated in S2 (in August) whether separate Work Items on some of the listed LCS issues are seen needed.

Work Item Description

# Title

Support of Location Services in Release 5, System and Core Network aspects.

1		3GPP Work Area

|  | Radio Access |
| --- | --- |
| X | Core Network |
| X | Services |

2		Linked work items

Support of Location Services in UTRA FDD

###### Support of Location Services in UTRA TDD

Support of Location Services in Packet Switched GERAN and circuit switched LCS enhancements

3		Justification

LoCation Services (LCS) provide the mechanisms to support mobile location services for operators, subscribers and third party service providers.

This Work Item is needed in order to fulfill the requirements on supporting LCS in the packet switched domain and to enhance LCS with new service features in Release 5.

4		Objective

LCS shall be supported in the circuit switched domain and in the packet switched domain. LCS shall be supported in the Access Networks UTRAN FDD, UTRAN TDD and GERAN.

The objective is to define signaling and  interactions, protocol elements and operations for the Core Network, Access Network and User Equipment from the system point of view to support location services.

UMTS LCS and GSM LCS should be aligned and a well defined set of LCS specifications should be developed for Release 5.

The detailed LCS time plan is documented in the 3GPP Work Plan.

| Task | Planned Start | Planned Finish |
| --- | --- | --- |
| Work Item Revision | 5/2001 | 6/2001 |
| Work Item Approval |  | 6/2001 |
| Drafting and discussion, updates of specifications | 7/2001 | 12/2001 |
| Submission to TSG CN and SA for approval |  | 12/2001 |
| Possible remaining corrections and clarifications |  | 6/2002 |

5		Service Aspects

New LCS service features shall be evaluated and included in the  Service description specification (Stage 1) as seen feasible. The service description should be common for all Access Networks. Possible new LCS service features identified so far are:

- Location services in GERAN packet switched domain

The task is to handle the system and core network aspects of LCS support inGERAN packet switched domain in Rel'5.

- Event based LCS reques and Periodic LCS request (Task 1171)

Event based LCS is planned to be included in Rel'4 to determine the position of a mobile when it becomes active in the network. Other types of events to initiate location reports should be investigated for Rel-5.

Periodic LCS could be seen as Event based LCS, where the event is that a defined period timer is running out and activates a location request/report. In Release 4 it has been decided that GMLC handles periodic LCS request, but in Rel-5 it should be investigated if also other network elements like MSC, SGSN, UE and RNC could handle periodic location reports autonomously. Depending upon how these possibilities are resolved, there may need to be a separate new RAN work item to cover RNC based periodic LCS and a new T work item to cover UE based periodic LCS aspects in Rel-5.

## - LCS interoperation stage 2 aspects (Task 544)

Task 544 should be renamed "LCS interoperation aspects UTRAN- GERAN". Possibly there should be a separate Work Item created for Release 5, which should describe the problems and possibilities of UTRAN – GERAN LCS interoperation. If the task or new work item is not agreed the current task 544 should be deleted from the 3GPP work plan.

## - LCS support in the IM CN subsystem (task 1183)

S1 is indicated to be doing a feasibility study on Location services in IMS, but there have been no contributions. A new work item might be needed, if this issue is seen relevant to be studied for standardization. If the task or a new work item is not agreed the current task 1183 should be deleted from the 3GPP work plan.

## - Standardization of the Le interface

There is a new Work Item on a standardized Le interface between the LCS Client and GMLC.

## - Localised services enabling features

Some of the location based services require information of the UE location and movements. These are for example

- Push services limited to UEs in defined area

- Location based charging where tariff is changed according to terminal where-abouts.

To support these type of services a location detection mechanism (= location based service trigger) would be needed in the network. The task would be to determine and specify an appropriate mechanism for this.

This task should probably be described in a separate Work Item. If this task is not agreed or if no new work item is proposed or agreed, this new task should not be added to the Work Plan.

## - Adding the parameter "Velocity" in stage 2 and stage 3 specifications

The current LCS stage 1 specification Rel-4 includes the parameter "Velocity" as location information of the target UE. This parameter needs to be added in stage 2 and stage 3 specifications.

6		MMI-Aspects

7		Charging Aspects

8		Security Aspects

It should be studied are there some implications on User Privacy in periodic or deferred location reporting.

9	Impacts

| Affects: | USIM | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes | X | X | X | X |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  |  |

- Expected Output and Time scale (to be updated at each plenary)

| New specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specification No. | Title |  | Prime rsp. WG | 2ndary rsp. WG(s) | Presented for information at plenary# |  | Approved at plenary# |  | Comments |
| Affected existing specifications |  |  |  |  |  |  |  |  |  |
| Specification No. | CR | Subject |  |  |  | Approved at plenary# |  | Comments |  |
| 22.071 |  | Location Services (LCS); Service description, Stage 1 |  |  |  | #8 (Rel'4) |  |  |  |
| 23.271 |  | Functional stage 2 description of LCS |  |  |  | #10 (Rel'4) |  |  |  |
| 24.008 |  | Mobile Radio Interface Layer 3 Specification |  |  |  | #11(Rel'4) |  |  |  |
| 29.002 |  | Mobile Application Part (MAP) |  |  |  | #8(Rel'4) |  |  |  |
| 25.305 |  | Stage 2 Functional Specification of Location Services in UTRAN |  |  |  | #11 (Rel'4) |  |  |  |
| 25.413 |  | UTRAN Iu interface RANAP signalling |  |  |  | #11(Rel'4) |  |  |  |
| 43.059 |  | Functional Stage 2 Description of Location Services in GERAN |  |  |  | #11(Rel'4) |  |  |  |

11		Work item raporteurs

[Jan Kall., Nokia]

12		Work item leadership

TSG SA WG 2.

13		Supporting Companies

To be added.

14		Classification of the WI (if known)

|  | Feature (go to 14a) |
| --- | --- |
| X | Building Block (go to 14b) |
|  | Work Task (go to 14c) |

14b	The WI covers several Building Blocks: List of Work Tasks  under these Building Blocks

(Note that the table below is based on the 3GPP Work Plan for LCS. Some LCS tasks are already listed for Release 5 and some new tasks have been added.

Comments are included regarding the LCS tasks for Release 4 in the Work Plan, but these table entries could be removed from the final version of this WID.)

| ID | Unique_ID | Task Name | Release | Comment/ Proposal |
| --- | --- | --- | --- | --- |
| 1 | 2044 | VERSION 2001 March 30th |  |  |
| 321 | 1536 | Location Services enhancements |  |  |
| 322 | 1171 | Event based (=deferred location request) and Periodic LCS | Rel5 | New work items might be needed to define tasks for Release 5, new events, periodic in MSC/SGSN and/or RNC/UE |
| 323 | 1641 | Stage 1, Event based, periodic LCS | Rel5 |  |
| 324 | 1538 | Stage 2 specification, Event based, periodic LCS | Rel5 |  |
|  |  | Stage 3 specifications (MAP, RANAP, RRC) , Event based, periodic LCS | Rel5 |  |
| 325 | 1179 | Impact on MAP | Rel4 | Completed |
| 326 | 519 | (copy) Charging and OAM&P (!) | Rel4 | Completed or unknown? |
| 327 | 521 | New security aspects of LCS (not identified) | Rel4 | Completed |
| 328 | 523 | LCS support in the CS domain | Rel4 | Completed in Rel99 |
| 329 | 525 | LCS support in the PS domain | Rel4 Rel5 | Completed for UMTS in Rel4. Planned for GERAN Rel5. |
| 330 | 1642 | Stage 1, LCS support in the GERAN PS domain | Rel5 | Completed in UMTS Rel-4. Planned for GERAN Rel5. |
| 331 | 1181 | Stage 2, LCS support in the GERAN PS domain | Rel5 | Completed in UMTS Rel-4. Planned for GERAN Rel5. |
| 332 | 1180 | Stage 3, LCS support in the GERAN PS domain | Rel5 | Completed in 24.007 and 24.080 UMTS Rel-4. Planned for GERAN Rel5. |
| 333 | 526 | Layer 3 LCS signaling UE (MS) –SGSN (UMTS PS and and GSM-GPRS) | Rel4 Rel5 | Same as task 1180? Task 526 to be deleted. |
| 334 | 2462 | MAP impacts of LCS | Rel4 | Rel-4 completed in case CR to add Event type in MAP is agreed in CN June. |
| 335 | 527 | GTP signaling for LCS | Rel4 | Task and its status unknown. |
| 336 | 544 | LCS interoperation stage 2 aspects | Rel5 | No contributions yet. New Work Item might be needed for Rel-5 |
| 337 | 2434 | LCS interoperation stage 2 aspects to GERAN |  | Same as 544, Task 2434 to be deleted. |
| 338 | 2435 | Co-ordinated development of GSM LCS Phase 2 and UMTS LCS, S2 and GERAN | Rel4 | Task completed if CR022 to 23.271 is approved in SA June. |
| 366 | 1796 | (Copy) LCS application interfaces (LCS-OSA) | Rel4 | Completed |
| 367 | 1183 | Feasibility Study on LCS support in the IMS | Rel5/6? | A New work item might be needed. |
|  |  | Possible new tasks in Rel'5 in addition to those listed above |  |  |
|  |  | Standardization of the Le interface | Rel5 |  |
|  |  | Localised services enabling features | Rel5 |  |
|  |  | Adding the parameter "Velocity" in stage 2 specifications | Rel5 |  |
|  |  | Adding the parameter "Velocity" in stage 3 specifications | Rel5 |  |