# Liaison Statement

From:	3GPP TSG-SA2

To:	RAN3

Cc:	RAN1, RAN2, SA4

Subject: 	RAB negotiation and re-negotiation

Contact:	Ina Widegren mail to ina.widegren@era.ericsson.se

Bonnie Chen mail to bchen1@motorola.com

___________________________________________________________________________

SA2 thanks RAN3 for their liaison (R3-011030 or S2-011399) with RAB negotiation/re-negotiation of three new QoS parameters (SDU error ratio, residual bit error rate, transfer delay), in addition to the two already existing parameters: maximum bitrate and guaranteed bitrate.

There were two views on how to respond to this Liason Statement.

- One view is that SA2 doesn’t have sufficient information to decide whether the negotiation/re-negotiation of the SDU error, residual bit error rate and the transfer delay between the UTRAN and the CN in the Release 5 time frame is feasible from end-user or application perspective, and whether it is possible for the end-user/application to set the values so that it brings savings on radio resource usage. 

For the attributes SDU Error Ratio, Residual Bit Error Ratio and Transfer Delay the merits of allowing the user to specify a range or list of values need to be analysed from an end-user or application point of view. How can this function be used by a Codec application to give UTRAN/GERAN a flexible choice between appropriate values for optimised radio conditions?  

As there is a trade off to consider between optimisation of the signalling load / bearer setup time and a compact QoS profile coding, it is important that a negotiation function only is introduced if clearly motivated from end-user/application point of view.  TSG S2 proposes that expertise on codec applications be consulted, to better understand the role these parameters would play in an end-to-end QoS (re)-negotiation scheme.

- The other view is that SA2 doesn’t see any issues of being able to negotiate or re-negotiate the SDU error, residual bit error rate and the transfer delay between the UTRAN and the CN in the Release 5 time frame especially if this could bring savings on radio resource usage. 

In fact, if certain RAB parameters cannot be supported by the UTRAN for example, it is better for the UTRAN to be able to initiate negotiation/renegotiation of these parameters than not being able to accept the call, or having to drop the call. Negotiation/renegotiation of RAB parameters in general, and of SDU Error Ratio, Residual Bit Error Ratio, and Transfer Delay (parameters addressed in the LS from RAN WG3) in particular, doesn’t violate any QoS architecture and is consistent from a QoS perspective.

Regarding the transfer delay, SA2 would like to inform RAN3 that no upper limit has been defined so far in the TS 23.107 (QoS concept and architecture). This may be a problem for the UTRAN to iniate the negotiation/re-negotiation of this parameter without any range or finite set of values.