# Liaison Statement

From:	3GPP TSG-SA2

To:	RAN3

Cc:	RAN1, RAN2, SA4

Subject: 	RAB negotiation and re-negotiation

Contact:	Ina Widegren mail to ina.widegren@era.ericsson.se

Bonnie Chen mail to bchen1@motorola.com

___________________________________________________________________________

SA2 thanks RAN3 for their liaison (R3-011030 or S2-011399) with RAB negotiation/re-negotiation of three new QoS parameters (SDU error ratio, residual bit error rate, transfer delay), in addition to the two already existing parameters: maximum bitrate and guaranteed bitrate.

When evaluating whether the negotiation/re-negotiation of the SDU error, residual bit error rate and the transfer delay between the UTRAN and the CN in the Release 5 time frame is feasible a number of issues need to be considered: the ability of end-users or applications to provide sufficient information, detailed bearer QoS architecture, trade off considering the size of information elements, and the potential for radio resource savings.

- Is it possible for the end-user/application to set the values so that it brings savings on radio resource usage?

- How can this function be used by a Codec application to give UTRAN/GERAN a flexible choice between appropriate values for optimised radio conditions?

- Trade off to consider between optimisation of the radio utilisation in data phase and a compact QoS profile coding. From UTRAN point of view

- , if certain RAB parameters cannot be supported by the UTRAN, it is better for the UTRAN to be able to initiate negotiation/renegotiation of these parameters than not being able to accept the call, or having to drop the call. On the other hand a range or list of values increase the size of the QoS profile (defined in 24.008) signalled over the radio interface.

- Regarding the transfer delay, SA2 would like to inform RAN3 that no upper limit has been defined so far in the TS 23.107 (QoS concept and architecture). This may be a problem for the UTRAN to initiate the negotiation/re-negotiation of this parameter without any range or finite set of values.

The conclusion of TSG S2 is:

Negotiation/renegotiation of RAB parameters in general, and of SDU Error Ratio, Residual Bit Error Ratio, and Transfer Delay (parameters addressed in the LS from RAN WG3) in particular, is consistent and doesn’t violate the UMTS QoS architecture.

Before the details of RAB negotiation is defined, TSG S2 proposes that expertise on codec applications be consulted, to better understand the role these parameters would play in an end-to-end QoS (re)-negotiation scheme.