3GPP - TSG SA - WG2 – MIP																Tdoc S2 M 99 026

9-11 March 1999

Stockholm - Farsta

Title: 		Definitions for MIP report

Date: 		1999-03-11

Source: 	Telia

Purpose:	For approval and inclusion in MIP report v0.5.0, chapter 6

This document contains some of the terms used in the MIP report. It is not an exhaustive list and needs to be extended later.

Mobile Equipment – has not been found in 02.60 or 03.60 docs – need to look in UMTS docs

User Equipment – has not been found in 02.60 or 03.60 docs - need to look in UMTS docs

6.1 Definitions

Agent Advertisement [rfc2002] An advertisement message constructed by attaching a special Extension to a router advertisement [4] message.

Care-of Address [rfc2002] The termination point of a tunnel toward a mobile node, for datagrams forwarded to the mobile node while it is away from home.  The protocol can use two different types of care-of address:  a "foreign agent care-of address" is  an address of a foreign agent with which the mobile node is registered, and a "co-located care-of address" is an externally obtained local address which the mobile node has associated with one of its own network interfaces.

Correspondent Node [rfc2002] A peer with which a mobile node is communicating.  A correspondent node may be either mobile or stationary.

Foreign Network [rfc2002] Any network other than the mobile node's Home Network.

Home Address [rfc2002] An IP address that is assigned for an extended period of time to a mobile node.  It remains unchanged of where the node is attached to the Internet.

Home Network [rfc2002] A network, possibly virtual, having a network prefix matching that of a mobile node's home address. Note that standard IP routing mechanisms will deliver datagrams destined to a mobile node's Home Address to the mobile node's Home Network.

Link [rfc2002] A facility or medium over which nodes can communicate at the link layer.  A link underlies the network layer.

Link-[rfc2002] Layer Address The address used to identify an endpoint of some communication over a physical link.  Typically, the Link-Layer address is an interface's Media Access Control (MAC) address.

Mobile IP  Mobile IP as defined in RFC 2002.

Mobile IP+ Mobile IP and the ongoing work in IETF on Mobile IP. Where applicable this term should be accompanied by specific references.

Mobile Node The mobile station. The term is used in IETF.

Mobile Station [GSM02.60] Equipment intended to access a set of GSM PLMN telecommunication services. Services may be accessed while the equipment capable of surface movement within the GSM system area is in motion or during halts at unspecified points (source: GSM 01.04). The mobile station may include a mobile termination and terminal equipment.

Mobile Termination [GSM02.60] The part of the mobile station which terminates the radio transmission to and from the network and adapts terminal equipment capabilities to those of the radio transmission (source GSM 01.04).

Mobility Agent [rfc2002] Either a home agent or a foreign agent.

Mobility Binding [rfc2002] The association of a home address with a care-of address, along with the remaining lifetime of that association.

Mobility Security Association [rfc2002] A collection of security contexts, between a pair of nodes, which may be applied to Mobile IP protocol messages exchanged between them

Node  [rfc2002]  A host or a router.

Nonce [rfc2002] A randomly chosen value, different from previous choices, inserted in a message to protect against replays.

Security Parameter Index (SPI) [rfc2002] An index identifying a security context between a pair of nodes among the contexts available in the Mobility Security Association.  SPI values 0 through 255 are reserved and MUST NOT be used in any Mobility Security Association.

Terminal equipment: [gsm02.60] Equipment that provides the functions necessary for the operation of the access protocols by the user (source: GSM 01.04). A functional group on the user side of a user-network interface (source: ITU-T I.112). The part of the mobile station that is not the mobile termination.

Tunnel  [rfc2002]  The path followed by a datagram while it is encapsulated.  The model is that, while it is encapsulated, a datagram is routed to a knowledgeable decapsulating agent, which decapsulates the datagram and then correctly delivers it to its ultimate destination.

Virtual Network [rfc2002] A network with no physical instantiation beyond a router  (with a physical network interface on another network. The router (e.g., a home agent) generally advertises reachability to the virtual network using conventional routing protocols.

Visited Network [rfc2002] A network other than a mobile node's Home Network, to which the mobile node is currently connected. One visited network is related to one FA, i.e. when the mobile node switches FA’s (at e.g. handover), it switches visited networks, even if the old and the new visited networks are within the same administrative domain.

Visitor List [rfc2002] The list of mobile nodes, visiting a foreign agent.

6.2  Abbreviations

| USIM | User Services Identity Module |
| --- | --- |
| ME | Mobile Equipment |
| MT | Mobile Termination |
| TE | Terminal Equipment |