3GPP TSG SA WG3 Security — S3#54bis	S3-090541

# Sophia Antipolis, France, 23 – 27 March 2009

Source:	InterDigital

Title:	Comments on S3-090485, a CR on TR 33.820, from Huawei, on section 7.2.2.3. H(e)NB Authentication

Document for:	Discussion and Approval

Agenda Item:	8.1

Work Item / Release:	Work Item on H(e)NB security

# 1. Introduction

In S3-090485, Huawei proposed a CR to section 7.2.2.3 of the TR 33.820 v1.3.0, that attempts to clarify the various contexts of H(e)NB authentication (e.g. initial authentication, vs. re-authentication).

In general InterDigital supports the contents of this pCR but we felt that the CR needed some significant editorial clarifications and also some content modifications.

Several comments and suggested changes are thus provided, using a Commenting pCR format for the submission of this document.

The changes we propose, on top of Huawei’s proposed change of the text from the TR, are marked with a different color than the one used by Huawe, which is visible and distinguishable.

# 2. Commenting pCR part begins here

3GPP TSG-SA3 (Security)	S3-09abcd

Meeting SA3#54 Adhoc, Sophia Antipolis, 24 - 26 Mar 2009	9xyzw

| CR-Form-v9.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.820 | CR | CRNum | rev | - | Current version: | 1.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to section 7.2.2.3 H(e)NB Authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_HNB_Sec |  |  |  |  |  |  | Date: |  | 11/03/2009 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) |  |
| Reason for change: |  | It is proposed that the term explanation listed in this contribution is included in the TR, and hence the corresponding editor’s note be deleted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following the last meeting conclusion, A clarification is given here about the definition of all authentication terms in section 7.2.2.3 H(e)NB Authentication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 7.2.2.3	H(e)NB Authentication

H(e)NB authentication consists of:

- H(e)NB identity authentication;

This refers to device authentication, which is described in section 7.6.2.

- TrE identity authentication;

Section 7.2.1.1 (Trusted Environment: General) proposes a requirement for a TrE to securely authenticate its identity to core network, so thatcan validate a TrE_ID as being that of a valid, issued, TrE and hence H(e)NB.

In cases where the TrE securely stores the H(e)NB_ID and uses it to represent itself as well as the H(e)NB; that is, if H(e)NB_ID serves as both the ID of the H(e)NB and the ID of the TrE, then the device authentication procedure effectively performs authentication of both the H(e)NB ID and the TrE_ID, and there is no need to perform a separate step for TrE ID authentication.

Although the H(e)NB_ID can be chosen as the Identity of the TrE itself, there is still a valid case in which the TrE has its own separate identity to identify it uniquely. .

Editor’s Note:  The current device authentication procedure description in section 7.6.2 does not consider the case where the TrE_ID needs to be separately authenticated from authentication of the H(e)NB_ID. Procedures for this case are FFS.

- H(e)NB device identity and TrE identity logical binding;

This refers to use of a unique TrE identity (TrE_ID) which is logically linked to a separate H(e)NB identity. In this case, the core network should store this binding relationship in advance. According to the pre-stored records, the core network can validate whether the new received IDs match in the authentication procedure.

- The H(e)NB integrity validation

This refers to the H(e)NB validation, more specifically, it refers to either verification of the signaling message that H(e)NB may send to the SeGW regarding the outcome or an aspect of a Semi-Autonomous Validation it has performed, as described in section 7.5.2.4, or the remote validation described in section 7.5.2.3

NOTE 1: the H(e)NB identity and the TrE identity can be used interchangeable when needed.

NOTE 2: the authentication we described here is for some specific scenarios when TrE has a separate identity. Whether it can be optimized depends on the specific policy of the operator.

There could be two broad categories of H(e)NB authentication scenarios:

- Initial authentication

- Re-authentication.

Initial authentication refers to the authentication that the H(e)NB performs to connect to the core network for the first time after the H(e)NB is delivered to the hosting party. In initial authentication, the four above a)b)c)d) steps and a related authentication are needed. After the above steps are completed, the authentication data that relates to the customization (such as H(e)NB location and Access Control List) will be downloaded and stored in the TrE securely. Because these data cannot be provided before the H(e)NB is ordered by the hosting party, it is necessary to securely access them online and then provision them into the TrE in a secure way.

During the H(e)NB initial authentication, the following are executed: the H(e)NB identity and TrE identity authentication, authentication of the binding relationship between H(e)NB device identity and TrE identity, the H(e)NB platform integrity verification, and any other H(e)NB related authentication/verification (such as HPM authentication, H(e)NB Location verification and UE authentication). When the last category of H(e)NB related authentication is successfully performed, the authentication data from these H(e)NB related authentication will be downloaded and stored in the TrE securely.

A H(e)NB related authentication/verification (e.g., HPM authentication, H(e)NB Location verification and UE authentication, etc) does not make use of pre-provisioned credentials, unlike in the case of the authentication of the H(e)NB or TrE identities. The related authentication may include H(e)NB Location verification and so on. These authentication data (i.e. the criterion)  generated and only stored in the core network at first.

During the H(e)NB re-authentication, the authentication process is similar to the initial authentication except for the fact that the H(e)NB-related authentication is performed by the TrE, because the TrE has stored the authentication data as a result of the initial authentication.  According to the data , the TrE can complete the related authentication.

End of modifications