3GPP TSG SA WG3 #4 (Security)						   3GPP S3-99-167

London, UK, 16.-18. June 1999

Agenda Item:	9.6

Source:	France Telecom

Title:	Change of status of the annex B of document 33.102

Document for:	Approval

Version:	15th June 1999

The user identity confidentiality mechanism described in document 33.102 in section 6.1 and 6.2 clearly describe the sending of a permanent identity as an operator choice about whether to send it in clear text or to use some sort of ciphering of this identity. Since the way to cipher that identity should be operator control, we would like that the status of annex B is changed to make it clear that this annex is informative.  We therefore submit the attached CR.

| 3GPP TSG-SA WG3 meeting #4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  | ???99??? |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| London, UK, 16-18 June 1999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 33.102 |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.0.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submision to TSG |  |  |  |  | SA#3 |  |  |  | for approval |  |  |  |  |  | x |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  |  | ME |  |  |  |  |  |  |  | x |  | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | France Telecom |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 15-06-99 |  |  |  |  |
| Subject: |  |  | Precision of the status of annex B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  | 3G Security architecture |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | Modification of status of annex B on user identity confidentiality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | Annex B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

Annex B (Informative):
Enhanced user identity confidentiality

This mechanism allows the identification of a user on the radio access by means of  the permanent user identity encrypted by means of a group key. The mechanism described here can be used in combination with the mechanism described in 6.2 to provide user identity confidentiality in the event that the user not known by means of a temporary identity in the serving network.

The mechanism assumes that the user belongs to a user group with group identity GI. Associated to the user group is a secret group key GK which is shared between all members of the user group and the user’s HE, and securely stored in the USIM and in the HE.

The mechanism is illustrated in Figure B.1.

Figure B.1: Identification by means of the IMUI encrypted by means of a group key

The user identity procedure is initiated by the visited VLR. The visited VLR requests the user to send its permanent user identity.

Upon receipt the user generates a time variant parameter TVP. The user encrypts the time variant parameter TVP and the IMUI with enciphering algorithm f6 and his group key GK. The TVP prevents traceability attacks. The user sends a response to the VLR that includes the HE identity, the group identity GI and the encrypted mobile user identity (EMUI).

Upon receipt of that response the SN/VLR should resolve the user’s HE address from HE-identity and forwards the group identity GI and the user’s EMUI to the user’s HE.

Upon receipt the HE retrieves the group key GK associated with the group identity GI. The HE then decrypts EMUI with the deciphering algorithm f7 (f7 = f6-1) and the group key GK and retrieves TVP and IMUI. The HE then sends the IMUI in a response to the visited SN/VLR.