TSG SA WG3 #4, London, 16-18 June 1999-06-16												S3-99179

| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | TS 33.102 |  |  |  |  |  | CR |  |  |  | Xxx |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | V3.0.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to TSG |  |  |  |  | SA#4 |  |  | for approval |  |  |  |  |  | x |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  | ME |  |  |  |  |  |  |  |  | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  | x |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Siemens AG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  | 16-06-99 |  |  |  |  |
| Subject: |  |  | Conditions on use of authentication information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  | Security |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | Authentication information needs to be handled by the SN/VLR according to certain rules. These rules are not clearly described in the current version of 33.102 and are therefore added for clarification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | 6.3.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

### 6.3.3	Authentication and key agreement

The purpose of this procedure is to authenticate the user and establish a new pair of cipher and integrity keys between the SN/VLR and the MS. During the authentication, the user verifies the freshness of the authentication vector that is used.

Figure 1: Authentication and key establishment

The SN/VLR invokes the procedure by selecting the next unused authentication vector from the ordered array of authentication vectors in the VLR database.  The SN/VLR sends to the user the random challenge RAND and an authentication token for network authentication AUTN from the selected authentication vector.

Upon receipt the user proceeds as shown in Figure 2.

Figure 2: User authentication function in the USIM

Upon receipt of RAND and AUTN the user first computes the anonymity key AK = f5K (RAND) and retrieves the sequence number SQN = (SQN Å AK) Å AK.

Next the user computes XMAC = f1K (SQN || RAND || MODE) and compares this with MAC which is included in AUTN. If they are different, the user sends user authentication reject back to the SN/VLR with an indication of the cause and the user abandons the procedure.

Next the user verifies the freshness of the received sequence number SQN.

For each mode the USIM keeps track of one counter: SQNMS/CS for authentications initiated by the CS CN nodes, and SQNMS/PS for authentications initiated by the PS CN nodes.

To verify the freshness of the sequence number SQN, the USIM compares SQN with SQNMS/MODE. If SQN > SQNMS/MODE  the MS considers the sequence number as fresh and subsequently sets SQNMS/MODE  to SQN.

Note:	The HE has some flexibility in the management of sequence numbers. Annex C contains alternative method for the generation and verification of sequence numbers.

If the user considers the sequence numbers not fresh, he sends user authentication reject back to the SN/VLR with an indication of the cause and the user abandons the procedure.

If the sequence number is consider fresh however, the user computes RES = f2K (RAND) and includes this parameter in a user authentication response back to the SN/VLR. Finally the user computes the cipher key CK = f3K (RAND) and the integrity key IK = f4K (RAND). Note that if this is more efficient, RES, CK and IK could also be computed earlier at any time after receiving RAND.

Upon receipt of user authentication response the SN/VLR compares RES with the expected response XRES from the selected authentication vector. If XRES equals RES then the authentication of the user has passed. The SN/VLR also selects the appropriate cipher key CK and integrity key IK from the selected authentication vector.

Conditions on the use of authentication information by the SN/VLR: Using the procedures described in subsections 6.3.1, 6.3.2 and 6.3.4, authentication vectors will have to be used in the specific order in which they were generated, otherwise the user will reject the authentication attempt. The SN/VLR shall use an authentication vector only once and, hence, shall send out each user authentication request RAND || AUTN only once no matter whether the authentication attempt was successful or not. A consequence is that authentication vectors cannot be reused. When a user changes from one VLR to another one and the new VLR requests remaining authentication vectors from the old VLR (cf. subsection 6.3.4) then the old VLR shall not retain any copies of these authentication vectors. When a VLR receives a “cancel location” request for a certain user it shall delete all authentication vectors relating to that user.  When a VLR receives a location update request from a user and the VLR notices that authentication vectors relating to that user are still stored in the VLR it will delete this information and request fresh authentication vectors from the HE/AuC.

Different rules may apply when one of the alternative schemes for sequence number handling described in Annex C is applied. This is true in particular when the schemes based on windows or lists described in Annexes C.3 and C.4 are applied.