# 3GPP TSG-SA3 Meeting #100bis-e 	S3-202437

# e-meeting, 12 – 16 October 2020

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Discussion on editor notes in SQNms concealment solution

# Document for:	Discussion and Decision

Agenda Item:	2.5

# 1	Decision/action requested

This contribution proposes an update to solution #4.3 resolving editor notes.

# 2	References

(Reference - in list form - should be made to previous related SA3/3GPP/etc. documents.)

[1]    3GPP TS 33.501 " Security architecture and procedures for 5G system".

[2]    3GPP TR 33.846 “Study on authentication enhancements in 5G System”.

[3]    3GPP TS 24.501 “Non-Access-Stratum (NAS) protocol for 5G System”.

# 3	Rationale

This discussion paper is to resolve the editor notes in solution #4.3 in TR33.846 [2].

# 4	Discussion on Editor notes

## 4.1	Editor's notes about AMF receiving 5G-GUTI

The following editor’ note has been captured in SA3#100 in the solution summary for solution #4.3:

Editor’s Note: It is ffs if this solution works for synchronization failure when AMF receives 5G-GUTI and decides to initiate authentication.

Observations:

Solution of “SQNms protection by concealment” works with existing procedure of Authentication failure (Sync failure) received with 5G-GUTI in AMF. If AMF decides to initiate the authentication after the synchronization failure (section 6.1.3.3.1 from TS 33.501 [1]), IDENTITY REQUEST is sent to UE and UE generates a fresh SUCI (SUCI with current value of SQNms in the USIM). Generated SUCI is sent in IDENTITY RESPONSE message to AMF. These procedures as described in section 5.4.1.3.6 of TS 24.501 [3] remain same as before and are not modified by this solution.

Resolution:

It is proposed to delete this editor's note.

## 4.2	Editor's notes about backward compatibility

The following editor’s note has been captured in SA3#100 in the solution summary for solution #4.3:

Editor’s Note: Backward compatibility analysis is FFS.

Observations:

The solution only affects UDM and USIM. The configurations for USIM is received from UDM. Two possibilities are UDM being older version with USIM being newer version and UDM being newer version with USIM being older version. With any of these two possibilities since HPLMN owns the subscription, both USIM and UDM changes can be synchronised by HPLMN operator at same time. So, there is no backward compatibility issue with the solution “SQNms protection by concealment”.

Resolution:

It is proposed to delete this editor's note.