# 3GPP TSG-SA3 Meeting #100bis-e 	S3-202446

# e-meeting, 12 – 16 October 2020

# Source:	Nokia, Nokia Shanghai Bell

# Title: 	Comparison of different solutions for Key issue#4.1

# Document for:	Discussion and Decision

Agenda Item:	2.5

# 1	Decision/action requested

This contribution proposes a comparison of different solutions for Key issue#4.1

# 2	References

[1]    3GPP TR 33.846 “Study on authentication enhancements in 5G System”.

# 3	Rationale

This discussion paper is to evaluate different solutions for Key issue #4.1 Protection of SQN during AKA re-synchronisations.

| Solutions from TR 33.846 | Algorithm impact | Applicability LTE & 5G | Key Issues solved | Comments on impact |
| --- | --- | --- | --- | --- |
| ### 6.4.1 Solution #4.1: Using ### MACS as freshness in the calculation of AK | Yes. It impacts Algorithm (MILNAGE and TUAK) and needs updates | Applicable for 4G & 5G | Only addresses one Key issue (1. Key Issue #4.1: Protection of SQN during AKA re-synchronisations) | 1.Increases the computation for extra f5*. 2. SUCI replay attack not addressed. 3. how easy it is for USIM adaptable with this algorithm solution |
| ### 6.4.2 Solution #4.2: Using ### symmetric encryption ### function to protect SQN ### during a re-synchronisation ### procedure in AKA | Yes. It impacts Algorithm (MILNAGE and TUAK) and needs updates | Applicable for 4G & 5G | Only addresses one Key issue(1. Key Issue #4.1: Protection of SQN during AKA re-synchronisations) | 1.Increases the computation for extra f6*. 2. SUCI replay attack not addressed. 3. how easy it is for USIM adaptable with this algorithm solution |
| ### 6.4.3 Solution #4.3: SQN ### protection by concealment ### with SUPI | No impact on Algorithm | Applicable for 5G | Three Key issues addressed (1. Key Issue #4.1: Protection of SQN during AKA re-synchronisations 2. Key Issue #2.2: SUCI replay 3. Key issue #3.2 KI to mitigate the SUPI guessing attacks) | 1. 4G solution not available |
| ### 6.4.4 Solution #4.4: SQN ### protection during re- synchronisation procedure in AKA | No impact on Algorithm | Applicable for 3G, 4G & 5G | Only addresses one Key issue (1. Key Issue #4.1: Protection of SQN during AKA re-synchronisations) | 1. SUCI replay attack not addressed. |

There are 4 solutions (#4.1 – #4.4) that address key issue #4.1 on protection of SQN during AKA re-synchronisations. All of them have impact on UDM and USIM.

It is recommended to resolve this issue in a pragmatic way by enhancing the security for authentication. It is important to address different 3GPP generations. Operators may want to choose which solution to implement in its network for which generation.

Since for solution 4.1 and 4.2 the USIM impact is not clear i.e. how easy it is for USIM adaptable with this algorithm solution, it is proposed to base final selection on solutions #4.3 and #4.4.

As practical solution selection it is recommended to use solution #4.4at least for previous releases. By choosing this solution, the probability of mounting an attack can be reduced by big storage assigned for the RAND comparison. Since the solution is not fully secure, for 5G, solution #4.3 is recommended. The clear advantage of this solution over the others is that it does not only address the resynch key issue but also provides means against SUCI replay and SUPI guessing attacks.

Another clear advantage of this combo is that both have only little impact on USIM and UDM.

# 4	Detailed proposal

Endorse the comparison.