3GPP TSG-SA3 Meeting #100e 	S3-201591-r1

- e-meeting, 17 – 28 August 2020

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Updating 33.501 Annex L text for PTP support

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	Rational

This conclusion is for describing the reason and setting a reminder for updating the existing annex L to also address PTP security requirements, since the protection of the transfer of timing information for exposure of time synchronisation in Rel-17 shall consider all supported PDU session types. I.e. the security requirements for authenticity, integrity and replay protection are the same for the transfer in the user plane of gPTP messages and PTP messages in Ethernet or IP PDU sessions.

# References

[1]	3GPP TR 21.905

[2]	3GPP TS 33.501

[3]	3GPP TS 23.501

[4]	3GPP TR 23.700-20

[5]	IEEE 802.1Qcc

[6]	IEEE 1588-2008

[7]	RFC 7384

## Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5G				Fifth Generation

5GS				Fifth Generation System

ARP				Address Resolution Protocol

CNC				Centralized Network Configuration

CP				Control Plane

CUC				Centralized User Configuration

DS-TT				Device Side Translator

DoS				Denial of Service

gPTP				generalized Precision Time Protocol

IIoT				Industrial Internet of Things

IP				Internet Protocol

KI				Key Issue

Ln				Layer n

MAC				Media Access Control

NW-TT				Network Side Translator

PTP				Precision Time Protocol

TSC				Time Sensitive Communication

Rel				Release

UE				User Equipment

TSN				Time Sensitive Networking

TSN AF				TSN Application Function

UPF				User Plane Function

UP				User Plane

****** START OF CHANGES

# 7	Conclusions

## 7.1. Actions on existing Annex L in 33.501

An update of the existing annex L to also address PTP security requirements is needed.

Reasoning:

The Rel-17 study on enhancements of support of IIoT (3GPP TR 23.700-20 [4]) introduces the exposure of TSC capabilities by the 5GS to an AF making requests for any type of PDU session. Especially the exposure of time synchronisation applies for Ethernet and IP PDU sessions. Four methods for timing and time synchronisation are envisaged (see [4], Key Issue#3B), making use of the PTP protocol (IEEE 1588-2008 [6]) in Rel-17 additionally to the gPTP protocol (IEEE 802.1AS) in Rel-16. This leads to the following possible cases for the transfer of the timing information in a PDU session:

- For Ethernet type PDU session: both PTP over Ethernet and gPTP over Ethernet

- For IP type PDU session: PTP over UDP/IP

In Rel-16 the protection of the transfer of PTP messages has not be considered so far.

The use of PDU sessions of the type IP occurs when in 5GS UPF acts as IP router and provides Layer 3 connectivity, whereby for PDU sessions of the type Ethernet the connectivity is provided at Layer 2.

****** END OF CHANGES