3GPP TSG-SA3 Meeting #100-e 	S3-201637

- e-meeting, 17-28 August 2020

- Source:	CableLabs, Apple, Deutsche Telekom AG, InterDigital Communications, Rogers Communications, Charter Communications

- Title:	Updates to solution #20 - resolving ENs in 6.20.2.4

- Document for:	Approval

Agenda Item:	5.2

# 1	Decision/action requested

It is proposed to approve the changes of solution #20 in TR 33.809.

# 2	References

N/A

# 3	Rationale

This contribution addresses the following Editor's Notes in solution #20.

Editor's Note: The impact on the N2 interface from digital signing requests and responses is FFS.

.

# 4	Detailed proposal

****START OF CHANGES ***

#### 6.20.2.4	Procedures for digital signature request and response

The overall procedure for gNB to request digital signatures is given below.

Fig 6.20.2.4-1 Procedure for DS request

SIB Digital Signing Requests and responses have impact on the N2 interface by consuming network bandwidth and adding processing overhead.

The actual bandwidth consumption depends on the frequencies of signing requests and responses being exchanged between gNB and AMF. Assume that a digital signature for a SIB is valid for a certain duration, e.g., 1 second (i.e., the signature can be readverised for this SIB within each second), and each signing request is for only one signature. In other words, for every second, there is a signing request sent by gNB to AMF, and there is a corresponding response from AMF to gNB.

Further assume that a SIB is of maximal size of 2976 bits (372 bytes) [2] and the digital signature algorithm is 256-bit ECDSA with the digital signature of 64 bytes). Without considering the overhead from other data (e.g., packet headers), the bandwidth consumption for each request and response is of 372 bytes and 64 bytes respectively. For each day without considering the bandwidth overhead from other accompanying data in each request and response, the total bandwidth consumption from signing requests and responses are about 32MB and 5.6MB respectively. When the number of SIBs to be protected increases, the bandwidth consumption from a signing request increases. When signing requests are aggregated, bandwidth consumption from a signing request is reduced. For example, if each signing request asks for 60 digital signatures (to be used for a next minute), the bandwidth consumption from sending signing requests would be reduced by 60 folds to 0.5MB per day.

From AMF perspective, one AMF may interact with a number of gNBs. Thus, the bandwidth consumption at the AMF is proportional to the number of served gNBs and inversively proportional to the number of requested signatures in each signing request. The table below provides bullbalk estimates of bandwidth and processing overhead at AMF, based on the estimation at gNB as described above.

| Frequency of broadcasting a new signature | Total # of signatures required per day | # of signatures asked per signing request | Total # of signing requests per day | Total bandwidth overhead (bytes) at gNB | # of served gNBs per AMF | Total # of messages processed at AMF | Total bandwidth overhead at AMF (bytes) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Per second (This means that SIBs broadcasted with a periodicity less than 1 second will reuse a same signature for a duration of 1 second) | 3600 x 24=86400 | 1 | 86400 | Signing Requests: 32M Signing responses: 5.6M | 100 | Signing requests from all gNB: = Signig responses to all gNB: = 8,640,000 | Signed requests from all gNB: 3.2G Signig responses to all gNB: 56M |
|  |  |  |  |  | 1000 | Signing requests from all gNB: = Signig responses to all gNB: = 86,400,000 | Signed requests from all gNB: 32G Signig responses to all gNB: 560M |
|  |  | 60 (each request asks 60 signatures to be used for a minute) | 60x24=1440 | Signing Requests: 0.5M Signing responses: 5.6M | 100 | Signing requests from all gNB: = Signig responses to all gNB: = 144,000 | Signed requests from all gNB: 50M Signig responses to all gNB: 56M |
|  |  |  |  |  | 1000 | Signing requests from all gNB: = Signig responses to all gNB: = 1,440,000 | Signed requests from all gNB: 500M Signig responses to all gNB: 560M |

Editor Note: Further evaluation of processing overload on AMF and gNB are FFS.

***END OF CHANGES***