3GPP TSG-SA3 Meeting #100-e 	Draft-S3-201720-r1

- e-meeting, 17 – 28 August 2020                                              merger of S3-201720 and S3-201951

- Source:	Huawei, Hisilicon

- Title:	New Key Issue on Security Protection of MBS traffic

- Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes a new KI for TR 33.850.

# 2	References

[1]	3GPP TR 23.757: " Study on architectural enhancements for 5G multicast-broadcast services ".

[2]	3GPP TS 33.246: " Security of Multimedia Broadcast/Multicast Service (MBMS) ".

# 3	Rationale

In SA2, there is an ongoing study on MBS in TR 23.757 [1]. Traffic delivery methods are discussed and defined.

In SA3, eMBMS security in LTE was analysed in TS 33.246[2]. Threat and requirements on protection of MBS traffic are described.

This contribution proposes a new Key Issue for MBS TR to identify specific traffic protection issues.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR33.850.

***	BEGIN OF CHANGE	***

## 5.X	Key Issue #X: Security protection of MBS traffic

### 5.X.1	Key issue details

According to TR 23.757 [xx], MBS traffic needs to be delivered from application service provider to multiple UEs through 5GS. Depending on many factors, multiple delivery methods may be used to deliver MBS traffic. As depicted in the following figure, Shared PTP or PTM delivery method and Individual delivery method may be used at the same time for a 5G MBS session depending on selected solution.

Figure 5.X-1: Schematic showing delivery methods

The 5GS may provide multiple interfaces for transferring MBS data between UE and external services/networks, such as Uu, N3, N6. MBS traffic need to be properly protected especially in air interface. While it is still possible to support security for multicast/broadcast traffic at the application layer, it is necessary to consider a security natively provided by the 5G system for the following reasons:

- There would be multicast/broadcast services that do not have application level security (e.g., due to protocol overhead) but want to leverage the security provided by 5G system.

- Switching between unicast and multicast/broadcast bearers are to be supported for reliable data delivery as an enhancement over eMBMS.

As a result, MBS protection is independent of application layer protection. This key issue investigates security protection of 5G MBS PDU sessions/flows at the transport or service level.

Editor’s Note: this key issue may need to be updated based on the progress of the 5G MBS architecture design by SA2 and RAN WGs.

### 5.X.2	Security threats

Attackers may eavesdrop MBS traffic on the air-interface. Users that have not joined and activated a MBS service receiving that service without being charged.

Modifications and replay of messages in a way to fool the user of the content from the actual source, e.g. replace the actual content with a fake one.

### 5.X.3	Potential security requirements

The 5GS shall support the confidentiality protection, integrity protection, and anti-replay protection of MBS traffic.

***	END OF CHANGE	***